Playing Samples on the STe

Playing  samples  using the STe hardware could  not  be  simpler.

Basically, all we have to do is set up the start address, the end

address, a few volume, bass and treble data, and tell it to go!

The    definitions    for   the   sound    registers    are    in

\INCLUDES\STE_DEFS.S

There  are also several routines in \INCLUDES\SAMPLES.S  which  I

use: set_up_ste, set_down_ste, load_a_sample

THE REGISTERS

The  register  we use to play samples are as follows (I  got  the

info  from  an  ST  USER  coverdisk  document  written  by   Adam

Greenwood).

enable      $ffff8900 - sound playing mode

base_high   $ffff8902 - start address of sample

base_mid    $ffff8904

base_low    $ffff8906

address_high $ffff8908 - address currently being played

address_mid  $ffff890a

address_low  $ffff890c

end_high     $ffff890e - end address of sample

end_mid      $ffff8910

end_low      $ffff8912

sound_mode   $ffff8920 - mono/stereo ans sample speed

micro_data   $ffff8922

micro_mask   $ffff8924 - registers for altering volume etc

The enable register can be:

00   sound off

01   play it once

11   play forever

sound_mode controls whether the sample is mono or stereo.   Bit 7

is the mono/stereo flag.  0 for stereo, 1 for mono.  Bits 0 and 1

dictate the frequency of the sample:

00 = 6.258 KHz

01 = 12.517 KHz

10 = 25.033 KHz

11 = 50.066 KHz

The  micro_mask and micro_data registers are used to  change  the

volumes (master,  left,  right), the bass and treble, and the mix

with the sound chip.

To alter a setting,  you put $7ff into the mask, and then a value

into the data. The possible values are:

master_vol  %10011000000

left_vol    %10101000000

right_vol   %10100000000

treble      %10010000000

bass        %10001000000

mix         %10000000000

Before you put the value into micro_data, you add the setting you

require.   So if you want the master volume at 20,  you would  do

this:

move.w    #$7ff,micro_mask

move.w    #master_vol+20,micro_data

The ranges are as follows:

Master Vol     0 - 40

Left Vol       0 - 20

Right Vol      0 - 20

Treble         0 - 12

Bass           0 - 12

Mix            0 - 3

The values of Mix mean the following:

0 = ??? -12dB apparently!  whatever that means!

1 = mix with sound chip

2 = don't mix with sound chip

3 = reserved for future use

Apparently,  from looking at Adam's source code,  you have to  be

sure that the data has gone

First, I call load_a_sample, to load in \INCLUDES\SAMPLE_1.AVR

It returns the sample address and the length, which I store away.

Then,  I disable the sound.  I get the address of the sample, and

put  it into base_high,  using a MOVEP.   Then I add  the  sample

length to get the end address,  which I put into end_high,  using

another MOVEP.

I call the routine in SAMPLES.S which initialises the STe sound -

master volume, bass, treble etc.

I  set  the sample to "mono" and 6Khz,  and then set  it  off  in

infinite repeat mode.

Wait for a keypress.

Turn off the sound,  and call "set_down_ste", which allows normal

STFM sound to be played again (the keyclick for example).

James Ingram, May 16th 1994

I do not mind you using any parts of my code,  or any  ideas

of  mine.   All  I ask is that you give me a credit  for  my

ideas or code.  Cheers.