Playing more than one sample

Playing one sample over and over again gets tedious.   There must

be a way to play more than one sample.   Suprise,  suprise, there

is,  and it isn't all that hard,  either.  When we put the sample

player  into  "play forever" mode,  it doesn't  necessarily  mean

forever.   Using Timer A, we can play the sample a certain number

of times.  This is the proceedure:

Start sample playing

Set  up Timer A in Event Count Mode (mode 8).   Set the data  for

Timer  A to the number of times we want the sample to  play  (say

4).

When the sample has been played 4 times, Timer A kicks in.  Then,

using  Timer A we can get the next sample addresses and  set  the

playing off again.

2_PLAY.S  is the source code to do this.   You should already  be

familiar  with  Timer B from the no top and bottom  border  code.

Timer A is exactly the same, but uses different addresses.

The vector is at $134,  for example, and it uses bit 5 instead of

bit 0 for the mask and enable registers.

2_PLAY frees memory,  stores the timers and so on,  then loads in

the samples.

The "play_em" subroutine doesn't seem like much;  initialise  the

STe sound chip,  call a routine to play the next sample, and then

set up Timer A.

To set up Timer A, I:

-    disable Timer A

-    Put in my routine address

-    put  in  the  number  of times I want  to  play  the  sample

(returned by "play_next_sample)

-    Put Timer A into Event Count Mode

-    Mask it in!

Let's take a look at "play_next_sample"

I get the position in the sample list,  and get the number of the

next sample to play (-1 ends the list)

I multiply by 10, which is the length of data for 1 sample:

Start address       LW

End address         LW

Frequency           W

I add this offset to the start of the sample list,  and take  the

start and end addresses.  These I put into the registers.  I then

take the sound mode, and put that into the appropriate register.

Then, I turn on the sound to "loop forever".

Finally,  I get a counter from the list of samples to tell me how

many times to play the sample.  This value is returned in d0 from

the routine.

Now,  our first sample has been played 4 times,  and Timer A  has

been activated.  It turns off Timer A, stores all registers, sets

up the next sample to play,  and then re-enables itself with  the

new count.  That's it.  Simple.

If you look at "list_start", the list of samples to play, you can

see that I play sample 0 4 times (that's the BEAT).

"sam_list" holds the start and end addresses of the samples.   In

addition, I have already put in the type of sound that they are -

they are all mono, and they all (except SAX) play at 6Khz.

James Ingram, May 17th 1994

I do not mind you using any parts of my code,  or any  ideas

of  mine.   All  I ask is that you give me a credit  for  my

ideas or code.  Cheers.