Getting rid of the top border

Everyone can do this,  right?   Well,  not if you don't know how.

It's a simple technique, actually...  All you do is, at the right

time,  flick  into 60Hz and then back into 50Hz.   The  code  for

doing that part is...

eor.b     #2,$ffff820a.w

rept 8

nop

endr

eor.b     #2,$ffff820a.w

And how to do it at the right time?   Simply install your routine

on the $70 vector.  What the routine does is this...

put status register on stack

turn off all interrupts

store registers on stack

pause for around 14900 clock cycles

flick in and out

get registers from stack

get interrupt status from stack

How do we pause for 14900 clock cycles?  A simple pause loop:

move.w    #1064,d0

pause:

nop

dbra d0,pause

And that's all there is to it.... almost

Timer C will interfere,  and cause flickering every few  seconds.

To prevent this, simply disable timer C.

To  see  the  effect  of timer  C,  assemble  1_TOP.S  with  c_on

commented out.  Notice the flicker.

The  macros c_on and c_off are in MACROS.S,  and are only 1  line

long.

Everything else should speak for itself.

James Ingram, March 20th 1994

I do not mind you using any parts of my code,  or any  ideas

of  mine.   All  I ask is that you give me a credit  for  my

ideas or code.  Cheers.