Getting rid of the bottom border

Almost as simple as the top border.   This time, we have to flick

in and out of 60Hz at the bottom of the screen.  For this, we use

timer B.  If you've played around with my TEST.PRG, and tryed the

timer B numbers I suggested,  you will see how timer B can get us

to the bottom of the screen without any hassle.

In the main part of my routine,  I turn timer B off.  Then, I set

up the address,  and mask it in.   It is not yet on.  Although it

is  masked  in,  the control byte is still 0.   Then I set  up  a

simple $70 vector.   What this does, is turn timer B on, and tell

it to execute on the 199th line of the screen.

The timer B code does the following when executed:

store registers

wait for the next screen line

flick into 60Hz

pause

flick back into 50Hz

restore register

clear the interrupt bit

return from interrupt

James Ingram, March 20th 1994

I do not mind you using any parts of my code,  or any  ideas

of  mine.   All  I ask is that you give me a credit  for  my

ideas or code.  Cheers.