SUB-ROUTINE NAME        Hz_to_timer

BRIEF DESCRIPTION       Convert Number in d0.l to Control and Data values

for System Timers A to D.

FILENAME                HZ_TIMER.S

OTHER RESOURCES         Macro Div_l in INTMATH.I

LANGUAGE                Assembler (Devpac)

AUTHOR                  Simon.H.Rigby - aka PoshPaws - aka Si(gh)

ENTRY PARAMETERS        D0 = Interrupt Frequency Required (in Hertz)

EXIT PARAMETERS         D0 = Control Value in High Byte

Data Value in Low Byte

DETAILS -

Takes the contents of  d0  and  converts  it  into  Control  and Data Values

suitable for feeding into the Interrupt Timer Registers. It uses the closest

match it  can  to  the  frequency  required  by  dividing  the  master clock

(2457600Hz) by 4, 16 or 64 and  then  working  out the number of clock ticks

required  to  give  the  required  interrupt  frequency.  Destroys  previous

contents of d0-d1.

Example - Library

move.l  #12000,d0          ;12KHz required

bsr     Hz_to_timer

move.w  d0,d2              ;Data value

swap    d0

move.w  d0,d1              ;Control value

...

...

Xbtimer #0,d1,d2,#Int_sub  ;enable Timer A (0) interrupt

...                        ;with control and data to call

...                        ;Int_sub routine

rts

Int_sub         ...

...

rte                        ;return from interrupt