DOCUMENTATION FOR JOYVBL.S

By Paul Ditchfield

This program has been tested and  works  perfectly  on a Falcon 030 it

should work just as well on any STE  but  I have not been able to test

this.  The source code  uses  the  macros  supplied  by Peter Hibbs in

issue 9.

WHAT THE PROGRAM DOES

The program JOYVBL.TOS shows how to install the joypad reading routine

into the VBL  queue  and  how  to  read  and  use  the results.  After

installing the reading routine into  the  VBL  queue, the program then

fetches the results from Abuff and  prints  them  on the screen in the

form of a 32 bit binary number.   You  can see which buttons set which

bits simply  by  pressing  the  appropriate  buttons  on  a controller

connected to port A.  The program can easily be changed to display the

results from port B by passing  the  address  of Bbuff to the printing

routine instead of or as well as the address of Abuff.

The  program  will  quit  when  the   buttons  *  and  #  are  pressed

simultaneously this shows how easy it  now is to test for combinations

of buttons being pressed (when connected to a Jaguar * and #  causes a

reset).

HOW THE PROGRAM WORKS

The  first piece of code  simply  runs  a  small routine in supervisor

mode using xbios #38  "Supexec".   The  routine itself simply installs

the joypad reading routine  into  the  VBL  queue  so  that it will be

executed at  every  vertical  blank.   The  VBL  queue  is  a  list of

addresses held at $4ce which is in  protected memory hence the need to

be in supervisor mode.  These addresses point to routines which are to

be executed every vertical blank.  There  are  8 slots available (this

can be expanded) for addresses of routines.  One or more of these will

already be in use.  The routine looks  for  a slot which is empty i.e.

equal to zero, then puts the  address  of  the joypad routine into the

slot and saves the address of the slot we used.

Now our routine will be executed  every  VBL.   We save the address of

the slot we used because when  we  quit  our program, the slot must be

reset to zero otherwise the  machine  will  crash.  The routine called

"restore" is responsible for resetting the slot back to zero.

From now on then,  until  we  quit  from  the  program the two buffers

labelled Abuff and Bbuff are being fed  data from the joypads at 50,60

or 70 times a second.

The next section of code loads one of the buffers (in this case Abuff)

into a data register and tests each bit  in turn.  If the bit is a one

then the ASCII code for a "1" is sent to a buffer if it is a zero then

the code for a zero is sent.   What  we are actually doing is building

up an ASCII string in the buffer which can then be printed out.

Next we wait  for  a  vertical  blank  to  occur,  then  print out the

contents of the buffer using GEMDOS  function #9 "Cconws".  The buffer

has added on to it, some VT52 codes which position the cursor and make

it invisible.

The next chunk of code tests to see  if the "*" and "#" keys are being

pressed on the joypad.  If they are then we reset the VBL slot to zero

using the routine "restore"  then  we  quit  using  GEMDOS function #0

"Pterm0" otherwise we loop back and print out whatever value is now in

Abuff.

The remaining code is the joypad routine itself.

WHAT "jtest" DOES

See note to Dick Teuber before reading more.

It can be  seen  from  the  description  of  how  the  joypad uses the

available registers that it  can  be  quite  complicated to find which

buttons are being pressed at any  given moment.  This is especially so

if you need to test for different combinations of buttons.

For a start you have to  be  in  supervisor  mode, then if you want to

test for combinations of buttons  in  different  columns of the matrix

you have to mess  about  setting  a  bit,  copying  the registers then

setting another bit and copying the registers etc, etc.

It would be  much  better  if  you  only  had  to  deal  with just two

registers one for each controller  and  if these registers were usable

in either user or supervisor mode.  This is precisely what the routine

"jtest" does.  The 21  buttons  of  controller  A  are reported in the

least  significant  21  bits   of   the   long  word  labelled  Abuff.

Simultaneously the 21 buttons  of  controller  B  are  reported in the

least significant 21 bits of  the  long  word labelled Bbuff.  Now any

button or combination of buttons will have its own unique value making

it very easy to test for any combination you like.

Because the "jtest" routine is installed  into  the VBL queue it works

away in the background, all you have  to  do is to install it into the

VBL queue. Then to get complete  information  about either pad all you

have to do is examine Abuff and  Bbuff which are in unprotected memory

and are updated every 50, 60 or  even  70 times a second (according to

the sort of monitor attached) which is plenty fast enough!

FORMAT OF Abuff AND Bbuff

bit  #                  use

----------------------------

0.......................Up

1.......................Down

2.......................Left

3.......................Right

4.......................*

5.......................7

6.......................4

7.......................1

8.......................0

9.......................8

10......................5

11......................2

12......................#

13......................9

14......................6

15......................3

16......................Pause

17......................Fire 0 (labelled A on joypad)

18......................Fire 1 (labelled B on joypad)

19......................Fire 2 (labelled C on joypad)

20......................Option

Bits 21 to 31 of  Abuff  and  Bbuff  are  unused,  they are set to all

zeros.

HOW "jtest" WORKS

We can only test for one column  in  the  matrix  at a time but we can

test for both pads at once.  The  first  few chunks of code simply set

the appropriate bits in FF9202 to  0  and  then moves the results from

FF9200 and FF9202 into data  registers.   Then  we invert all the bits

using a logical NOT, this is done  because it seems far easier to work

with bits which are being set by events rather than cleared by them.

Now only some of the bits in  FF9200 and FF9202 are used for reporting

results the rest is rubbish and  must  be masked out, this is achieved

by using a logical AND.

Now we have the results for  all  of  the  buttons on both pads safely

held in data registers and cleaned  up  from any rubbish.  Events from

the joypad are shown by bits set to one.

The next few sections of code  copy the results into working registers

where they are manipulated so that the appropriate bits can be slotted

into their correct places  in  holding  registers.   When all the bits

have been slotted in, the holding  registers are copied into Abuff and

Bbuff and the routine is finished until  the next VBL when the process

is repeated.

Here is an example of the way in which the bits are manipulated:

Note  that  "a"=results  from  pad  A,  "b"=results  from  pad  B  and

"r"=rubbish

d0=0000 0000 0000 0000 0000 0000 0000 0000      d0 is clear

move.w  $ffee,$ff9202                   Test for U,D,L,R,pause, F0

move.l  $ff9200,d0                      Move result into d0

d0=0000 0000 0000 0000 rrrr rrrr rrrr bbaa

swap    d0

d0=rrrr rrrr rrrr bbaa 0000 0000 0000 0000

move.w  $ff9202,d0

d0=rrrr rrrr rrrr bbaa bbbb aaaa rrrr rrrr

not.l   d0                                      All bits are inverted

andi.l  #$fff00                                 Mask off the crap

d0=0000 0000 0000 bbaa bbbb aaaa 0000 0000

This procedure  is  repeated  until  all  the  results  are  stored in

registers d0-d3.

Now the results  are  moved  into  working  registers, manipulated and

moved into holding registers.

move.l  d0,d4                           Move into working reg

d4=0000 0000 0000 bbaa bbbb aaaa 0000 0000

andi.l  #$30f00                         Mask off pad B results

d4=0000 0000 0000 00aa 0000 aaaa 0000 0000

ror.w   #8,d4                           Rotate bits into position

d4=0000 0000 0000 00aa 0000 0000 0000 aaaa

eor.l   d4,d5                           Slot them into holding reg

d5=0000 0000 0000 00aa 0000 0000 0000 aaaa

The same sort of process is used on all the results in d0-d3 until all

21 possible events on each pad have been accounted for.