Atari ST Machine Specific Programming In assembly

Chapter 9: Interfacing With Peripheral Hardware

I am going to spend most of this chapter discussing printing

algorithms and print buffers.  Not because these objects alone

are important enough to warrant the considerable time I shall

devote to them, but because the topics will create an atmosphere

in which it will be convenient to discuss other relevant issues.

These other issues involve the parallel port, in general, and

that itself is an important resource, yet I will not explore all

of the ramifications of its use, but I will certainly try to

eliminate any mystery surrounding such use; certain system

functions that are useful, as well, in applications which do not

involve printing, and among these will be at least one which does

not function as specified; and certain problems that materialize

during the testing of algorithms--sometimes it is difficult to

determine just what is being tested.  Yes, the central theme will

be print buffers, but the asides shall provide opportunities for

an expansion of knowledge about machine specific programming in

assembly language on the Atari ST.

Because the data transfer rates of printing mechanisms are

usually much slower than that of the other components involved

with printing, most users prefer to lessen the effect of the

printing mechanism's speed impact on the rest of the system by

installing some sort of buffer between the program that is

controlling the printing and that mechanism.  The buffer is, of

course, memory, which can be part of that which we call main ram;

or it can be memory within the printer itself; or it can be the

memory in a device that is designed to be installed between the

computer and the printer.  In all cases, the memory that is used

is hardware; but when the memory comprising the buffer is a

portion of computer ram that is allocated by a program executed

within the computer, the term software buffer is often applied.

When the behavior of the algorithm that stores data in the buffer

permits us to visualize the buffer's structure as if it were

circular, as opposed to linear, the term spooler is often

applied.  In many references the word queue is used in place of

the word buffer.

There is a description of a print spooler on pages 68

through 70 of Atari ST Tricks & Tips, a Data Becker book,

published by Abacus Software.  Following that discussion is an

assembly language program.  If you are able to read the

discussion in that book, it will provide you with additional

descriptive information concerning print spoolers with which to

complement that which I am going to present here.  However, I

suggest that you do not try to implement the program given as an

example in that book; there are much better ones available.  In

fact, several will be presented shortly.

Data Transfer Rates

The term parallel interface port includes all circuits

involved with data transfer through the port connector that is

located at the rear of the computer.  When we execute an

algorithm to send data through the port, the rate at which data

flows is the port's instantaneous data transfer rate.  I say

instantaneous transfer rate because the data flow could be

limited by the algorithm which is sending the data; a different

algorithm might permit the exchange of data at a different rate.

In this particular case, there are two data transfer rates

involved; the explicit instantaneous rate and the implicit

maximum data transfer rate of the port.  For sure, sending

algorithms are not going to push valid data through the port at

rates that exceed its maximum data transfer rate.  Other devices

which could exert pressure on the transfer of data through the

port are the printer's port, and its internal buffer, if it has

one; the printing mechanism; an external print buffer connected

between the printer and the port; and any buffer that has been

constructed by software within the computer.

By peripheral hardware, in this chapter, I mean those

semiconductors that exist on the periphery of the microprocessor.

These semiconductors interface the MC68000 to other components of

the ST system and to devices connected to the computer.  Among

others, this group of integrated circuits includes the MC68901

Multi-Function Peripheral (MFP) and the AY-3-8910 (aka YM-2149,

according to the Abacus Internals book and their Graphics & Sound

book) Programmable Sound Generator (PSG).  These are the two

peripheral hardware components I will discuss in this chapter.

I will concentrate my discussions of the MFP and the PSG on

those portions of the devices which are involved with the

parallel interface port.  You can obtain a complete reference

guide for the MFP from a local Motorola office, from a

distributor or from Motorola Semiconductors, 3501 Ed Bluestein

Blvd., Austin, Texas 78721.  You can find a reference guide for

the PSG (listed as AY-3-8910A) in the ARCHER Semiconductor

Reference Guide, available from Radio Shack stores.

Unfortunately, this guide does not present any information

concerning the use of the two PSG I/O ports.  Naturally, these

are the parts of the integrated circuit with which I am most

interested.      In addition, these devices are scantily

discussed in the Internals book.  I suggest that you try to

obtain General Instrument's AY-3-8910/8912 Programmable Sound

Generator Data Manual.  If you are interested in the sound

capabilities of the PSG, you can refer to appropriate books and

magazine articles that address sound on the ST.  The other

peripheral components, such as the disk drive circuits and the

RS-232 circuits, are also discussed in various references.  You

can begin with the material in the Internals book and progress to

other references as you desire.

The proper system configuration for the material in this

chapter consists of the ST system and a printer connected to the

parallel port.  Initially, there should be nothing but a cable

between the ST port and the printer port; that is, eliminate all

dongles (A dongle is a hardware device that is designed to be

installed in the parallel interface port, but which has nothing

to do with printing.) and hardware print buffers.  If your

printer contains an internal buffer that is greater that one line

in length, and if this buffer can be disabled, you will be able

to gain from the experience if you do disable it until you are

finished with the material in this chapter.

The Multi-Function Peripheral

The MFP is a very complicated device.  I am going to discuss

only the components of this chip that I must in order to provide

a lucid presentation of its interaction with the ST's parallel

interface port.  The MFP has 24 directly addressable registers:

of these, as far as this discussion is concerned, we are

interested in the General Purpose I/O Register (GPIR)--for some

reason Motorola uses the mnemonic GPIP for this register--as you

can see, I use GPIR; the Active Edge Register (AER); the Data

Direction Register (DDR); the Interrupt In-Service Register B

(ISRB) and the Vector Register (VR).  In addition, we will be

interested in bit 0 of the 8-bit General Purpose I/O Interrupt

Port (GPIP).

The MFP registers are discussed on pages 32 through 40 of

the Internals book.  The memory addresses for the registers are

listed on pages 60 and 61.  Access to these locations is

permitted only while the processor is in supervisor mode.  You

can enter the supervisor mode from the AssemPro debugger simply

by clicking on the S status register bit.  The ST's interrupt

structure is discussed on pages 240 through 244.  The precise

bits of knowledge that must be extracted from the references in

the Internals book, from the Motorola MFP manual (pages 3 and 4)

and from explorations within the AssemPro debugger are these:

1. The contents of the VB register can be read at

location $FFFA17.  Those contents are $48

(binary 01001000).  Notice that the 3rd bit,

called the S bit is set.  Because this

particular bit is set, the MFP operates in the

sosftware end-of-interrupt mode.  When the MFP

operates in this mode, interrupt handlers for

MFP generated interrupts must clear the

appropriate bit in the appropriate in-service

register at the conclusion of the handler

algorithm.

2. For pin 11 of the parallel interface port the

in-service bit is bit 0 of ISRB.  The address

of this register is $FFFA11.  An instruction

that will clear the appropriate bit is:

bclr #0, $FFFA11.

3. The contents of the AER can be read at $FFFA03.

The byte of data there is $4 (binary 00000100).

The state of bit 0 determines whether an

interrupt will be generated by a one-to-zero

transition on pin 11, or by a zero-to-one

transition.  Because the state of pin 0 is 0,

interrupts will be generated by a one-to-zero

transition.

4. The contents of the DDR can be read at $FFFA05.

The byte of data there is $0 (binary 00000000).

The state of each bit determines whether the

corresponding bit of the GPIP is to function as

an input pin or an output pin.  When the logic

level of a bit in the DDR is 0, the associated

GPIP pin functions as an input.  Of interest,

in this particular case, is the fact that bit 0

is programmed to be an input pin.  And this is

the bit to which pin 11 of the parallel

interface is connected.

5. The contents of the GPIR can be read at

$FFFA01.  The data stored in the GPIR is

precisely that which is present at the GPIP;

therefore, we can determine the logic state of

pin 11 any time we choose to do so, by

observing the bit 0 state of the data byte

stored at $FFFA01.

When the printer is off; that is, no power applied, the

logic state of pin 11 is high.  When the printer is on; that is,

power is applied, and on-line (there is a difference) the logic

state is low.  When the printer is on (power applied) but off-

line, I can't tell you the state for your particular printer.  I

can tell you that my Star NX-10 keeps the line low when the

printer is initially put off-line, which is a little silly

because the printer cannot accept data when it is off-line.

However, if the computer attempts to send a byte of data through

the interface, the printer forces pin 11 high.  On page 218 of

the printer manual is a statement which indicates that pin 11 is

high when the printer is off-line, but the restriction mentioned

above is not specified in detail.  Two other conditions force the

logic state of pin 11 high; a full buffer and lack of paper.

In summary, bit 0 of the MFP GPIP is programmed by the

operating system to be an input; as such it is connected to pin

11 of the parallel interface port.  The logic state of this pin

is controlled by the printer connected to the port.  If the logic

state is low the computer assumes that the printer is ready to

receive data; if the logic state is high the computer assumes

that the printer cannot receive data.

We can read the byte of data stored at $FFFA01 almost as

easily as we can read data at any other location; almost as

easily because the data at $FFFA01 can't be read unless the

processor is in supervisor mode--remember the second MC68000

flaw.  Let us observe the data stored at that location, now.  In

the AssemPro debugger, click on the Status Register S-bit to

toggle the processor into the supervisor state.  In the register

field, double click on one of the registers, say A4, and, in the

dialog box which appears, select .B, then clear the dialog line

by pressing the Esc key.  Finally, type $FFFA01 on that line and

press the Return key or click on the OK box.  In place of

L:A4       =        0

you should see

B:$FFFA01  =      $77

if the printer is off, but

B:$FFFA01  =      $76

if the printer is on and ready to receive data.  If you see a

value other than $76 or $77, it will be because one or more of

the GPIR bit values in your system differ from those of mine.

Refer to page 61 of the Internals book for a list of the GPIP

bits and the use applied to each by the ST.  For example, bit 7

is used to detect a monochrome monitor.  I have a monochrome

monitor and that bit is 0 in my GPIR.  I deduce that you will see

that bit as a 1 if you have a color monitor; other things being

equal, you would probably see $F6 or $F7 as the contents of

$FFFA01 if you have a color monitor.

If the printer was off, turn it on now; if it was on, turn

it off.  The contents displayed for $FFFA01 will not change until

you do something to cause the register field portion of the

debugger screen to be rewritten.  You can force that simply by

double clicking on the Show register button just beneath the

register field.  Using this method, you can determine the logic

state of pin 11 for any printer state.  Remember that bit 0 of

the data at $FFFA01 indicates the state of pin 11; hence, the

relevant data nibble is the least significant of the two; that

is, the second digit will be even if the printer is on, but it

will be odd when the printer is off.

Program 60 has been designed to determine printer status via

the invocation of BIOS function #8.  See pages 161 and 154 of the

Internals book.  When the program is executed from the desktop,

the ready condition of the printer is printed on the video

screen.  Press the Return key to terminate the program.  Program

61 determines printer status via direct communication with the

MFP GPIR.  You should execute these programs for each of your

printer states.

Program 60. Using BIOS Function #8 to Determine Printer

Status.

; Program Name: PRG_7AP.S

;      Version: 1.002

; Assembly Instruction:

;     Assemble in PC-relative mode; save program with .TOS suffix.

; Execution Instructions:

;     Execute from the desktop, once for each relevant printer state.

; Function:

;     Invokes BIOS function #8, bcostat, to determine the logic state of

; pin 11 of the parallel interface port.  See pages 161 and 154 of the

; Internals book.  When this function is invoked, the value -1 is returned

; in data register D0 if the logic state of pin 11 is low; the value 0 is

; returned if the logic state is high.

determine_printer_status:

move.w  #0, -(sp)              ; 0 means pin 11 of parallel interface.

move.w  #8, -(sp)              ; Function = BIOS bcostat.

trap    #13                    ; Value returned in D0 indicates logic

addq.l  #4, sp                 ; state of pin 11 of parallel interface.

btst    #0, d0                 ; If bit 0 = 0, logic state is high,

beq.s   printer_not_ready      ; else logic state is low.

lea     ready, a0

bra.s   print_string

printer_not_ready:

lea     not_ready, a0

print_string:

pea        (a0)

move.w     #9, -(sp)

trap       #1

addq.l     #6, sp

wait_for_keypress:

move.w    #8, -(sp)

trap      #1

addq.l    #2, sp

terminate:

move.w     #0, -(sp)           ; Function = GEMDOS p_term_old.

trap       #1

data

ready:     dc.b 'Printer is ready.',$D,$A,0

not_ready: dc.b 'Printer is not ready.',$D,$A,0

align

end

Program 61. Determining Printer Status via direct

communication with the MFP GPIR.

; Program Name: PRG_7BP.S

;      Version: 1.003

; Assembly Instructions:

;      Assemble in PC-relative mode and save with a TOS suffix.

; Execution Instructions:

;     Execute from the desktop, once for each relevant printer state.

; Function:

;     Determines the logic state of pin 11 of the parallel interface port by

; testing bit 0 of the data stored at address $FFFA01, which is the

; address of the MFP's General Purpose I/O register.  If the state of this

; bit is zero (low), then the printer is ready to receive data; if the state

; of the bit is one (high), then the printer is not ready to receive data.

;     Remember: if your printer keeps the logic level of pin 11 low when

;               it is off-line, then the test will erroneously indicate that

;               the printer is ready to receive data when the printer is in

;               that state.

mainline:

; NOTE: Must enter supervisor mode in order to access the MFP's addresses.

enter_supervisor_mode:

move.l    #0, -(sp)

move.w    #$20, -(sp)          ; Function = SUPER.

trap      #1                   ; Supervisor mode is active after TRAP.

addq.l    #6, sp               ; D0 = SSP.

move.l    d0, d3               ; Preserve SSP in D3.

btst      #0, $FFFA01          ; $FFFA01 is address of MFP GPIR.

beq.s     printer_ready        ; Branch if logic state of pin 11 is low.

lea       not_ready, a0        ; Print string if logic state is high.

bra.s     print_string

printer_ready:

lea       ready, a0

print_string:

pea        (a0)

move.w     #9, -(sp)

trap       #1

addq.l     #6, sp

wait_for_keypress:

move.w    #8, -(sp)

trap      #1

addq.l    #2, sp

enter_user_mode:

move.l    d3, -(sp)            ; D3 contains SSP.

move.w    #$20, -(sp)          ; Function = SUPER.

trap      #1                   ; User mode is active after TRAP.

addq.l    #6, sp

terminate:

move.w    #0, -(sp)            ; Function = GEMDOS p_term_old.

trap      #1

data

ready:     dc.b 'Printer is ready.',$D,$A,0

not_ready: dc.b 'Printer is not ready.',$D,$A,0

align

end

Using the AssemPro debugger to monitor address $FFFA01, or

by executing programs 60 and 61, we are able to determine the

static logic level of pin 11 of the parallel interface port.

When data is being transferred through the port, however, the

logic level of pin 11 is dynamic; that is, constantly changing to

reflect the printer's ability to receive a data byte from the

computer.  The printer forces pin 11 high shortly after pin 1 of

the parallel interface is forced low by the computer.  The

computer forces pin 1 low just before it sends a data byte to the

printer, then it forces pin 1 high after the data byte has been

sent.  Shortly after pin 1 has been forced high, the printer

forces pin 11 low again.  During data transfers, therefore, the

logic level of pin 11 is represented by a series of pulses,

instead of a steady state level.

When the operating system is used to effect data transfers

to the printer, its functions take care of pin 1's logic levels,

and these functions monitor the state of pin 11.  A programmer

who restricts his repertoire to operating system functions need

not be concerned with such detail.  It is only when one desires

to bypass the operating system that intimate knowledge of the MFP

and the PSG becomes important.  Of course, we want to bypass the

operating system; that's why we are writing and reading this

book.

The Programmable Sound Generator

I begin with the information given on pages 59 and 60 of the

Internals book, under the heading $FF8800 Sound Chip.  There, it

is stated that the PSG has 16 registers, none of which is

directly addressed.  This information does not agree with the

specification guide in the Radio Shack book; the information in

the Radio Shack book is incorrect.  I dwell on this point because

a device such as the PSG will often permit direct access to its

registers via memory-mapped I/O.  This device, however, does not;

yet the PSG is connected in the ST so that it behaves as if data

exchanges with the parallel interface are partially accomplished

via directly addressable registers.

In the Internals book, reference is made, on pages 59 and

60, to a select register.  Yet, there is no mention of this

register on pages 48 through 54, where the PSG and all of its

registers is discussed.  A select register is not mentioned in

the Radio Shack description of the PSG.  However, PSG I/O port

selection in the ST is accomplished via a memory location which

behaves as if it is a select register.  The term select register

is used to describe a register through which device parameters

are set.  The conclusion to be drawn at this point is that we are

more concerned with the apparent behavior of the PSG as it

operates in the ST than we are in the description of the PSG as

given by reference books or by the manufacturer of the device.

Register 7 of the PSG is described as a mixer control-- I/O

Enable register in the Radio Shack book.  The second half of this

description is the one in which we are interested, because it

refers to the I/O ports.  It is the state of two bits in that

register which determine the direction of data flow through the

two I/O ports.  Bit 6 controls the direction of data flow through

port A; bit 7 controls the direction of data flow through port B.

When a port bit in register 7 is 0, the direction of flow must be

into the PSG; when a port bit is 1, the direction of flow must be

out of the PSG.  In the ST, both port bits are set to 1 during

the system initialization sequence, therefore, data flow through

both ports must be out of the PSG, unless the bits are altered by

an appplication.

Since that is the configuration we need, it would seem that

nothing else about this subject need be said.  However, because

register 7 is programmable, and because it may come to past that

one of your applications (or one that you purchase) alters the

bits in register 7, it is reasonable that I give the following

admonition: if you alter specific bits in register 7, then you

must do so in a manner that alters only those you intend.  In

order to assure this, you make changes in the register with a

mask.  You will see how this is done with the OR and AND

instructions in an example to be presented shortly.

PSG register 17 (aka port B) is the one through which the

code bound for the printer actually travels.  Just to confuse

you, the designers of the chip made the address of this register

15, which is hex F.  Register 16 (aka port A) is the one through

which a signal is sent to inform the printer that the computer

wants to send a byte of data.  Again, just to make your life

miserable, the designers of the chip decided that the address of

this register should be 14 = $E.

The data byte bound for the printer is usually stored in

port B as the second step in a two-step process.  The first step

involves the placement of the register number on the PSG's

internal bus located at its so called base address (A base

address is usually the address of the first memory-mapped

register of a device.).  Let me say this carefully, because it is

as confusing as it seems.  Address $FF8800 is the address to

which we write the number of the PSG register in which we want to

place data or from which we want to retrieve data, and it is also

the address from which we do read data.  That is, we select a

register by storing its address at ram location $FF8800; then, if

we want to read the data stored in the selected register, we read

location $FF8800.  If we want to write to the selected register,

on the other hand, ram location $FF8802 is the address to which

we write the data that is to be stored in a previously selected

register.

This information does not agree with that given in any

specification guide that I have seen, and I have seen them all,

but I have verified that these are the only memory locations

which can be used to pass the data through registers 16 and 17 of

the PSG.  Note that the information in the Internals book is not

entirely correct.  There it states that reading address $FF8800

gives you the last PSG register used.  The truth is this: if you

simply read the content of address $FF8800, perhaps by moving

that content into a data register, the data read (moved into the

data register) will be the content (not the register number

itself) of the last PSG register selected.  However, unless you

already know which register was the last to be selected, you

can't know from which register the data is being read.

The two-step write process mentioned above

requires two move instructions, as follows:

1. move.b  register_address, $FF8800

2. move.b  data, $FF8802

where register_address is the address (using any valid addressing

mode) of the register that is to receive the data which will

subsequently be placed on the PSG internal bus, in this case

register 17 at address $F, and data is that data (using any valid

addressing mode).

Once the data has been placed in PSG register 17, pin 1 of

the parallel interface must be pulsed from high to low and back

to high to effect the data transfer through the parallel

interface port.  This is also known as strobing pin 1.  Pin 1 of

the parallel interface is normally held high by the computer.

When the computer is ready to send data to the printer, it

strobes pin 1 low for a specific length of time.

When the printer has received the data, it strobes pin 10 of

the parallel interface low for a specific length of time.  In

other words, the printer acknowledges reception of the data.

Unfortunately, the ST does not provide a method of recognizing

this acknowledge strobe.  Too bad, it would be of great

assistance were it available.  To circumvent this inconvenience,

we are forced to rely on pin 11 of the parallel interface to tell

us, in a roundabout way, that the printer has received the data.

Pin 11 of the parallel interface is held high by the printer

while data transfer is occurring and when the printer is off (and

probably when it is off-line); in other words, whenever the

printer cannot accept data.  At all other times, the printer

holds this pin low.  So, we use this pin as a pseudo ACK

(acknowledge) pin.  Because of this double duty assigned to pin

11, when we check the dynamic status of pin 11 and find that it

is high, we can't determine the true printer status; we don't

know whether it is off or simply transferring data.

Therefore, when we want to send a character to the printer,

we sometimes program a waiting period in an elapsed time counter,

or counting loop, and continuously check the status of pin 11

until its logic level is low, but give up the waiting if the

predetermined elapsed time expires.  (It is also possible to let

the printer interrupt the processor and declare that it is ready

to accept a byte of data.)  If pin 11 has not become low during

the waiting period, we assume that the printer is turned off,

then we abandon the data transfer attempt and/or send a warning

message to the video screen.

To complete the data transfer through the PSG, then, pin 1

of the interface must be strobed.  PSG register 16 (aka port A),

at address $E, provides the access to this pin.  However, as

indicated in the Internals book, this port is used for 7 other

functions as well.  Therefore, when we manipulate the port A bit

connected to pin 1 of the parallel interface, we must leave the

other bits unaltered.

Bit 5 of register 16 is the bit we strobe to transfer the

data from register 17 to the printer.  To preserve the state of

the other seven bits, we first select register 16 for the

transaction, and read its content into a data register.  Then we

place a 0 in bit 5 of the data register by logically ANDING the

binary value 11011111 (hexadecimal DF) with the content of the

data register.  Finally, we write the data register content to

port A.  The instructions are as follows:

1. move.b  #$E, $FF8800 (select register 16)

2. move.b  $FF8800, dn  (read contents)

3. andi.b  #$DF, dn     (combine with mask)

4. move.b  dn, $FF8802  (write combination)

These four instructions strobe pin 1 of the printer's

parallel interface low.  We finish up by strobing the pin high

with the following instructions, again, preserving the state of

the other seven bits:

1. ori.b   #$20, dn     (combine with new mask)

2. move.b  dn, $FF8802  (write combination)

where, dn, in the above sets of instructions is any data

register.  When setting up the data register for the high strobe,

we OR the binary value 00100000 (hexadecimal #$20) with the data

register's content.  Using the strobing instructions, we generate

electrical signals in a manner that is analogous to flipping

switches manually.  Program 62 is an example which illustrates a

method of programming the PSG to effect printer output without

the assistance of the ST's operating system functions.  Of

course, since the addresses by which we access the PSG are

protected, we must enter supervisor mode in order to gain

accessibility.  We do use system commands for this.

Program 62. Direct output via the parallel port.

; Program Name: PRG_7CP.S

;      Version: 1.002

; Assembly Instructions:

;      Assemble in PC-relative mode and save with a TOS suffix.

; Function:

;      Provides direct output via the Parallel Port Interface.

; DESCRIPTION

;    The object of this experiment is to produce a minimal algorithm that

;    will send a string of characters to a printer connected to the parallel

;    port.

mainline:

; NOTE: Must enter supervisor mode in order to access MFP and PSG addresses.

enter_supervisor_mode:

move.l    #0, -(sp)

move.w    #$20, -(sp)          ; Function = SUPER.

trap      #1                   ; Supervisor mode is active after TRAP.

addq.l    #6, sp               ; D0 = SSP.

print_string:

lea       string, a3           ; Fetch address of string.

print_character:

move.b    (a3)+, d1

beq       enter_user_mode

printer_ready_test:

btst      #0, $FFFA01          ; Check logic level of pin 11.

bne.s     printer_ready_test   ; Loop until printer is ready to receive.

; NOTE: If my printer is off-line, the program will remain in the above loop

;       after the first character is sent, until the printer is placed back

;       on line.  Why?  Because the printer does not permit pin 11 to return

;       to zero after it receives that first character.  Therefore, there is

;       a kind of delayed reaction to the off-line condition.

lea       $FF8800, a2          ; Fetch address of PSG data bus.

move.b    #$F, (a2)            ; Latch address of port B.

move.b    d1, 2(a2)            ; Write character to port B.

move.b    #$E, (a2)            ; Latch address of port A.

move.b    (a2), d1             ; Read content of port A.

andi.b    #$DF, d1             ; Reset bit 5 of d1 to zero.

move.b    d1, 2(a2)            ; Reset bit 5 of port A. Strobe low.

ori.b     #$20, d1             ; Set bit 5 of d1.

move.b    d1, 2(a2)            ; Set bit 5 of port A. Strobe high.

bra       print_character

enter_user_mode:

move.l    d0, -(sp)            ; D0 contains SSP.

move.w    #$20, -(sp)          ; Function = SUPER.

trap      #1                   ; User mode is active after TRAP.

addq.l    #6, sp

terminate:

move.w    #0, -(sp)           ; Function = p_term_old = GEMDOS $0.

trap      #1                  ; GEMDOS call.

data

string:   dc.b  'This string is written to the printer using a routine',$D,$A

dc.b  'that communicates directly with the PSG ports.',$D,$A,0

align

end

Sending Files To The Printer

I have covered MFP and PSG basics.  Now, it order to delve a

little deeper, I must move into the general subject of sending

files to the printer.  But before I begin, let me simplify some

of the terminology that I will have to use.  Usually, we are

forced to utilize cumbersome phrases to designate the transfer of

data from the computer to a peripheral.  One reason is that we

use the word printing when referring to the sending of data to

the video screen, as well as when referring to the sending of

data to the printer.  I intend to eliminate the need for phrases

by calling the one screening and the other printing.  I shall use

the word print to indicate output from the computer to the

printer; and I shall use the word screen to indicate output from

the computer to the video screen.

I will breach the subject slowly, beginning with program 63.

I try nothing fancy in this program.  It is assembled as a TTP

program that accepts a file name on the TTP dialog box input

parameter line and prints, not screens, the file, using simple

operating system functions.  With this program, I also illustrate

the kind of sequential process that I use to obtain information

about the operation of the system.

Program 63. Printing a file with a TTP program using

system functions.

; Program Name: PRG_7DP.S

;      Version: 1.002

; Assembly Instructions:

;      Assemble in PC-relative mode and save with a TTP extension.

; Function:

;      Accepts a file name on the TTP dialog box input parameter line and

; prints that file using operating system functions.

; Execution Instructions:

;      Can be executed from the desktop, and, if the command line is fixed

; manually as described in the note below, can be executed from the AssemPro

; debugger in an uncorrupted system environment; that is, don't try it if

; you are using a program such as Switch/Back or Revolver to divide your

; system into partitions.  Both this program and the file to be printed must

; be in the same directory.

; Note:

;      This program was designed and assembled using a MEGA ST2.  While

; most of the information concerning the GEMDOS functions probably apply

; to all ST's, there may be some minor differences in the MEGA, such as

; the size of the buffer that is used to store a file as it is read.

;      The name of the file used to test the program was GEMDOS_9.S, which

; was 859 decimal bytes long.  First executions were from the desktop.  The

; name of the file to be printed was typed on the TTP dialog box input

; parameter line; once in upper case, once in lower case.  The file was

; printed correctly in both instances.

;      The program was then executed from the AssemPro debugger, using the

; following steps:

;      1. Single click on Execute program button.

;      2. On the command line, typed a leading space followed by the name of

;         the file, GEMDOS_9.S.

;      3. Clicked on OK button in dialog box.

;      4. In the File Selector box, double clicked on PRG_7DP.TTP.

;      5. On the first line in debugger output window, noted address of

;         command line in the instruction: LEA $89A68(PC),A3.

;      6. Clicked on from address button and typed 89A68 on dialog line at

;         bottom of screen.

;      7. Clicked on disassembled button.  Noted file name in output window.

;      8. Counted number of characters in file name = 10.

;      9. Typed in 0A at address $89A68.

;     10. Clicked on disassembled button.

;     11. Double clicked on from address button.

;     12. Installed a breakpoint at TST.W D0 instruction located just below

;         the TRAP #1 instruction, which is located just below the fetch_byte:

;         label in the source listing.

;     13. Clicked on the Run program button to execute to breakpoint.

;     14. Removed the breakpoint.

;     15. Clicked on the from address button.

;     16. Pressed the Return key to get to address $0.

;     17. Clicked on the Search button.

;     18. Typed ' ; Program Name', which is part of the first line of file

;         GEMDOS_9.S, on the parameter line of the Search for : dialog box.

;     19. Clicked on the dialog box's OK button.

;     20. Clicked on disassembled button.  Noted that file GEMDOS_9.S was

;         in ram; location of first byte at address $79C4.

;     21. Clicked on output window's right bar to get to the end of the data

;         read into the GEMDOS function $3F buffer in order to observe the

;         number of bytes of the file which had just been read in.  The last

;         character that could be identified as part of file GEMDOS_9.S was

;         the ) character, which is part of the line that reads

;                              move.w   #9, -(sp)

;         in the source listing.  This character was located at address $7BC3.

;     22. Calculated the number of bytes read as

;                 $7BC3 - $79C4 + 1 = $200 = 512 decimal bytes.

;         No output to printer yet, at this point.

;     23. Clicked on disassembled.

;     24. Double clicked on from address button to get back to program.

;     25. Clicked on right window bar to get to the LEA $C(A7),A7 instruction,

;         which is located just below the "end_of_file" label in the source

;         listing.  Placed a breakpoint there.

;     26. Clicked on the Run program button in order to execute program to

;         breakpoint.  Output to printer began.  Entire file was printed.

;     27. Removed breakpoint.

;     28. Clicked on from address button.

;     29. Typed address 79C4 on parameter line.

;     30. Clicked on disassembled button.

;     31. At address $79C4, again observed the first line of file GEMDOS_9.S.

;     32. Clicked on the right window bar to get to the end of the file, noting

;         that the entire file was in ram.  Last byte of the file proper

;         was a linefeed character, $0A, located at address $7D1D.  The first

;         byte of this file segment was located at the byte following the last

;         byte of the previously read segment; that is, at address $7BC4.

;         Following the character located at $7D1D was a NULL byte at location

;         $7D1E.

;     33. Calculated the number of bytes read as

;                 $7D1D - $7BC4 + 1 = $15A = 346 decimal bytes.

;         The sum of the first and second reads = 512 + 346 = 858 bytes, one

;         byte less than the size of the file on disk.  Concluded that the NULL

;         byte is supposed to be included as part of the file; added 1 to 858

;         to obtain the correct size of 859 bytes.

;     34. Clicked on the disassembled button.

;     35. Clicked on the Run program button.  Program terminated.

; Conclusions:

;      The GEMDOS $3F function reads in 512 bytes each disk access and stores

; the data in a buffer that is larger than 512 bytes but probably smaller

; than 1 megabyte.  If the file is smaller than the buffer, then the function

; will continue to read until the entire file has been read before moving on.

;      By processing a 16,758 byte file and searching through ram after the

; file had been printed, I was able to determine that the GEMDOS function $3F

; buffer size is $400 = 1024 decimal.

fetch_pertinent_addresses:

lea        -$82(pc), a3        ; Fetch "command line" address.

lea        -$80(a3), a1        ; Fetch "basepage" address.

lea        program_end, a0     ; Fetch "end of program" address.

lea        stack, a7           ; Fetch stack address.

lea        buffer, a4          ; Fetch buffer address.

calculate_program_size:

suba.l     a1, a0

return_unused_memory:

pea        (a0)                ; Push program length.

pea        (a1)                ; Push basepage address.

move.l     #$4A0000, -(sp)     ; Function = m_shrink = GEMDOS $4A.

trap       #1                  ; Invoke GEMDOS exception.

lea        $C(a7), sp          ; Reset stack pointer to top of stack.

process_command_line:

move.b     (a3)+, d0           ; Fetch command line character count.

ext.w      d0                  ; Extend to word for next instruction.

move.b     #0, 0(a3,d0.w)      ; Store a null at end of command line input.

open_file:                      ; Function returns file handle in D0.

move.w     #0, -(a7)           ; Open as read only.

pea        (a3)                ; Push address of file name.

move.w     #$3D, -(a7)         ; See Internals page 127.

trap       #1

addq.l     #8, a7

move.w     d0, d3              ; Store file handle in D3.

read_file:                      ; Function returns 1 in D0 unless end of file is

; reached or a read error occurs.

pea        (a4)                ; Push address of buffer.

move.l     #1, -(a7)           ; Number of bytes to read from

; GEMDOS $3F's buffer.

move.w     d3, -(a7)           ; Push file handle.

move.w     #$3F, -(a7)         ; See Internals page 129.

; Note: The system will not disturb this stack setup, therefore, it must be

;       initialized only once.  Thereafter, a branch need be taken only to the

;       following instruction.

fetch_byte:

trap       #1

tst.w      d0                  ; When NULL at end of file is read,

ble.s      end_of_file         ; D0 will be 0.

print_byte:

move.b     (a4), d0            ; Must read buffer one byte each time.

move.w     d0, -(sp)           ; But must push a word here.

move.w     #0, -(sp)

move.w     #3, -(sp)           ; See Internals page 155.

trap       #13

addq       #6, sp

bra.s      fetch_byte

end_of_file:

lea        $C(sp), sp

close_file:

move.w     d3, -(sp)

move.w     #$3E, -(sp)         ; See Internals page 128.

trap       #1

addq       #4, sp

terminate:

move.w     #0, -(sp)

trap       #1

data

buffer:       dc.w   $0  ; Character buffer.

bss

ds.l   96  ; Program stack.

stack:        ds.l    0  ; Address of program stack.

program_end:  ds.l    0

end

Now we begin to get fancy.  The next program accomplishes

the same task as does program 63.  Here, however, the parallel

interface is accessed directly.  The documentation presented in

programs 62 and 63 apply to this program also.  Note that I am

not wasting space on error checking of any kind; nothing should

go wrong; but if anything does go wrong, please feel free to use

the switches located on the rear of your computer.

Program 64. Printing a file by accessing the parallel

interface directly.

; Program Name: PRG_7EP.S

;      Version: 1.001

; Assembly Instructions:

;      Assemble in PC-relative mode and save with a TOS extension.

; Function:

;      Accepts a file name on the TTP dialog box input parameter line and

; prints that file, as does PRG_7DP.S, however, this program accesses the

; parallel interface directly, as does program PRG_7CP.S.

; Execution Instructions:

;      Execute from the desktop.  The name of the file typed on the TTP

; dialog parameter line and PRG_7EP.TTP must reside in the same directory.

; See documentation in PRG_7CP.S and PRG_7DP.S.

fetch_pertinent_addresses:

lea        -$82(pc), a3        ; Fetch "command line" address.

lea        -$80(a3), a1        ; Fetch "basepage" address.

lea        program_end, a0     ; Fetch "end of program" address.

lea        stack, a7           ; Fetch stack address.

lea        buffer, a4          ; Fetch buffer address.

calculate_program_size:

suba.l     a1, a0

return_unused_memory:

pea        (a0)                ; Push program length.

pea        (a1)                ; Push basepage address.

move.l     #$4A0000, -(sp)     ; Function = m_shrink = GEMDOS $4A.

trap       #1                  ; Invoke GEMDOS exception.

lea        $C(a7), sp          ; Reset stack pointer to top of stack.

process_command_line:

move.b     (a3)+, d0           ; Fetch command line character count.

ext.w      d0                  ; Extend to word for next instruction.

move.b     #0, 0(a3,d0.w)      ; Store a null at end of command line input.

open_file:                      ; Function returns file handle in D0.

move.w     #0, -(a7)           ; Open as read only.

pea        (a3)                ; Push address of file name.

move.w     #$3D, -(a7)         ; See Internals page 127.

trap       #1

addq.l     #8, a7

move.w     d0, d3              ; Store file handle in D3.

; NOTE: Must enter supervisor mode in order to access MFP and PSG addresses.

enter_supervisor_mode:

move.l    #0, -(sp)

move.w    #$20, -(sp)          ; Function = SUPER.

trap      #1                   ; Supervisor mode is active after TRAP.

addq.l    #6, sp               ; D0 = SSP.

move.l    d0, d4               ; Preserve SSP in D4.

read_file:

pea        (a4)                ; Push address of buffer.

move.l     #1, -(a7)           ; Read one byte.

move.w     d3, -(a7)           ; Push file handle.

move.w     #$3F, -(a7)         ; See Internals page 129.

fetch_byte:

trap       #1

tst.w      d0                  ; When NULL at end of file is read,

ble.s      end_of_file         ; D0 will be 0.

print_byte:

move.b     (a4), d0            ; Must read buffer one byte each time.

printer_ready_test:

btst       #0, $FFFA01         ; Check logic level of pin 11.

bne.s      printer_ready_test  ; Loop until printer is ready to receive.

lea        $FF8800, a2         ; Fetch address of PSG data bus.

move.b     #$F, (a2)           ; Latch address of port B.

move.b     d0, 2(a2)           ; Write character to port B.

move.b     #$E, (a2)           ; Latch address of port A.

move.b     (a2), d0            ; Read content of port A.

andi.b     #$DF, d0            ; Reset bit 5 of d1 to zero.

move.b     d0, 2(a2)           ; Reset bit 5 of port A. Strobe low.

ori.b      #$20, d0            ; Set bit 5 of d1.

move.b     d0, 2(a2)           ; Set bit 5 of port A. Strobe high.

bra.s      fetch_byte

end_of_file:

lea        $C(sp), sp

close_file:

move.w     d3, -(sp)           ; Push file handle.

move.w     #$3E, -(sp)         ; See Internals page 128.

trap       #1

addq       #4, sp

enter_user_mode:

move.l    d4, -(sp)            ; D4 contains SSP.

move.w    #$20, -(sp)          ; Function = SUPER.

trap      #1                   ; User mode is active after TRAP.

addq.l    #6, sp

terminate:

move.w     #0, -(sp)

trap       #1

data

buffer:       dc.w   $0  ; Character buffer.

bss

ds.l   96  ; Program stack.

stack:        ds.l    0  ; Address of program stack.

program_end:  ds.l    0

end

The custom trap handler used in program PRG_6KC.S can be

improved by replacing the algorithm used to print therein with

that used to print in program 64.  Multiple entries into the trap

#13 handler is eliminated, and the function required to screen

can be removed.  Now that I think about it, maybe that particular

function could have been eliminated in PRG_6JC and PRG_6KC, in

the first place.  No matter, If I were that good, I'd be rich and

I would not be doing this, in the second place.  Anyway, program

65 is an improvement; I can see that easily enough.  And it gets

us a step closer to the pertinent goals to be reached in this

chapter.

Program 65. An improved version of the program which

redirects output bound for the screen to both the

printer and the screen.

; Program Name: PRG_7FP.S

;      Version: 1.001

; Assembly Instructions:

;      Assemble in PC-relative mode and save with a TOS extension.

; Function:

;      The function of this program is identical to that of PRG_6JC.S and

; PRG_6KC.S.  This program also redirects output bound for the screen to the

; printer; however, this program accesses the parallel port directly, as does

; program PRG_7EP.S.

; Execution Instructions:

;      Execute from the desktop.  This program expects the printer to be on

; and will loop at the printer ready test until that condition is true.

;      After this program has been executed, execute REG_TST2.TOS to exercise

; it.  Turn the printer on before executing REG_TST2.TOS.

program_start:                  ; Compute program size and retain result.

lea        program_end, a3     ; Fetch program end address.

movea.l    4(a7), a4           ; Fetch basepage address.

suba.l     a4, a3              ; Program size in A3.

load_stack_address:

lea        stack, a7

install_custom_trap_13_vector:

pea        custom_trap_handler

move.w     #$2D, -(sp)          ; Trap 13 vector number.

move.w     #5, -(sp)            ; Function = setexec = BIOS $5.

trap       #13                  ; Current trap handler vector returned in D0.

addq.l     #8, sp

lea        preempted_handler_address, a0

move.l     d0, (a0)

relinquish_processor_control:    ; Maintain memory residency.

move.w     #0, -(sp)            ; See page 121 of Internals book.

move.l     a3, -(sp)            ; Size of memory to remain resident.

move.w     #$31, -(sp)          ; Function = ptermres = GEMDOS $31.

trap       #1

custom_trap_handler:

move.l     usp, a0              ; Load address of current top of user stack.

get_processor_status:

btst       #5, (sp)             ; User mode test.

beq.s      was_user_mode        ; No adjustment is necessary.

movea.l    sp, a0               ; Load current top of supervisor stack.

addq.l     #6, a0               ; Adjust SSP for user mode type data access.

was_supervisor_mode:

was_user_mode:                   ; Processing for either mode follows.

cmpi.w     #3, (a0)             ; Writing a character to a device?

bne        not_bconout_call

cmpi.w     #2, 2(a0)            ; Is device the screen?

bne        not_screen

esc_sequence_test:

lea        esc_sequence_flag, a1

tst.b      (a1)

bne.s      reset_esc_sequence_flag

cmpi.w     #$1B, 4(a0)

bne.s      not_esc_sequence

move.b     #1, (a1)

bra.s      use_preempted_handler

reset_esc_sequence_flag:

move.b     #0, (a1)

use_preempted_handler:

movea.l    preempted_handler_address, a0

jmp        (a0)                 ; JUMP TO PREEMPTED TRAP #13 HANDLER.

not_esc_sequence:

move.w     4(a0), d0            ; Store character for printer.

ascii_code_test:                 ; Filter out undesirable codes.

cmpi.w     #$1B, d0

bgt.s      printer_ready_test

cmpi.w     #$A, d0

beq.s      printer_ready_test

cmpi.w     #$D, d0

bne.s      undesirable_ascii

printer_ready_test:

btst       #0, $FFFA01          ; Check logic level of pin 11.

bne.s      printer_ready_test   ; Loop until printer is ready to receive.

lea        $FF8800, a0          ; Fetch address of PSG data bus.

move.b     #$F, (a0)            ; Latch address of port B.

move.b     d0, 2(a0)            ; Write character to port B.

move.b     #$E, (a0)            ; Latch address of port A.

move.b     (a0), d0             ; Read content of port A.

andi.b     #$DF, d0             ; Reset bit 5 of d1 to zero.

move.b     d0, 2(a0)            ; Reset bit 5 of port A. Strobe low.

ori.b      #$20, d0             ; Set bit 5 of d1.

move.b     d0, 2(a0)            ; Set bit 5 of port A. Strobe high.

write_character_to_screen:

undesirable_ascii:

not_screen:

not_bconout_call:

movea.l    preempted_handler_address, a0

jmp        (a0)                 ; JUMP TO PREEMPTED TRAP #13 HANDLER

bss

character:                  ds.w    1

preempted_handler_address:  ds.l    1

esc_sequence_flag:          ds.b    1

align

ds.l   96    ; Stack

stack:                      ds.l    1    ; Address of stack.

program_end:                ds.l    0

end

Print Buffers

A print buffer is memory that is used as a reservoir in

which characters are stored.  Because of the speed differential

between the printer and other computer system hardware, the

reservoir can be filled faster than it can be emptied.  This is

good.  That's how reservoirs are supposed to work.  I have

already discussed, in qualitative terms, the maximum rate at

which the reservoir may be emptied; that is accomplished by

accessing the parallel port directly when sending data to the

printer.  Now I shall present methods of filling the reservoir.

But first, let me preface that discussion with my

interpretation of the reason that print buffers are used and the

reason that they are constructed as they are.  In the old days,

when computers were housed in a large room in a building far away

from the many programmers using it, we communicated with the

computer via a video terminal (before that, we used teletype

machines--before that we beat hollow logs with sticks).  Most

often, only one printer was available for the general programming

population, and it was also located at the computer site.

Programmers did not request hard copy until it was absolutely

necessary; for two reasons: one, each printing job was placed in

a queue, and since each programmer's request was likely to be one

of many, some time was likely to elapse before the hard copy

appeared; two, the walk to the printer was usually not a short

one.  Under such conditions, it was easy to justify the existence

of a print buffer (or queue).

Now, as we sit at our complete systems, we become so spoiled

with immediate response that we find it difficult to occupy

ourselves for a few minutes while the printer does its business.

Of course, some printings require much more than a few minutes,

but, for those, I get up and walk away because I don't have room

for a printer silencer and I can't stand the noise generated by

the printer for more than a minute or two.  Still, I find print

buffers convenient because data transfers between the printing

program and a buffer is so much faster than it is between the

program and a bufferless printer, usually.

Now, because memory is, and always has been, a limited

commodity in any computer system, the buffers that we are most

likely to acquire are circular; that is, the buffers are of a

significant length, with a beginning address and an ending

address; and when we have sent enough data to fill a buffer, it

does not overflow as would a reservoir; instead, it begins to

refill, overwriting previously entered data.  At this point, the

first disadvantage of circular buffers become evident.  What do

you suppose happens to the data transfer rate of a print buffer

when it has become full.  Well, from that point on, it can accept

a new character only after one has been extracted from the buffer

by the printer, therefore, the buffer's data transfer rate

degenerates to that of the printing mechanism.  Note that the

buffer is considered to be empty when a particular printing task

has been accomplished.  For example, if we send a single file to

the printer through the buffer and permit the task to terminate

before sending another, then the second file sees an empty

buffer.  If, on the other hand, we send a two or more files back

to back, the buffer will not appear to be empty until all files

have been processed.

I am going to explain the operation of a circular print

buffer in detail, not because I think that it is the type you

should use, but because it is the type you are most likely to

see, because that is the historical print buffer design.  After

considering its weaknesses, I shall present another type--a

linear buffer.  In addition, the format of circular buffers

demands the use of algorithms which you might find useful in

other applications.  Finally, it is by comparing the data

transfer rate of the linear buffer to that of the circular buffer

that I shall be able to stress the linear buffer's advantage.

Program 66 introduces a program that installs a circular print

buffer in ram.

Program 66. This program installs a circular print

buffer.  The buffer has not been designed for practical

use, it has been designed for study.

; Program Name: PRG_7GR.S

;      Version: 1.014

; Assembly Instructions:

;      Assemble in Relocatable mode and save with a TOS and TTP suffix.

; Program Function:

;      Circular printer buffer.  The default buffer size for the TOS version

; of this program is only 100 bytes because this is a special program that is

; designed to be used to test printing programs and similar endeavors.  While

; this program is resident, those other programs can be executed from the

; AssemPro debugger, permitting this program's address locations and registers

; to be monitored.  Breakpoints can't be set in this program from the debugger

; window after it is resident, but, from a program under test in the debugger,

; you can single step in such a way as to end up within the custom trap #13

; handler installed by this program.

;      Once a printing session has begun, it cannot be cancelled merely by

; turning the printer off.  The problem is this: after a printing session

; has been initiated, if the printer is turned off to cancel the session,

; printing resumes when the printer is turned back on, because turning the

; printer back on causes the logic level of pin 11 to drop from high to low,

; thereby providing the falling edge required to invoke the interrupt handler.

;      Two exception handlers are installed by this program.  The trap #13

; handler intercepts all trap #13 invocations.  It handles only those that

; are relevant to the printing process; the others are passed on to the trap

; #13 handler that is preempted by the custom handler installed by this

; program.  The interrupt handler is invoked each time the logic level of

; pin 11 of the parallel interface port drops from high to low.  No provisions

; are made within this program to preserve the function of a pin 11 handler

; existing at the time this program's custom interrupt handler is installed.

;      When the printing program begins to send characters to the parallel

; port via system function invocations, if the print buffer established by

; this program is empty (because it has not yet been used or because all

; characters previously stored have been processed), then characters will be

; send to the printer by the trap #13 handler until the printer becomes

; busy enough so that it is unable to keep up with the flow of characters

; into and out of the trap #13 handler.  Once the printer falls behind,

; characters are thereafter stored in the print buffer.  From that point on,

; characters must be processed by the interrupt handler.

;      A special file has been prepared to provide a visual view of the

; concepts in the above paragraph.  With the printer on and this program

; installed, execute PRG_7DP.TTP and type SPOOLTST.DOC on the parameter input

; line.  All characters processed by the trap #13 handler will be printed

; in lower case; those printed by the interrupt handler will be printed in

; upper case.  You should print SPOOLTST.DOC once with the default buffer

; size, then execute PRG_7GR.TTP and type 32 on its input parameter line to

; increase the buffer size to 32 kilobytes.  Then print SPOOLTST.DOC again

; using PRG_7DP.TTP.

;      The instruction within the interrupt handler that converts characters

; from lower case to upper case is prominently marked.  No other file should

; be printed while this instruction is active because the character conversion

; instruction will wreck havoc with numeric and punctuation characters.  To

; print any other file with this program, use a semicolon to inactivate that

; instruction and assemble this program again.  Other versions of the circular

; print buffer which do not include that instruction will be introduced

; shortly.

; Execution Instructions:

;      May be executed from the desktop with a TOS or TTP extension.  May be

; executed from an AUTO folder if it has a PRG extension.

;      If a larger buffer size is desired, the program extension can be

; changed from TOS to TTP.  If a buffer size is declared on the TTP parameter

; line, any size, up to the maximum available machine memory, may be specified.

; When the TTP program is executed, type the desired size, in kilobytes, on the

; parameter line; the maximum number of digits which can be typed on the

; parameter line is 4.  Do not type spaces before the digits.  Do not type

; anything between digits.  Do not type anything after the digits, except the

; Return key.  The OK button may be clicked in lieu of a Return key press.

;      The length of the buffer will be (input X 1024) bytes.  Ex: if input is

; 32, length of the buffer will be 32,768 bytes.  After converting the ASCII

; decimal input to binary, it is much easier to simply shift the binary result

; 10 bits left to multiply it by 1024 than it is to multiply it by 1000.

;      The buffer length (either the default size or the specified size) is

; combined with the program size to determine the parameter which must be

; passed to GEMDOS function $31.

;      In order to permit observation of this programs variables and registers,

; its location in memory and other pertinent addresses are screened as the

; program is being loaded.

;      When the addresses appear on the screen, write them down.  Terminate

; execution by pressing the Return key.  In the AssemPro debugger, you can go

; to pertinent addresses using the "from address" function.  You can double

; click on any register in the register output field and use the dialog box to

; specify an address in this program that you wish to monitor.

;    Once this program has been executed, it will remain in ram until the

; computer is rebooted.

; WARNING:

;      If this program or any other LSR program is executed from within the

; AssemPro debugger, upon exit from AssemPro, the operating system will not be

; able to clear the program from memory.  Because the program will be residing

; in an area of memory that was controlled by AssemPro, the system environment

; will be corrupted.  You can confirm this by trying to reexecute AssemPro

; after you exit.  You will receive a Bus error message.

;      Furthermore, you may not be able to assemble a program until you reset

; the system.

;      There is only one thing you can do if you execute a LSR program from

; within AssemPro; you must reset the system by turning it completely off, then

; back on.  You can try a warm reset, but all bets are off if you do that.

;      Furthermore, remember that programs with a memory preserving or limiting

; algorithm require special treatment if they are executed while they are in

; memory because of having just been assembled; that is, when they are not

; loaded via the "execute program" button.  The special treatment consist of

; skipping over the initializing instructions that reference the locations

; 4(a7) and the basepage address.  That's because programs not loaded via the

; "execute program" button do not have a basepage.  The special treatment also

; means that the system function used to relinquish processor control must not be

; executed.

; NOTE:

;      The loop which processes the ASCII decimal input is formed with a dbra

; instruction.  This is a word size instruction, therefore, when the loop

; counter register is prepared by subtracting one from the number of input

; digits, the value in the counter register must be extended to word size to

; match the size of the dbra instruction.

program_start:                  ; Calculate program size and retain result.

lea        program_end(pc), a3 ; Fetch program end address into A3.

; If this program is assembled and observed in the debugger, the label

; program_end will not be seen because the label buffer has been declared in

; in the bss section as buffer: ds.l 0, and the end of the program has also

; been marked with program_end: ds.l 0.  Effectively, the label program_end

; does not even exist as far as the assembled bss section of the program is

; concerned.

movea.l    4(a7), a4           ; Fetch basepage address into A4.

suba.l     a4, a3              ; Program size is in A3.

lea        stack(pc), a7       ; Fetch address of the program stack.

process_input_parameter:        ; Calculate requested buffer length.

lea        $80(a4), a4         ; Fetch basepage parameter line start address.

lea        length(pc), a0      ; Fetch address of the length variable.

moveq      #0, d0              ; Clear counter register.

move.b     (a4)+, d0           ; Fetch parameter line character count.

beq.s      compute_size        ; Branch if no parameter--use default length.

cmpi.b     #4, d0              ; Greater than four test.

bgt        too_many_characters ; Branch if more than four characters.

subq.b     #1, d0              ; Set up counter.

ext.w      d0                  ; Extend to match size of DBRA instruction.

moveq      #0, d2              ; Initialize accumulator.

moveq      #0, d1              ; Clear scratch register.

fetch_digit:                    ; Convert input from ASCII decimal to binary.

move.b     (a4)+, d1           ; ASCII decimal digit to D1.

sub.b      #$30, d1            ; Convert ASCII decimal digit to decimal digit.

bmi        not_decimal         ; Branch if digit is less than zero.

cmp.b      #9, d1              ; Greater than nine test.

bgt        not_decimal         ; Branch if digit is greater than nine.

multiply_accumulator_content_by_ten:

move.l     d2, d3              ; Copy accumulator in D3.

lsl.l      #3, d2              ; Shift accumulator content to multiply by 8.

add.l      d3, d2              ; Add D2's original quantity twice to

add.l      d3, d2              ; complete the multiplication by 10.

add_new_digit_to_accumulator_content:

add.l      d1, d2              ; Add decimal number in D1 to accumulator.

dbra       d0, fetch_digit     ; Loop until d0 becomes negative.

extend_to_thousands_of_bytes:   ; Length will be (input X 1000) plus

moveq      #10, d3             ; some even number of bytes that is less

lsl.l      d3, d2              ; than 1000.  Ex: if input is 32, length

move.l     d2, (a0)            ; is 32768.  A0 contains address of length

; variable

compute_size:                   ; This will be parameter to GEMDOS $31.

adda.l     (a0), a3            ; Add buffer length to program size.

compute_and_store_buffer_end:   ; Presently, buffer_end = buffer start.

lea        buffer(pc), a1      ; Load buffer starting address.

adda.l     (a0), a1            ; Add buffer length to buffer start address.

move.l     a1, buffer_end      ; Store buffer_end address.

install_custom_trap_13_vector:  ; ******************************************

pea        trap_13_handler(pc) ; Push custom trap handler address onto stack.

move.w     #$2D, -(sp)         ; Trap 13 vector number.

move.w     #5, -(sp)           ; Function = setexec.

trap       #13                 ; Current trap 13 vector returned in D0.

addq.l     #8, sp

move.l     d0, preempted_trap_13_address

install_interrupt_handler_vector:  ; ***************************************

pea        interrupt_handler(pc)  ; Push interrupt handler address.

move.w     #0, -(sp)              ; Interrupt level for centronics ready.

move.w     #$D, -(sp)             ; Functon = XBIOS #13 dec = mfpint.

trap       #14

addq.l     #8, sp

print_memory_locations:

bsr        print_newline          ; For debugger screen protection.

lea        load_message(pc), a0

bsr        print_string

move.l     #program_start, d1     ; Print program start address.

bsr        bin_to_hex             ; bin_to_hex expects binary number in D1.

lea        hexadecimal(pc), a0    ; Print the hexadecimal string.

bsr        print_string

lea        separator(pc), a0      ; Print a separator between addresses.

bsr        print_string

move.l     #program_end, d1       ; Print program end address.

bsr        print_address

bsr        print_newline

lea        int_hand_msg(pc), a0   ; Print interrupt handler address.

bsr        print_string

move.l     #interrupt_handler, d1

bsr        print_address

bsr        print_newline

lea        trap_13_msg(pc), a0    ; Print custom trap #13 address.

bsr        print_string

move.l     #trap_13_handler, d1

bsr        print_address

bsr        print_newline

lea        io_buffer_msg(pc), a0  ; Print address of variable 'io_buffer'.

bsr        print_string

move.l     #io_buffer, d1

bsr        print_address

lea        content_msg(pc), a0    ; Print content of io_buffer variable.

bsr        print_string

move.l     io_buffer, d1

bsr        print_content

lea        length_msg(pc), a0     ; Print address of variable 'length'.

bsr        print_string

move.l     #length, d1

bsr        print_address

lea        content_msg(pc), a0    ; Print content of length variable.

bsr        print_string

move.l     length, d1

bsr        print_content

lea        extract_msg(pc), a0    ; Print add of variable 'extract_pointer'.

bsr        print_string

move.l     #extract_pointer, d1

bsr        print_address

lea        content_msg(pc), a0    ; Print content of extract_pointer.

bsr        print_string

move.l     extract_pointer, d1

bsr        print_content

lea        insert_msg(pc), a0     ; Print add of variable 'insert_pointer'.

bsr        print_string

move.l     #insert_pointer, d1

bsr        print_address

lea        content_msg(pc), a0    ; Print content of insert_pointer.

bsr        print_string

move.l     insert_pointer, d1

bsr        print_content

lea        end_msg(pc), a0        ; Print add of variable 'buffer_end'.

bsr        print_string

move.l     #buffer_end, d1

bsr        print_address

lea        content_msg(pc), a0    ; Print content of buffer_end.

bsr        print_string

move.l     buffer_end, d1

bsr        print_content

wait_for_keypress:              ; Give time to write down memory addresses.

move.w     #8, -(sp)           ; Function = c_necin = GEMDOS $8.

trap       #1                  ; GEMDOS call.

addq.l     #2, sp

relinquish_processor_control:   ; Maintain memory residency.

move.w     #0, -(sp)           ; See page 121 of Internals book.

move.l     a3, -(sp)           ; Program size.

move.w     #$31, -(sp)         ; Function = ptermres = GEMDOS $31.

trap       #1

not_decimal:

pea        message_1(pc)       ; Print error message.

move.w     #9, -(sp)           ; Function = c_conws = GEMDOS $9.

trap       #1                  ; GEMDOS call

addq.l     #6, sp              ; Reset stack pointer to top of stack.

bra.s      terminate

too_many_characters:

pea        message_2(pc)       ; Print error message.

move.w     #9, -(sp)           ; Function = c_conws = GEMDOS $9.

trap       #1                  ; GEMDOS call

addq.l     #6, sp              ; Reset stack pointer to top of stack.

terminate:                      ; Wait for keypress.

move.w     #8, -(sp)           ; Function = GEMDOS $8 = cnecin.

trap       #1

addq.l     #2, sp

move.w     #0, -(sp)           ; Function = p_term_old = GEMDOS $0.

trap       #1                  ; GEMDOS call.

; In the following discussion, whenever the word "printer" is used,

; realize that it includes any hardware print buffer that is connected

; between the computer and the printer.  In other words, the word printer,

; as it is used below, can mean printer, print buffer or the peripheral

; combination.  Note also that an external print buffer can function while

; the printer is off; that is, it can receive characters and store them.

; This means that you can complete a lot of print handling testing with the

; printer off to save paper.  To get rid of text in an external buffer, just

; press its clear button; to print the data stored, just turn on the printer.

; The interrupt handler is activated whenever pin 11 of the centronics

; interface is forced from a high to a low logic level.  The printer does

; does this when it turned on and when it sends a busy signal over pin 11.

; When the printer is off, the logic level of pin 11 is constantly high, but

; during the time that the printer is busy receiving a character, the logic

; level of pin 11 is placed high and held there only until the printer is

; ready to receive another character from the computer.  It is the falling

; edge of the logic level pulse generated by the printer that causes an MFP

; pin 0 interrupt.  This can be verified by observing the contents of the

; MFP's Active Edge Register (AER) located at address $FFFA03.  The content

; of that register is 4, therefore, the GPIP bit 0 input pin generates an

; interrupt on a one-to-zero transition.  Reference pages 4-1 and 4-2 of

; the Motorola MC68901 Multi-Function Peripheral manual and pages 32 and

; 60 of the Internals book.

; So, whenever the interrupt occurrs, we know that the printer is ready to

; receive a character, therefore, we can immediately send one to it.

; However, when we want to send a character, but we have no knowledge of

; the printer's capability to receive, we must perform a test to detect the

; printers state.  This is done by testing bit #0 of the MFP's General

; Purpose I/O data register (GPIP).  If that bit is a zero, then the printer

; is ready to receive a character.  The trap handler must perform this test.

; NOTE: Both the interrupt handler and the trap handler use a similiar

;       algorithm to print.  A common subroutine was not used (but could

;       be used) because the additional time required by the interrupt

;       handler would be 34 clock cycles for each character printed.  It

;       is the interrupt handler that actually does the bulk of the printing.

;       The more time used by the interrupt handler, the less time

;       available to the process you are running while printing is being

;       accomplished via the interrupt handler.

interrupt_handler:              ; Parallel interface pin #11 interrupt handler.

movem.l    d0/a0-a1, -(sp)     ; Save content of registers used by handler.

lea        io_buffer, a0       ; Load buffer structure pointer into A0.

movea.l    8(a0), a1           ; Content of buffer extract pointer into A1.

; NOTE:    A1 is now a simulated extract pointer.  It contains the location

;          from which the next character is to be fetched.  The simulated

;          pointer is used to scout ahead and determine what would happen

;          if the actual extract pointer were incremented.

cmpa.l     12(a0), a1          ; Compare location stored in insert pointer

; to location stored in simulated extract

; pointer.

beq.s      buffer_is_empty     ; Branch if location in insert pointer is equal

; to location stored in simulated extract

; pointer.

addq.l     #1, a1              ; Increment simulated extract pointer to

; location of next character.

cmpa.l     16(a0), a1          ; Compare buffer_end to location stored in

; simulated extract pointer.

bcs.s      end_of_buffer_not_reached

movea.l    (a0), a1            ; Move simulated extract pointer to front of

; buffer by storing buffer address in simulated

; extract pointer because the simulated extract

; pointer has reached the buffer limit.

end_of_buffer_not_reached:

move.b     (a1), d0            ; Fetch character at location stored in

; simulated extract pointer.

move.l     a1, 8(a0)           ; Put location stored in simulated extract

; pointer in actual extract pointer.

print_character:

lea        $FF8800, a1         ; Address of Programmable Sound Generator.

; *************************************************************************

andi.b     #$DF, d0            ; Convert character to upper case.

; The above statement is used to determine the frequency of interrupt handler

; use by various printing programs.  It does this by converting lower case

; characters to upper case each time a character is processed by this

; interrupt handler.  For a properly functioning printer buffer, the above

; statement must be removed.  Experiments show that most of the characters

; handled by the print buffer will be processed by the interrupt handler,

; unless there is an external print buffer connected between the printer

; and the parallel port, or unless the printer contains a significantly

; large buffer and that buffer is able to keep up with the flow of characters

; into the trap #13 handler.

; *************************************************************************

move.b     #$F, (a1)           ; Select port B (register 17) of PSG.

move.b     d0, 2(a1)           ; Write character to PSG register 17.

move.b     #$E, (a1)           ; Select port A (register 16) of PSG.

move.b     (a1), d0            ; Get current register 16 value.

strobe_low:

andi.b     #$DF, d0            ; Bit 5 to zero.

move.b     d0, 2(a1)           ; bit 5 of port A to zero.

strobe_high:

ori.b      #$20, d0            ; Bit 5 to one.

move.b     d0, 2(a1)           ; Bit 5 of port A to one.

buffer_is_empty:

bclr       #0, $FFFA11         ; Clear MFP in service bit.

movem.l    (sp)+, d0/a0-a1     ; Restore registers used by handler.

rte

trap_13_handler:

move.l    usp, a0              ; Load address of current top of user stack.

get_processor_status:

btst      #5, (sp)             ; User mode test.

beq.s     was_user_mode        ; No adjustment is necessary if the

; processor was in user mode.

movea.l   sp, a0               ; Load current top of supervisor stack.

addq.l    #6, a0               ; Adjust SSP for user mode type data access.

was_supervisor_mode:

was_user_mode:                  ; Processing for either mode follows.

tst.w      2(a0)               ; Is device the printer?

bne        not_printer

cmpi.w     #3, (a0)            ; Writing a character to a device?

bne        not_bconout_call

fetch_character:

move.w     4(a0), d0

move.w     #$2700, SR          ; Disable interrupts.

; NOTE: If interrupts are not disabled during this critical portion of the

;       trap handler algorithm, the interrupt handler could be invoked and

;       screw up the buffer pointers.

buffer_empty_test:              ; Want to know if buffer is empty or not.

lea        io_buffer, a0       ; Fetch buffer structure address.

; NOTE: The address stored in the insert pointer is placed in register A1

;       for two reasons; (1) for comparison with the extract pointer,

;       (2) in order to simulate an increment of the address later.

movea.l    12(a0), a1          ; Fetch content of buffer insert pointer.

cmpa.l     8(a0), a1           ; Compare content of extract pointer with

; content of insert pointer to determine if

bne        buffer_not_empty    ; buffer is empty.

try_to_print_character:

btst       #0, $FFFA01         ; Is printer (or hardware print buffer)

; busy (or off)?

bne.s      printer_busy        ; Branch if either peripheral is busy

; or off.

print_character_if_not_busy:    ; Means peripherals are not busy and are on.

lea        $FF8800, a1         ; Address of Programmable Sound Generator.

move.b     #$F, (a1)           ; Select port B of PSG.

move.b     d0, 2(a1)           ; Write character to PSG.

move.b     #$E, (a1)           ; Select port A of PSG.

move.b     (a1), d1            ; Get current register 16 value.

_strobe_low:

andi.b     #$DF, d1

move.b     d1, 2(a1)

_strobe_high:

ori.b      #$20, d1

move.b     d1, 2(a1)

moveq      #-1, d0             ; In case calling source wants to know

rte                            ; that character was dispatched.

buffer_not_empty:

printer_busy:

addq.l     #1, a1              ; Simulate insert location increment.

cmpa.l     16(a0), a1          ; Compare buffer_end to simulated insert

; location.

bcs.s      not_end_of_buffer   ; Branch if simulated location is not equal

; to the end of buffer location.

movea.l    (a0), a1            ; Simulate a reset of insert pointer to the

; first buffer location; that is, the front of

; the buffer is the new simulated insert

; location.

not_end_of_buffer:

cmpa.l     8(a0), a1           ; Compare location stored in extract pointer

; to simulated insert location.

beq.s      buffer_full         ; If they are equal, the buffer is full.

store_character_in_buffer:

move.b     d0, (a1)            ; Store character at the now valid location.

move.l     a1, 12(a0)          ; Store simulated location as the new insert

; location in the insert pointer.

moveq      #-1, d0             ; In case calling source wants to know

rte                            ; that character was dispatched.

buffer_full:

move.l     $4BA, d1            ; Fetch current _hz_200 value.

add.l      #$BB8, d1           ; Add count for desired elapsed time value.

; NOTE: The content of $4BA is incremented every 5 milliseconds.  Compute

;       the value to add to the current count by dividing the number of

;       milliseconds to wait (equals number of seconds times 1000) by 5.

;       In this case, to wait 15 seconds, 15000/5 = 3000 = $BB8.

move.w     #$2300, SR          ; Enable interrupts.

; NOTE: Here interrupts are enabled because the trap handler algorithm may

;       be idle for some time.  During this idle time, invoked interrupts

;       would have to remain pending.  There is a maximum amount of time

;       that an interrupt may remain pending, and that time depends on the

;       number of instructions a program can execute with the interrupts

;       masked off.  The amount of time that interrupts can be disabled is

;       usually restricted.

;       Reference: Programming the 68000 by Steve Williams, page 366.

;                  Published by SYBEX, Inc. 1985

wait:

cmpa.l     8(a0), a1           ; Compare location stored in extract pointer to

; location stored in simulated insert pointer.

bne.s      store_character_in_buffer

cmp.l      $4BA, d1            ; Elapsed time check.

bhi.s      wait                ; Wait until there is room in the buffer for

; another character, or until the programmed

; wait time has expired.

moveq      #0, d0              ; In case calling source wants to know

rte                            ; that character was not dispatched.

not_bconout_call:

cmpi.w     #8, (a0)            ; Requesting output device status?

bne.s      not_bcostat_call

; NOTE: The buffer status is returned in response to the bcostat invocation,

;       not the printer status.

determine_software_buffer_status:

moveq      #-1, d0             ; Assume buffer is not full.

lea        io_buffer, a0       ; Fetch buffer structure pointer.

move.l     12(a0), a1          ; Fetch insert pointer for simulation.

addq.l     #1, a1              ; Simulate moving insert pointer to next buffer

; location.

cmpa.l     16(a0), a1          ; Compare buffer_end to simulated insert

; location.

bcs.s      not_buffer_end

movea.l    (a0), a1            ; Simulate resetting insert pointer to front of

; buffer.

not_buffer_end:

cmpa.l     8(a0), a1           ; Compare location stored in extract pointer

; to simulated insert location.

bne.s      buffer_not_full     ; Buffer not full if extract location is not

; equal to simulated insert location.

moveq      #0, d0              ; Return buffer status to calling source.

buffer_not_full:                ; Return 0 if full, -1 if not.

rte

not_bcostat_call:

not_printer:

movea.l    preempted_trap_13_address, a0

jmp        (a0)                ; JUMP TO PREEMPTED TRAP #13 HANDLER.

;

; SUBROUTINES

;

; The binary to ASCII hexadecimal conversion routine expects a number to be

; passed as a longword in register D1.  Beginning with the most significant

; nibble (a nibble = four bits), each nibble is converted to its ASCII

; hexadecimal equivalent and stored in "hexadecimal", a null terminated

; buffer.  Maximum size of the binary number is 32 bits = 8 nibbles.

; The algorithm discards leading zeroes.

; The conversion from binary nibble to hex digit is accomplished by

; extracting the character in the hex table that is located at the position

; defined by the decimal value of the nibble.  For example, if the nibble

; is "1111", the decimal value is 15; the 15th element of the hex table is

; the letter F.  The location in the table is specified by an offset from

; the address of the first character of the table, which is stored in A1.

; The value of the offset is stored in register D0.  The addressing mode

; used to locate the appropriate table entry is "address register indirect

; with offset".

bin_to_hex:                      ; Expects binary number in D1.

lea        hexadecimal, a0      ; A0 is pointer to array "hexadecimal".

tst.l      d1                   ; Test for contents = 0.

beq.s      zero_passed          ; Branch if number is 0.

lea        hex_table, a1        ; A1 is pointer to array "hex_table".

lea        hex_table, a1        ; A1 is pointer to array "hex_table".

moveq      #7, d2               ; D2 is the loop counter for 8 nibbles.

discard_leading_zeroes:

rol.l      #4, d1               ; Rotate most significant nibble to the

; least significant nibble position.

move.b     d1, d0               ; Copy least significant byte of D1 to D0.

andi.b     #$F, d0              ; Mask out most significant nibble of D0.

bne.s      store_digit          ; Branch and store if not leading zero.

dbra       d2, discard_leading_zeroes

continue:

rol.l      #4, d1               ; Rotate most significant nibble.

move.b     d1, d0               ; Copy least significant byte of D1 to D0.

andi.b     #$F, d0              ; Mask out most significant nibble of D0.

store_digit:

move.b     0(a1,d0.w), (a0)+    ; Store ASCII hexadecimal digit in buffer.

dbra       d2, continue         ; Continue looping until D2 = -1.

move.b     #0, (a0)             ; Terminate hexadecimal string with a null.

rts

zero_passed:

move.b     #$30, (a0)+         ; Store an ASCII zero in "hexadecimal".

move.b     #0, (a0)            ; Terminate ASCII hexadecimal string with null.

lea        hexadecimal, a0

rts

print_address:

bsr        bin_to_hex

lea        hexadecimal(pc), a0

bsr        print_string

rts

print_content:

bsr        bin_to_hex

lea        hexadecimal(pc), a0

bsr        print_string

bsr        print_newline

rts

print_string:                   ; Expects address of string to be in A0.

pea        (a0)                ; Push address of string onto stack.

move.w     #9, -(sp)           ; Function = c_conws = GEMDOS $9.

trap       #1                  ; GEMDOS call

addq.l     #6, sp              ; Reset stack pointer to top of stack.

rts

print_newline:                  ; Prints a carriage return and linefeed.

pea        newline             ; Push address of string onto stack.

move.w     #9, -(sp)           ; Function = c_conws = GEMDOS $9.

trap       #1                  ; GEMDOS call

addq.l     #6, sp

rts

data

hex_table:    dc.b  '0123456789ABCDEF'

newline:      dc.b  $D,$A,0

load_message: dc.b  'Installing PRT_BUF program between hex addresses: ',0

separator:    dc.b  ' - ',0

trap_13_msg:  dc.b  'Custom trap #13 address:    $',0

int_hand_msg: dc.b  'Interrupt handler address:  $',0

io_buffer_msg:dc.b  'variable io_buffer address: $',0

length_msg:   dc.b  'variable length address:    $',0

extract_msg:  dc.b  'extract_pointer address:    $',0

insert_msg:   dc.b  'insert_pointer address:     $',0

end_msg:      dc.b  'buffer_end pointer address: $',0

content_msg:  dc.b  '  content = $',0

message_1:    dc.b 'Nondecimal character digit detected on parameter line.'

dc.b $D,$A,'Press Return key to terminate.',$D,$A,0

message_2:    dc.b 'Parameter line input limit is 4 decimal digits.',$D,$A

dc.b 'The program converts the input to thousands of bytes.'

dc.b  $D,$A,'Default buffer size is 100 bytes.',$D,$A

dc.b  'Press Return key to terminate.',$D,$A,0

align

; The variable "io_buffer" is a pointer to the print buffer's starting

; address.  The address of io_buffer also serves as the address of a

; structure that is composed of the variables io_buffer, at offset 0,

; length, at offset 4, extract_pointer (also called the buffer output pointer),

; at offset 8, and insert_pointer (also called the buffer input pointer), at

; offset 12.  In this service, the address of io_buffer provides the means

; by which the other members of the structure may be referrenced.

; Characters are placed in the buffer at the address stored in the variable

; 'insert_pointer'; they are extracted from the buffer and sent to the printer

; from the address stored in the variable 'extract_pointer'.

; Time is saved because only the address of io_buffer need be loaded into

; an address register.  The other variables in the structure are referenced

; by adding their offsets to the content of the chosen address register.

; NOTE: Because this program is assembled in Relocatable mode, the run time

;       address for the variable "buffer" will be stored in the four pointers

;       at which it is declared in the io_buffer structure below.

io_buffer:       dc.l  buffer     ; Pointer to buffer's starting address.

length:          dc.l     $64     ; Default buffer length = 100 bytes.

extract_pointer: dc.l  buffer     ; Buffer character output pointer.

insert_pointer:  dc.l  buffer     ; Buffer character input pointer.

buffer_end:      dc.l  buffer     ; Last address in buffer = buffer + length.

bss

preempted_trap_13_address:  ds.l   1

hexadecimal:  ds.l   3     ; Output buffer.  Must be NULL terminated.

ds.l  96

stack:        ds.l   1

buffer:       ds.l   0     ; Length of buffer must be added to length of

program_end:  ds.l   0     ; program to determine total size to pass to

end                       ; GEMDOS function $31.

How the Circular Print Buffer Functions

I have prepared a document to be printed while the handlers

of program 66 are installed in ram.  The document is shown as

Document 1.  This document can be printed with PRG_7DP.TTP, but

there is no reason that any other program which uses system

functions to send data to the parallel port can't be used.

Document 1, SPOOLTST.DOC, has been prepared entirely in lower

case characters.  Within the interrupt handler of program 66,

there is an instruction that converts lower case characters to

upper case.  This instruction wrecks havoc when punctuation and

numeric characters are converted, so they have been omitted from

SPOOLTST.DOC.  The purpose of document 1 is to assist my

descriptions of program 66 with a visual representation of

character processing by the handlers installed by the program.

Baring major differences between our printers and computers, the

first line of SPOOLTST.DOC should appear in lower case, if your

printer contains only a single line buffer, and the rest of the

document should appear in upper case.

Document 1. SPOOLTST.DOC has been prepared entirely in

lower case so that you can determine which characters

are processed by program 66's trap #13 handler and

which characters are processed by the interrupt handler

when the document is printed.

punctuation this file dont need no stinking punctuation

all of the text in this file is lower case and no punctuation is used because

the purpose of this file is to permit the testing of circular print buffers

that contain a parallel interface interrupt handler and a trap thirteen handler

in the interrupt handler just before each character is printed there should be

an instruction to convert the character to upper case

all characters printed by the trap thirteen handler will be printed in lower

case therefore the printed output will separate characters printed by each

handler

such a test is conducted to prove that both handlers are functioning as expected

the number of characters of the file being printed that are processed by the

trap thirteen handler depends on the size of any buffers which exist between

the parallel interface port and the printer's print mechanism

this is so because such buffers can usually keep up with the flow of characters

into the trap handler

when that situation exists the trap handler does not perceive a busy signal

on pin 13 of the interface therefore no characters are stored in the trap

thirteen print buffer

instead the characters are sent immediately to the port

for example with only my star nx ten connected to the parallel port and with

its internal five kilobyte buffer disabled there exists within the printer

only a one line buffer

therefore the first line of text in this file will be processed by the trap

thirteen handler because the buffer response is fast enough to accept those

characters as they flow into the trap handler so no characters are stored in

the trap buffer

but as soon as the printer line buffer becomes full the trap thirteen handler

will begin storing characters in the buffer

when the attention of the printer returns to the parallel port the logic

state of pin eleven will drop from high to low and trigger the interrupt

handler into action

thereafter all characters will be processed by the interrupt handler

because the printer line buffer will not be empty again until all characters

of the file have been printed

if there is a print buffer between the parallel port and the printer or if the

printer five kilobyte internal buffer is enabled and if these other buffers are

able to keep up with the flow of characters into the trap thirteen handler then

then the interrupt handler may never be invoked

at least it will not be utilized until any and all external buffers are full

the point is this if the data transfer rate through the parallel port is fast

enough so that a character can be sent there as soon as it is detected by the

trap thirteen handler then all characters that are detected by the trap handler

are also printed by the trap handler in such cases the parallel port interface

interrupt handler does not function

Significantly, the system functions which exchange data

through the parallel interface port do so via trap #13

invocations.  Therefore, program 66 installs a custom trap #13

handler which intercepts all trap #13 invocations.  When the

program 66 specific handler is installed, it preempts whatever

trap #13 handler is installed before it; the address of that

handler is stored in a variable in program 66.  Those invocations

which are not relevant to the transfer of data through the

parallel interface are simply passed on to the preempted handler

by jumping to its address.

Program 66 also installs an interrupt handler that is

invoked each time the logic level of pin 11 of the parallel

interface port drops from high to low.  The installation of the

interrupt handler does not assume any prior installation; that

is, it does not save the address of any handler which may be

installed before the program 66 custom interrupt handler.  In

addition, program 66 creates a buffer in which characters are

stored by the trap handler and extracted by the interrupt

handler.

Refer to figure 9.1 during the following discussion.  Each

time the trap #13 handler is invoked by a program trying to send

data to the printer, the handler tries to send the character to

the printer immediately if program 66's buffer is empty and the

logic level of pin 11 of the parallel interface is low.  If the

buffer is not empty, or if pin 11 is high, then the character is

immediately stored, unless the buffer is full.  If the buffer is

full, a loop is executed until there is room for the character in

the buffer.  Assume that the buffer is empty because it has not

yet been used.  The first pictorial in figure 9.1 illustrates

this situation.  Both the insert pointer and the extract pointer

contain the address of the first buffer element.

Now, assume that a printing program begins to send

characters to the printer.  Further, assume that the printer

contains an 80 character buffer.  The response of the buffer is

quick enough so that the first 80 characters flow through the

handler directly to the printer.  Now the printer starts putting

characters on paper.  (Note this: the printer will begin to print

when its line buffer is full (assuming that any larger printer

buffer is disabled) or when it receives a carriage return.)  As

soon as the printer's line buffer becomes full logic level of pin

11 goes high long enough so that the trap handler is forced to

begin storing characters in program 66's buffer.  The second

pictorial in figure 9.1 depicts the situation just after the

first character has been stored.  From this point on, no

characters will be sent directly from the trap handler, unless

some event occurs that permits the extract pointer to catch up

with the insert pointer, which is an indication that the buffer

is empty.

The trap handler continues to insert characters into the

buffer in a linear fashion until the insert pointer reaches the

end of the buffer; in fact, it stops just short of the end

because within the trap handler algorithm the end of buffer test

is performed with a simulated insert pointer.  This simulated

pointer scouts ahead to determine if the actual pointer can be

safely moved.  The third pictorial of figure 9.1 illustrates this

instance.  Note that the last element of the buffer never

receives a character.  When the end of the buffer is reached, the

insert pointer must be wrapped around to the front of the buffer;

it is this action that instigates the circular configuration of

the buffer; that is, the buffer activity simulates that which it

would be if the buffer were circular.  Refer to the fourth

pictorial.

Figure 9.1 Circular buffer pointer activity.  The extract pointer

is represented by the letter E; the insert pointer by the letter

I.  Characters which have been inserted into the buffer, but have

not yet been extracted are represented by the letter C;

characters which have been extracted by the letter X.

While the insert pointer is placing characters into the

buffer at a transfer rate that depends on speed of the trap

handler and the transfer rate of the printing program, the

interrupt handler continues to extract characters from the

buffer.  Remember that the interrupt handler extraction process

began when a falling edge of the pulses being placed on pin 11 by

the printer invoked the interrupt handler at a time when the

buffer contained one or more characters.  (Note this: if program

66's buffer somehow became full, but nothing occurred to cause

the logic level on pin 11 to drop from high to low, the interrupt

handler extraction process would not begin.)

If character insertion continues past the wrap around point,

the new characters will simply be placed in previously occupied

buffer locations.  If the number of characters sent to the buffer

exceeds the buffer capacity, eventually the insert pointer will

catch up with the extract pointer; this is the full buffer

condition illustrated by pictorial 5 of figure 9.1.  When that

occurs, the trap handler wait loop is executed for a short period

of time until there is room for the character in holding.  When

all characters have been inserted, the insert pointer will be

idle, and the extract pointer will arrive at the location to

which the insert pointer is pointing.  This event simulates the

empty buffer condition depicted by pictorial 6.  I use the word

simulate here only to indicate that the buffer still contains the

characters not overwritten by insertions, but the buffer is

effectively empty because all characters have been sent to the

printer.

As stated previously, program 66 was prepared for use as a

tutorial agent.  Programs 67 and 68 are the practical versions of

the program; in these programs the instructions that print out

the locations of pertinent addresses have been omitted, as has

been the instruction which converts characters in the interrupt

handler.  Program 67 has been designed for assembly in the

Relocatable mode; program 68 for the PC-relative mode.  Having

the two versions to examine will permit you to see the special

handling required for the PC-relative version as compared to the

Relocatable version.

Program 67. A practical version of program 66, which is

designed for assembly in AssemPro's Relocatable mode.

; Program Name: PRG_7HR.S

;      Version: 1.007

; Assembly Instructions:

;      Assemble in Relocatable mode and save with a TOS or TTP suffix.

; Program Function:

;      Circular printer buffer.  The default buffer size for the TOS version

; is 32,768 bytes.  See PRG_7GR.S for further documentation.

; Execution Instructions:

;      May be executed from the desktop with a TOS or TTP extension.  May be

; executed from an AUTO folder if it has a PRG extension.

;      If a larger buffer size is desired, the program extension can be

; changed from TOS to TTP.  If a buffer size is declared on the TTP parameter

; line, any size, up to the maximum available machine memory, may be specified.

; When the TTP program is executed, type the desired size, in kilobytes, on the

; parameter line; the maximum number of digits which can be typed on the

; parameter line is 4.  Do not type spaces before the digits.  Do not type

; anything between digits.  Do not type anything after the digits, except the

; Return key.  The OK button may be clicked in lieu of a Return key press.

;      The length of the buffer will be (input X 1024) bytes.  Ex: if input is

; 32, length of the buffer will be 32,768 bytes.  After converting the ASCII

; decimal input to binary, it is much easier to simply shift the binary result

; 10 bits left to multiply by 1024, than it is to multiply it by 1000.

;      The buffer length (either the default size or the specified size) is

; combined with the program size to determine the parameter which must be

; passed to GEMDOS function $31.

program_start:                   ; Calculate program size and retain result.

lea        -$82(pc), a4         ; Fetch "command line" address.

lea        -$80(a4), a0         ; Fetch "basepage" address.

lea        program_end(pc), a3  ; Fetch "end of program" address.

suba.l     a0, a3               ; Program size is in A3.

lea        stack(pc), a7        ; Fetch address of the program stack.

process_input_parameter:         ; Calculate requested buffer length.

lea        length(pc), a0       ; Fetch address of the length variable.

moveq      #0, d0               ; Clear counter register.

move.b     (a4)+, d0            ; Fetch parameter line character count.

beq.s      compute_size         ; Branch if no parameter--use default length.

cmpi.b     #4, d0               ; Greater than four test.

bgt.s      too_many_characters  ; Branch if more than four characters.

subq.b     #1, d0               ; Set up counter.

ext.w      d0                   ; Extend to match size of DBRA instruction.

moveq      #0, d2               ; Initialize accumulator.

moveq      #0, d1               ; Clear scratch register.

fetch_digit:                     ; Convert input from ASCII decimal to binary.

move.b     (a4)+, d1            ; ASCII decimal digit to D1.

sub.b      #$30, d1             ; Convert ASCII decimal digit to decimal digit.

bmi.s      not_decimal          ; Branch if digit is less than zero.

cmp.b      #9, d1               ; Greater than nine test.

bgt.s      not_decimal          ; Branch if digit is greater than nine.

multiply_accumulator_content_by_ten:

move.l     d2, d3               ; Copy accumulator in D3.

lsl.l      #3, d2               ; Shift accumulator content to multiply by 8.

add.l      d3, d2               ; Add D2's original quantity twice to

add.l      d3, d2               ; complete the multiplication by 10.

add_new_digit_to_accumulator_content:

add.l      d1, d2               ; Add decimal number in D1 to accumulator.

dbra       d0, fetch_digit      ; Loop until d0 becomes negative.

extend_to_thousands_of_bytes:    ; Length will be (input X 1000) plus

moveq      #10, d3              ; some even number of bytes that is less

lsl.l      d3, d2               ; than 1000.  Ex: if input is 32, length

move.l     d2, (a0)             ; is 32768.  Store requested buffer size.

compute_size:                    ; This will be parameter to GEMDOS $31.

adda.l     (a0), a3             ; Add buffer length to program size.

compute_and_store_buffer_end:

lea        buffer(pc), a1       ; Fetch buffer starting address.

adda.l     (a0), a1             ; Add buffer length to buffer start address.

move.l     a1, buffer_end       ; Store buffer_end address.

install_custom_trap_13_vector:

pea        trap_13_handler(pc)  ; Push custom trap handler address onto stack.

move.w     #$2D, -(sp)          ; Trap 13 vector number.

move.w     #5, -(sp)            ; Function = setexec.

trap       #13                  ; Current trap 13 vector returned in D0.

addq.l     #8, sp

move.l     d0, preempted_trap_13_address

install_interrupt_handler_vector:

pea        interrupt_handler(pc); Push interrupt handler address.

move.w     #0, -(sp)            ; Interrupt level for centronics ready.

move.w     #$D, -(sp)           ; Functon = XBIOS #13 dec = mfpint.

trap       #14

addq.l     #8, sp

relinquish_processor_control:    ; Maintain memory residency.

move.w     #0, -(sp)            ; See page 121 of Internals book.

move.l     a3, -(sp)            ; Program size (plus buffer length).

move.w     #$31, -(sp)          ; Function = ptermres = GEMDOS $31.

trap       #1

not_decimal:

pea        message_1(pc)        ; Print error message.

move.w     #9, -(sp)            ; Function = c_conws = GEMDOS $9.

trap       #1                   ; GEMDOS call

addq.l     #6, sp               ; Reset stack pointer to top of stack.

bra.s      terminate

too_many_characters:

pea        message_2(pc)        ; Print error message.

move.w     #9, -(sp)            ; Function = c_conws = GEMDOS $9.

trap       #1                   ; GEMDOS call

addq.l     #6, sp               ; Reset stack pointer to top of stack.

terminate:                       ; Wait for keypress.

move.w     #8, -(sp)            ; Function = GEMDOS $8 = cnecin.

trap       #1

addq.l     #2, sp

move.w     #0, -(sp)            ; Function = p_term_old = GEMDOS $0.

trap       #1                   ; GEMDOS call.

interrupt_handler:               ; Parallel interface pin #11 interrupt handler.

movem.l    d0/a0-a1, -(sp)      ; Save content of registers used handler.

lea        io_buffer(pc), a0    ; Load buffer structure pointer into A0.

movea.l    8(a0), a1            ; Content of buffer extract pointer into A1.

; NOTE:    A1 is now a simulated extract pointer.  It contains the location

;          from which the next character is to be fetched.  The simulated

;          pointer is used to scout ahead and determine what would happen

;          if the actual extract pointer were incremented.

cmpa.l     12(a0), a1           ; Compare location stored in insert pointer

; to location stored in simulated extract

; pointer.

beq.s      buffer_is_empty      ; Branch if location in insert pointer is equal

; to location stored in simulated extract

; pointer.

addq.l     #1, a1               ; Increment simulated extract pointer to

; location of next character.

cmpa.l     16(a0), a1           ; Compare buffer_end to location stored in

; simulated extract pointer.

bcs.s      end_of_buffer_not_reached

movea.l    (a0), a1             ; Move simulated extract pointer to front of

; buffer by storing buffer address in simulated

; extract pointer because the simulated extract

; pointer has reached the buffer limit.

end_of_buffer_not_reached:

move.b     (a1), d0             ; Fetch character at location stored in

; simulated extract pointer.

move.l     a1, 8(a0)            ; Put location stored in simulated extract

; pointer in actual extract pointer.

print_character:

lea        $FF8800, a1          ; Address of Programmable Sound Generator.

move.b     #$F, (a1)            ; Select port B (register 17) of PSG.

move.b     d0, 2(a1)            ; Write character to PSG register 17..

move.b     #$E, (a1)            ; Select port A (register 16) of PSG.

move.b     (a1), d0             ; Get current register 16 value.

strobe_low:

andi.b     #$DF, d0             ; Bit 5 to zero.

move.b     d0, 2(a1)            ; Bit 5 of port A to zero.

strobe_high:

ori.b      #$20, d0             ; Bit 5 to one.

move.b     d0, 2(a1)            ; Bit 5 of port A to one.

buffer_is_empty:

bclr       #0, $FFFA11          ; Clear MFP in service bit.

movem.l    (sp)+, d0/a0-a1      ; Restore registers used by handler.

rte

trap_13_handler:

move.l    usp, a0               ; Load address of current top of user stack.

get_processor_status:

btst      #5, (sp)              ; User mode test.

beq.s     was_user_mode         ; No adjustment is necessary if the

; processor was in user mode.

movea.l   sp, a0                ; Load current top of supervisor stack.

addq.l    #6, a0                ; Adjust SSP for user mode type data access.

was_supervisor_mode:

was_user_mode:                   ; Processing for either mode follows.

tst.w      2(a0)                ; Is device the printer?

bne        not_printer

cmpi.w     #3, (a0)             ; Writing a character to a device?

bne        not_bconout_call

fetch_character:

move.w     4(a0), d0

move.w     #$2700, SR           ; Disable interrupts.

; NOTE: If interrupts are not disabled during this critical portion of the

;       trap handler algorithm, the interrupt handler could be invoked and

;       screw up the buffer pointers.

buffer_empty_test:               ; Want to know if buffer is empty or not.

lea        io_buffer(pc), a0    ; Fetch buffer structure address.

; NOTE: The address stored in the insert pointer is placed in register A1

;       for two reasons; (1) for comparison with the extract pointer,

;       (2) in order to simulate an increment of the address later.

movea.l    12(a0), a1           ; Fetch buffer insert pointer.

cmpa.l     8(a0), a1            ; Compare content of extract pointer with

; content of insert pointer to determine if

bne.s      buffer_not_empty     ; buffer is empty.

try_to_print_character:

btst       #0, $FFFA01          ; Is printer (or hardware print buffer)

; busy (or off)?

bne.s      printer_busy         ; Branch if either peripheral is busy

; or off.

print_character_if_not_busy:     ; Means peripherals are not busy and are on.

lea        $FF8800, a1          ; Address of Programmable Sound Generator.

move.b     #$F, (a1)            ; Select port B of PSG.

move.b     d0, 2(a1)            ; Write character to PSG.

move.b     #$E, (a1)            ; Select port A of PSG.

move.b     (a1), d1             ; Get current register value.

_strobe_low:

andi.b     #$DF, d1

move.b     d1, 2(a1)

_strobe_high:

ori.b      #$20, d1

move.b     d1, 2(a1)

moveq      #-1, d0              ; In case calling source wants to know

rte                             ; that character was dispatched.

buffer_not_empty:

printer_busy:

addq.l     #1, a1               ; Simulate insert location increment.

cmpa.l     16(a0), a1           ; Compare buffer_end to simulated insert

; location.

bcs.s      not_end_of_buffer    ; Branch if simulated location is not equal

; to the end of buffer location.

movea.l    (a0), a1             ; Simulate a reset of insert pointer to the

; first buffer location; that is, the front of

; the buffer is the new simulated insert

; location.

not_end_of_buffer:

cmpa.l     8(a0), a1            ; Compare location stored in extract pointer

; to simulated insert location.

beq.s      buffer_full          ; If they are equal, the buffer is full.

store_character_in_buffer:

move.b     d0, (a1)             ; Store character at the now valid location.

move.l     a1, 12(a0)           ; Store simulated location as the new insert

; location in the insert pointer.

moveq      #-1, d0              ; In case calling source wants to know

rte                             ; that character was dispatched.

buffer_full:

move.l     $4BA, d1             ; Fetch current _hz_200 value.

add.l      #$BB8, d1            ; Add count for desired elapsed time value.

; NOTE: The content of $4BA is incremented every 5 milliseconds.  Compute

;       the value to add to the current count by dividing the number of

;       milliseconds to wait (equals number of seconds times 1000) by 5.

;       In this case, to wait 15 seconds, 15000/5 = 3000 = $BB8.

move.w     #$2300, SR           ; Enable interrupts.

; NOTE: Here interrupts are enabled because the trap handler algorithm may

;       be idle for some time.  During this idle time, invoked interrupts

;       would have to remain pending.  There is a maximum amount of time

;       that an interrupt may remain pending, and that time depends on the

;       number of instructions a program can execute with the interrupts

;       masked off.  The amount of time that interrupts can be disabled is

;       usually restricted.

;       Reference: Programming the 68000 by Steve Williams, page 366.

;                  Published by SYBEX, Inc. 1985

wait:

cmpa.l     8(a0), a1            ; Compare location stored in extract pointer to

; location stored in simulated insert pointer.

bne.s      store_character_in_buffer

cmp.l      $4BA, d1             ; Elapsed time check.

bhi.s      wait                 ; Wait until there is room in the buffer for

; another character, or until the programmed

; wait time has expired.

moveq      #0, d0               ; In case calling source wants to know

rte                             ; that character was not dispatched.

not_bconout_call:

cmpi.w     #8, (a0)             ; Requesting output device status?

bne.s      not_bcostat_call

; NOTE: The buffer status is returned in response to the bcostat invocation,

;       not the printer status.

determine_software_buffer_status:

moveq      #-1, d0              ; Assume buffer is not full.

lea        io_buffer(pc), a0    ; Fetch buffer structure pointer.

move.l     12(a0), a1           ; Fetch insert pointer for simulation.

addq.l     #1, a1               ; Simulate moving insert pointer to next buffer

; location.

cmpa.l     16(a0), a1           ; Compare buffer_end to simulated insert

; location.

bcs.s      not_buffer_end

movea.l    (a0), a1             ; Simulate resetting insert pointer to front of

; buffer.

not_buffer_end:

cmpa.l     8(a0), a1            ; Compare location stored in extract pointer

; to simulated insert location.

bne.s      buffer_not_full      ; Buffer not full if extract location is not

; equal to simulated insert location.

moveq      #0, d0               ; Return buffer status to calling source.

buffer_not_full:                 ; Return 0 if full, -1 if not.

rte

not_bcostat_call:

not_printer:

movea.l    preempted_trap_13_address, a0

jmp        (a0)                 ; JUMP TO PREEMPTED TRAP #13 HANDLER.

data

message_1:    dc.b 'Nondecimal character digit detected on parameter line.'

dc.b $D,$A,'Press Return key to terminate.',$D,$A,0

message_2:    dc.b 'Parameter line input limit is 4 decimal digits.',$D,$A

dc.b 'The program converts the input to thousands of bytes.'

dc.b  $D,$A,'Default buffer size is 32,768 bytes.',$D,$A

dc.b  'Press Return key to terminate.',$D,$A,0

align

; The variable "io_buffer" is a pointer to the print buffer's starting

; address.  The address of io_buffer also serves as the address of a

; structure that is composed of the variables io_buffer, at offset 0,

; length, at offset 4, extract_pointer (also called the buffer output pointer),

; at offset 8, and insert_pointer (also called the buffer input pointer), at

; offset 12.  In this service, the address of io_buffer provides the means

; by which the other members of the structure may be referrenced.

; Characters are placed in the buffer at the address stored in the variable

; 'insert_pointer'; they are extracted from the buffer and sent to the printer

; from the address stored in the variable 'extract_pointer'.

; Time is saved because only the address of io_buffer need be loaded into

; an address register.  The other variables in the structure are referenced

; by adding their offsets to the content of the chosen address register.

; NOTE: Because this program is assembled in Relocatable mode, the run time

;       address for the variable "buffer" will be stored in the four pointers

;       at which it is declared in the io_buffer structure below.

io_buffer:       dc.l  buffer     ; Pointer to buffer's starting address.

length:          dc.l   $8000     ; Default buffer length = 32,768 bytes.

extract_pointer: dc.l  buffer     ; Buffer character output pointer.

insert_pointer:  dc.l  buffer     ; Buffer character input pointer.

buffer_end:      dc.l  buffer     ; Last address in buffer = buffer + length.

bss

preempted_trap_13_address:  ds.l   1

ds.l  96

stack:           ds.l   1

buffer:          ds.l   0     ; Length of buffer must be added to length of

program_end:     ds.l   0     ; program to determine total size to pass to

end                          ; GEMDOS function $31.

Program 68. A practical version of program 66, which is

designed for assembly in AssemPro's PC-relative mode.

; Program Name: PRG_7HP.S

;      Version: 1.007

; Assembly Instructions:

;      Assemble in PC-relative mode and save with a TOS or TTP suffix.

; Program Function:

;      Circular printer buffer.  The default buffer size for the TOS version

; is 32,768 bytes.  See PRG_7GR.S for further documentation.

; Execution Instructions:

;      May be executed from the desktop with a TOS or TTP extension.  May be

; executed from an AUTO folder if it has a PRG extension.

;      If a larger buffer size is desired, the program extension can be

; changed from TOS to TTP.  If a buffer size is declared on the TTP parameter

; line, any size, up to the maximum available machine memory, may be specified.

; When the TTP program is executed, type the desired size, in kilobytes, on the

; parameter line; the maximum number of digits which can be typed on the

; parameter line is 4.  Do not type spaces before the digits.  Do not type

; anything between digits.  Do not type anything after the digits, except the

; Return key.  The OK button may be clicked in lieu of a Return key press.

;      The length of the buffer will be (input X 1024) bytes.  Ex: if input is

; 32, length of the buffer will be 32,768 bytes.  After converting the ASCII

; decimal input to binary, it is much easier to simply shift the binary result

; 10 bits left to multiply by 1024, than it is to multiply it by 1000.

;      The buffer length (either the default size or the specified size) is

; combined with the program size to determine the parameter which must be

; passed to GEMDOS function $31.

program_start:                   ; Calculate program size and retain result.

lea        -$82(pc), a4         ; Fetch "command line" address.

lea        -$80(a4), a0         ; Fetch "basepage" address.

lea        program_end, a3      ; Fetch "end of program" address.

suba.l     a0, a3               ; Program size is in A3.

lea        stack, a7            ; Fetch address of the program stack.

process_input_parameter:         ; Calculate requested buffer length.

lea        length, a0           ; Fetch address of the length variable.

moveq      #0, d0               ; Clear counter register.

move.b     (a4)+, d0            ; Fetch parameter line character count.

beq.s      compute_size         ; Branch if no parameter--use default length.

cmpi.b     #4, d0               ; Greater than four test.

bgt        too_many_characters  ; Branch if more than four characters.

subq.b     #1, d0               ; Set up counter.

ext.w      d0                   ; Extend to match size of DBRA instruction.

moveq      #0, d2               ; Initialize accumulator.

moveq      #0, d1               ; Clear scratch register.

fetch_digit:                     ; Convert input from ASCII decimal to binary.

move.b     (a4)+, d1            ; ASCII decimal digit to D1.

sub.b      #$30, d1             ; Convert ASCII decimal digit to decimal digit.

bmi.s      not_decimal          ; Branch if digit is less than zero.

cmp.b      #9, d1               ; Greater than nine test.

bgt.s      not_decimal          ; Branch if digit is greater than nine.

multiply_accumulator_content_by_ten:

move.l     d2, d3               ; Copy accumulator in D3.

lsl.l      #3, d2               ; Shift accumulator content to multiply by 8.

add.l      d3, d2               ; Add D2's original quantity twice to

add.l      d3, d2               ; complete the multiplication by 10.

add_new_digit_to_accumulator_content:

add.l      d1, d2               ; Add decimal number in D1 to accumulator.

dbra       d0, fetch_digit      ; Loop until d0 becomes negative.

extend_to_thousands_of_bytes:    ; Length will be (input X 1000) plus

moveq      #10, d3              ; some even number of bytes that is less

lsl.l      d3, d2               ; than 1000.  Ex: if input is 32, length

move.l     d2, (a0)             ; is 32768.  Store requested buffer size.

compute_size:                    ; This will be parameter to GEMDOS $31.

adda.l     (a0), a3             ; Add buffer length to program size.

store_io_buffer_structure_components:

; NOTE: This program is to be assembled in PC-relative mode; therefore, the

;       buffer start address must be explicitly stored in the appropriate

;       io_buffer structure components.  In the Relocatable assembled version

;       of the program, this need not be done because the operating system

;       conveniently handles this task when the program is loaded for

;       execution, forcing the run time buffer start address into each

;       pointer at which it is declared in the data section of the program.

lea        buffer, a1           ; Fetch buffer starting address.

lea        io_buffer, a2        ; Fetch io_buffer structure base address.

move.l     a1, (a2)             ; Buffer starting address to base pointer.

move.l     a1, 8(a2)            ; Also to extract_pointer.

move.l     a1, 12(a2)           ; Also to insert_pointer.

adda.l     (a0), a1             ; Add buffer length to buffer start address.

move.l     a1, 16(a2)           ; Store buffer_end address.

install_custom_trap_13_vector:

pea        trap_13_handler      ; Push custom trap handler address onto stack.

move.w     #$2D, -(sp)          ; Trap 13 vector number.

move.w     #5, -(sp)            ; Function = setexec.

trap       #13                  ; Current trap 13 vector returned in D0.

addq.l     #8, sp

lea        preempted_trap_13_address, a0

move.l     d0, (a0)

install_interrupt_handler_vector:

pea        interrupt_handler    ; Push interrupt handler address.

move.w     #0, -(sp)            ; Interrupt level for centronics ready.

move.w     #$D, -(sp)           ; Functon = XBIOS #13 dec = mfpint.

trap       #14

addq.l     #8, sp

relinquish_processor_control:    ; Maintain memory residency.

move.w     #0, -(sp)            ; See page 121 of Internals book.

move.l     a3, -(sp)            ; Program size (plus buffer length).

move.w     #$31, -(sp)          ; Function = ptermres = GEMDOS $31.

trap       #1

not_decimal:

pea        message_1            ; Print error message.

move.w     #9, -(sp)            ; Function = c_conws = GEMDOS $9.

trap       #1                   ; GEMDOS call

addq.l     #6, sp               ; Reset stack pointer to top of stack.

bra.s      terminate

too_many_characters:

pea        message_2            ; Print error message.

move.w     #9, -(sp)            ; Function = c_conws = GEMDOS $9.

trap       #1                   ; GEMDOS call

addq.l     #6, sp               ; Reset stack pointer to top of stack.

terminate:                       ; Wait for keypress.

move.w     #8, -(sp)            ; Function = GEMDOS $8 = cnecin.

trap       #1

addq.l     #2, sp

move.w     #0, -(sp)            ; Function = p_term_old = GEMDOS $0.

trap       #1                   ; GEMDOS call.

interrupt_handler:               ; Parallel interface interrupt handler.

movem.l    d0/a0-a1, -(sp)      ; Save content of registers used by handler.

lea        io_buffer, a0        ; Load buffer structure pointer into A0.

movea.l    8(a0), a1            ; Content of buffer extract pointer into A1.

; NOTE:    A1 is now a simulated extract pointer.  It contains the location

;          from which the next character is to be fetched.  The simulated

;          pointer is used to scout ahead and determine what would happen

;          if the actual extract pointer were incremented.

cmpa.l     12(a0), a1           ; Compare location stored in insert pointer

; to location stored in simulated extract

; pointer.

beq.s      buffer_is_empty      ; Branch if location in insert pointer is equal

; to location stored in simulated extract

; pointer.

addq.l     #1, a1               ; Increment simulated extract pointer to

; location of next character.

cmpa.l     16(a0), a1           ; Compare buffer_end to location stored in

; simulated extract pointer.

bcs.s      end_of_buffer_not_reached

movea.l    (a0), a1             ; Move simulated extract pointer to front of

; buffer by storing buffer address in simulated

; extract pointer because the simulated extract

; pointer has reached the buffer limit.

end_of_buffer_not_reached:

move.b     (a1), d0             ; Fetch character at location stored in

; simulated extract pointer.

move.l     a1, 8(a0)            ; Put location stored in simulated extract

; pointer in actual extract pointer.

print_character:

lea        $FF8800, a1          ; Address of Programmable Sound Generator.

move.b     #$F, (a1)            ; Select port B (register 17) of PSG.

move.b     d0, 2(a1)            ; Write character to PSG register 17.

move.b     #$E, (a1)            ; Select port A (register 16) of PSG.

move.b     (a1), d0             ; Get current register 16 value.

strobe_low:

andi.b     #$DF, d0             ; Bit 5 to zero.

move.b     d0, 2(a1)            ; Bit 5 of port A to zero.

strobe_high:

ori.b      #$20, d0             ; Bit 5 to one.

move.b     d0, 2(a1)            ; Bit 5 of port A to one.

buffer_is_empty:

bclr       #0, $FFFA11          ; Clear MFP in service bit.

movem.l    (sp)+, d0/a0-a1      ; Restore registers used by handler.

rte

trap_13_handler:

move.l    usp, a0               ; Load address of current top of user stack.

get_processor_status:

btst      #5, (sp)              ; User mode test.

beq.s     was_user_mode         ; No adjustment is necessary if the

; processor was in user mode.

movea.l   sp, a0                ; Load current top of supervisor stack.

addq.l    #6, a0                ; Adjust SSP for user mode type data access.

was_supervisor_mode:

was_user_mode:                   ; Processing for either mode follows.

tst.w      2(a0)                ; Is device the printer?

bne        not_printer

cmpi.w     #3, (a0)             ; Writing a character to a device?

bne        not_bconout_call

fetch_character:

move.w     4(a0), d0

move.w     #$2700, SR           ; Disable interrupts.

; NOTE: If interrupts are not disabled during this critical portion of the

;       trap handler algorithm, the interrupt handler could be invoked and

;       screw up the buffer pointers.

buffer_empty_test:               ; Want to know if buffer is empty or not.

lea        io_buffer, a0        ; Fetch buffer structure address.

; NOTE: The address stored in the insert pointer is placed in register A1

;       for two reasons; (1) for comparison with the extract pointer,

;       (2) in order to simulate an increment of the address later.

movea.l    12(a0), a1           ; Fetch content of buffer insert pointer.

cmpa.l     8(a0), a1            ; Compare content of extract pointer with

; content of insert pointer to determine if

bne        buffer_not_empty     ; buffer is empty.

try_to_print_character:

btst       #0, $FFFA01          ; Is printer (or hardware print buffer)

; busy (or off)?

bne.s      printer_busy         ; Branch if either peripheral is busy

; or off.

print_character_if_not_busy:     ; Means peripherals are not busy and are on.

lea        $FF8800, a1          ; Address of Programmable Sound Generator.

move.b     #$F, (a1)            ; Select port B of PSG.

move.b     d0, 2(a1)            ; Write character to PSG.

move.b     #$E, (a1)            ; Select port A of PSG.

move.b     (a1), d1             ; Get current register value.

_strobe_low:

andi.b     #$DF, d1

move.b     d1, 2(a1)

_strobe_high:

ori.b      #$20, d1

move.b     d1, 2(a1)

moveq      #-1, d0              ; In case calling source wants to know

rte                             ; that character was dispatched.

buffer_not_empty:

printer_busy:

addq.l     #1, a1               ; Simulate insert location increment.

cmpa.l     16(a0), a1           ; Compare buffer_end to simulated insert

; location.

bcs.s      not_end_of_buffer    ; Branch if simulated location is not equal

; to the end of buffer location.

movea.l    (a0), a1             ; Simulate a reset of insert pointer to the

; first buffer location; that is, the front of

; the buffer is the new simulated insert

; location.

not_end_of_buffer:

cmpa.l     8(a0), a1            ; Compare location stored in extract pointer

; to simulated insert location.

beq.s      buffer_full          ; If they are equal, the buffer is full.

store_character_in_buffer:

move.b     d0, (a1)             ; Store character at the now valid location.

move.l     a1, 12(a0)           ; Store simulated location as the new insert

; location in the insert pointer.

moveq      #-1, d0              ; In case calling source wants to know

rte                             ; that character was dispatched.

buffer_full:

move.l     $4BA, d1             ; Fetch current _hz_200 value.

add.l      #$BB8, d1            ; Add count for desired elapsed time value.

; NOTE: The content of $4BA is incremented every 5 milliseconds.  Compute

;       the value to add to the current count by dividing the number of

;       milliseconds to wait (equals number of seconds times 1000) by 5.

;       In this case, to wait 15 seconds, 15000/5 = 3000 = $BB8.

move.w     #$2300, SR           ; Enable interrupts.

; NOTE: Here interrupts are enabled because the trap handler algorithm may

;       be idle for some time.  During this idle time, invoked interrupts

;       would have to remain pending.  There is a maximum amount of time

;       that an interrupt may remain pending, and that time depends on the

;       number of instructions a program can execute with the interrupts

;       masked off.  The amount of time that interrupts can be disabled is

;       usually restricted.

;       Reference: Programming the 68000 by Steve Williams, page 366.

;                  Published by SYBEX, Inc. 1985

wait:

cmpa.l     8(a0), a1            ; Compare location stored in extract pointer to

; location stored in simulated insert pointer.

bne.s      store_character_in_buffer

cmp.l      $4BA, d1             ; Elapsed time check.

bhi.s      wait                 ; Wait until there is room in the buffer for

; another character, or until the programmed

; wait time has expired.

moveq      #0, d0               ; In case calling source wants to know

rte                             ; that character was not dispatched.

not_bconout_call:

cmpi.w     #8, (a0)             ; Requesting output device status?

bne.s      not_bcostat_call

; NOTE: The buffer status is returned in response to the bcostat invocation,

;       not the printer status.

determine_software_buffer_status:

moveq      #-1, d0              ; Assume buffer is not full.

lea        io_buffer, a0        ; Fetch buffer structure pointer.

move.l     12(a0), a1           ; Fetch insert pointer for simulation.

addq.l     #1, a1               ; Simulate moving insert pointer to next buffer

; location.

cmpa.l     16(a0), a1           ; Compare buffer_end to simulated insert

; location.

bcs.s      not_buffer_end

movea.l    (a0), a1             ; Simulate resetting insert pointer to front of

; buffer.

not_buffer_end:

cmpa.l     8(a0), a1            ; Compare location stored in extract pointer

; to simulated insert location.

bne.s      buffer_not_full      ; Buffer not full if extract location is not

; equal to simulated insert location.

moveq      #0, d0               ; Return buffer status to calling source.

buffer_not_full:                 ; Return 0 if full, -1 if not.

rte

not_bcostat_call:

not_printer:

lea        preempted_trap_13_address, a0

movea.l    (a0), a0

jmp        (a0)                 ; JUMP TO PREEMPTED TRAP #13 HANDLER.

data

message_1:    dc.b 'Nondecimal character digit detected on parameter line.'

dc.b $D,$A,'Press Return key to terminate.',$D,$A,0

message_2:    dc.b 'Parameter line input limit is 4 decimal digits.',$D,$A

dc.b 'The program converts the input to thousands of bytes.'

dc.b  $D,$A,'Default buffer size is 32,768 bytes.',$D,$A

dc.b  'Press Return key to terminate.',$D,$A,0

align

; The variable "io_buffer" is a pointer to the print buffer's starting

; address.  The address of io_buffer also serves as the address of a

; structure that is composed of the variables io_buffer, at offset 0,

; length, at offset 4, extract_pointer (also called the buffer output pointer),

; at offset 8, and insert_pointer (also called the buffer input pointer), at

; offset 12.  In this service, the address of io_buffer provides the means

; by which the other members of the structure may be referrenced.

; Characters are placed in the buffer at the address stored in the variable

; 'insert_pointer'; they are extracted from the buffer and sent to the printer

; from the address stored in the variable 'extract_pointer'.

; Time is saved because only the address of io_buffer need be loaded into

; an address register.  The other variables in the structure are referenced

; by adding their offsets to the content of the chosen address register.

; NOTE: Even though the variable "buffer" is shown as being "stored" in

;       four pointers in the io_buffer structure, remember that IT DOES

;       NOT HAPPEN when a program is assembled in PC-relative mode.  The

;       address stored in those pointers during assembly is the ASSEMBLY

;       TIME address.  Therefore, the RUN TIME address must be explicitly

;       stored in those pointers for a PC-relative assembled program.

io_buffer:       dc.l  buffer     ; Pointer to buffer's starting address.

length:          dc.l   $8000     ; Default buffer length = 32,768 bytes.

extract_pointer: dc.l  buffer     ; Buffer character output pointer.

insert_pointer:  dc.l  buffer     ; Buffer character input pointer.

buffer_end:      dc.l  buffer     ; Last address in buffer = buffer + length.

bss

preempted_trap_13_address:  ds.l   1

ds.l  96

stack:           ds.l   1

buffer:          ds.l   0     ; Length of buffer must be added to length of

program_end:     ds.l   0     ; program to determine total size to pass to

end                          ; GEMDOS function $31.

A Linear Print Buffer

Circular print buffers are archaic, in spite of the fact

that they are still in use.  Remember that COBOL is also still in

use, as is PASCAL, BASIC, and FORTRAN.  I'm not saying that

archaic tools should not be used; I occasionally use them myself.

But I prefer faster more powerful tools when they are available

and applicable.  In that spirit, I am going to introduce you to a

different type of buffer.  At first, it will appear to be

cumbersome.  That is only because we have not yet reached the

point in the book at which desk accessories and the file selector

are among the tools available.  Trust me, I know what I'm doing.

Program 69 installs a linear buffer and a parallel interface pin

#11 interrupt handler.  Its companion, program 70, locates the

address of program 69's buffer, then reads a file and stores the

file directly in the buffer.  When the entire file has been

stored, program 70 initiates the interrupt controlled printing

process by invoking the handler installed by program 69.  The

speed differential between a printing program/circular buffer

combination and a printing program/linear buffer combination will

be quite startling.  Of course there will be quantitative

analyses shortly.

Program 69. This program installs a linear buffer and a

parallel interface pin #11 interrupt handler; it must

be executed prior to the execution of program 70.

; Program Name: PRG_7IP.S

;      Version: 1.005

; Assembly Instructions:

;      Assemble in PC-relative mode and save with a TOS and a TTP suffix.

; Program Function:

;      Linear printer buffer.  The default buffer size is 32,768 bytes.

; This program does not use a circular queue; instead, it uses a linear

; queue; therefore, no custom trap #13 handler is installed by this program;

; only an interrupt handler is required.

;      But this interrupt handler is different from the one used for the

; programs that established a circular buffer.  The data read from a file is

; placed in the handler's buffer by the GEMDOS $3F function.  PRG_7JP.TTP does

; this by locating the buffer and passing its address as a parameter to the

; the GEMDOS $3F function.  In addition, PRG_7JP.TTP stores the length of the

; file in this program's filesize variable so that the interrupt handler can

; disable itself at the end of file.

;      After the data has been transferred from the file to the buffer,

; PRG_7JP.TTP initiates the transfer through the parallel port by sending

; a NULL character through the port.  When the printer receives the NULL

; character, it will print nothing, but the falling edge of pin 11's busy

; indicative pulse will invoke the interrupt handler so that it can send the

; first character of the file.  The signal send by the printer after receiving

; that character and after receipt of each character thereafter will perpetuate

; the interrupt handler process until the end of the file has been reached.

;      As the interrupt handler transfers data from the buffer through the

; parallel port, it decrements the variable "filesize".  It stops sending

; when filesize = 0.

;      The two programs, PRG_7IP.S and PRG_7JP.S have been designed to work

; together.  This design was chosen more for its tutorial value than for its

; practical value.  The program combination can handle only one file at a

; time.  At times, it is convenient to print more than one file, and that can

; be done if a circular buffer of sufficient size is in use.

;      When a linear buffer is in use, the easiest way to permit the printing

; of multiple files is to create a file name buffer.  Actually, since it is

; not reasonable to expect all of the files to be printed to reside in the same

; directory, therefore, the buffer would be a "path" buffer.

;      The design of such a program is much easier when the file selector is

; used to gather the file paths.  Furthermore, the method obviates the need

; for two programs.  This type of program will be introduced in the chapter

; that discusses the file selector.

; Execution Instructions:

;      May be executed from the desktop with a TOS or TTP extension.  May be

; executed from an AUTO folder if it has a PRG extension.

;      If a larger buffer size is desired, the TTP version of this program can

; be used to declare a buffer size on the TTP parameter line.  Any size, up

; to the maximum available machine memory, may be specified.  When the TTP

; program is executed, type the desired size, in kilobytes, on the parameter

; line; the maximum number of digits which can be typed on the parameter line

; is 4.  Do not type spaces before the digits.  Do not type anything between

; digits.  Do not type anything after the digits, except the Return key.

; The OK button may be clicked in lieu of a Return key press.

program_start:                   ; Calculate program size and retain result.

lea        -$82(pc), a4         ; Fetch "command line" address.

lea        -$80(a4), a0         ; Fetch "basepage" address.

lea        program_end, a3      ; Fetch "end of program" address.

suba.l     a0, a3               ; Program size is in A3.

lea        stack, a7            ; Fetch address of the program stack.

process_input_parameter:         ; Calculate requested buffer length.

lea        length, a0           ; Fetch address of the length variable.

moveq      #0, d0               ; Clear counter register.

move.b     (a4)+, d0            ; Fetch parameter line character count.

beq.s      compute_size         ; Branch if no parameter--use default length.

cmpi.b     #4, d0               ; Greater than four test.

bgt        too_many_characters  ; Branch if more than four characters.

subq.b     #1, d0               ; Set up counter.

ext.w      d0                   ; Extend to match size of DBRA instruction.

moveq      #0, d2               ; Initialize accumulator.

moveq      #0, d1               ; Clear scratch register.

fetch_digit:                     ; Convert input from ASCII decimal to binary.

move.b     (a4)+, d1            ; ASCII decimal digit to D1.

sub.b      #$30, d1             ; Convert ASCII decimal digit to decimal digit.

bmi.s      not_decimal          ; Branch if digit is less than zero.

cmp.b      #9, d1               ; Greater than nine test.

bgt.s      not_decimal          ; Branch if digit is greater than nine.

multiply_accumulator_content_by_ten:

move.l     d2, d3               ; Copy accumulator in D3.

lsl.l      #3, d2               ; Shift accumulator content to multiply by 8.

add.l      d3, d2               ; Add D2's original quantity twice to

add.l      d3, d2               ; complete the multiplication by 10.

add_new_digit_to_accumulator_content:

add.l      d1, d2               ; Add decimal number in D1 to accumulator.

dbra       d0, fetch_digit      ; Loop until d0 becomes negative.

extend_to_thousands_of_bytes:    ; Length will be (input X 1000) plus

moveq      #10, d3              ; some even number of bytes that is less

lsl.l      d3, d2               ; than 1000.  Ex: if input is 32, length

move.l     d2, (a0)             ; is 32768.  Store requested buffer size.

compute_size:                    ; This will be parameter to GEMDOS $31.

adda.l     (a0), a3             ; Add buffer length to program size.

store_io_buffer_structure_components:

lea        buffer, a1           ; Fetch buffer starting address.

lea        io_buffer, a2        ; Fetch io_buffer structure base address.

move.l     a1, (a2)             ; Buffer starting address to base pointer.

move.l     a1, 8(a2)            ; Also to extract_pointer.

install_interrupt_handler_vector:

pea        interrupt_handler    ; Push interrupt handler address.

move.w     #0, -(sp)            ; Interrupt level for centronics ready.

move.w     #$D, -(sp)           ; Functon = XBIOS #13 dec = mfpint.

trap       #14

addq.l     #8, sp

relinquish_processor_control:    ; Maintain memory residency.

move.w     #0, -(sp)            ; See page 121 of Internals book.

move.l     a3, -(sp)            ; Program size (plus buffer length).

move.w     #$31, -(sp)          ; Function = ptermres = GEMDOS $31.

trap       #1

not_decimal:

pea        message_1            ; Print error message.

move.w     #9, -(sp)            ; Function = c_conws = GEMDOS $9.

trap       #1                   ; GEMDOS call

addq.l     #6, sp               ; Reset stack pointer to top of stack.

bra.s      terminate

too_many_characters:

pea        message_2            ; Print error message.

move.w     #9, -(sp)            ; Function = c_conws = GEMDOS $9.

trap       #1                   ; GEMDOS call

addq.l     #6, sp               ; Reset stack pointer to top of stack.

terminate:                       ; Wait for keypress.

move.w     #8, -(sp)            ; Function = GEMDOS $8 = cnecin.

trap       #1

addq.l     #2, sp

move.w     #0, -(sp)            ; Function = p_term_old = GEMDOS $0.

trap       #1                   ; GEMDOS call.

interrupt_handler:               ; Parallel interface interrupt handler.

movem.l    d0/a0-a1, -(sp)      ; Save content of registers used by handler.

move.l     filesize, d0         ; Fetch file size.

beq.s      clear_in_service_bit ; Exit if buffer is empty (filesize = 0).

lea        io_buffer, a0        ; Fetch address of io_buffer pointer.

movea.l    8(a0), a1            ; Fetch content of extract pointer.

subq.l     #1, d0               ; Decrement filesize.

move.l     d0, 12(a0)           ; Store new filesize value.

bne.s      print_character      ; Stop printing if filesize is 0.

move.l     (a0), 8(a0)          ; Put buffer start address in extract pointer.

bra.s      clear_in_service_bit

print_character:

move.b     (a1)+, d0            ; Fetch character.  Increment extract pointer.

move.l     a1, 8(a0)            ; Store location of next character.

lea        $FF8800, a1          ; Address of Programmable Sound Generator.

move.b     #$F, (a1)            ; Select port B (register 17) of PSG.

move.b     d0, 2(a1)            ; Write character to PSG register 17.

move.b     #$E, (a1)            ; Select port A (register 16) of PSG.

move.b     (a1), d0             ; Get current register 16 value.

strobe_low:

andi.b     #$DF, d0             ; Bit 5 to zero.

move.b     d0, 2(a1)            ; Bit 5 of port A to zero.

strobe_high:

ori.b      #$20, d0             ; Bit 5 to one.

move.b     d0, 2(a1)            ; Bit 5 of port A to one.

clear_in_service_bit:

movem.l    (sp)+, d0/a0-a1      ; Restore registers used by handler.

bclr       #0, $FFFA11          ; Clear MFP in service bit.

rte

data

message_1:    dc.b 'Nondecimal character digit detected on parameter line.'

dc.b $D,$A,'Press Return key to terminate.',$D,$A,0

message_2:    dc.b 'Parameter line input limit is 4 decimal digits.',$D,$A

dc.b 'The program converts the input to thousands of bytes.'

dc.b  $D,$A,'Default buffer size is 32,768 bytes.',$D,$A

dc.b  'Press Return key to terminate.',$D,$A,0

align

io_buffer:       dc.l  buffer     ; Pointer to buffer's starting address.

length:          dc.l   $8000     ; Default buffer length = 32,768 bytes.

extract_pointer: dc.l  buffer     ; Buffer character output pointer.

filesize:        dc.l       0     ; filesize = 0 when buffer is empty.

bss

ds.l  96

stack:           ds.l   1

buffer:          ds.l   0     ; Length of buffer must be added to length of

program_end:     ds.l   0     ; program to determine total size to pass to

end                          ; GEMDOS function $31.

Program 70. This program reads the file designated by

input on its command line into the buffer installed by

program 69, then it initiates the interrupt printing

process that is controlled by program 69.

; Program Name: PRG_7JP.S

;      Version: 1.002

; Assembly Instructions:

;      Assemble in PC-relative mode and save with a TPP extension.

; Function:

;      Reads the file designated by the parameter line input.  The address

; of the buffer passed to the GEMDOS $3F function is that of the parallel

; interface interrupt handler installed by PRG_7IP.TOS or TTP.

;      This program initiates the interrupt controlled printing process by

; sending a NULL character to the printer (A BELL character could also be

; used, if that is desirable.).

;      NOTE: Initially, I tried to send the initiating character using the

; BIOS #3 function (bconout); that method worked occasionally, but was not

; at all reliable.

; Execution Instructions:

;      PRG_7IP.TOS or TTP must install the parallel interface interrupt

; handler before this program is executed.  Execute this program from the

; desktop.  This program and the file to be printed must reside in the same

; directory.

fetch_pertinent_addresses:

lea        -$82(pc), a3         ; Fetch "command line" address.

lea        -$80(a3), a1         ; Fetch "basepage" address.

lea        program_end, a0      ; Fetch "end of program" address.

lea        stack, a7            ; Fetch stack address.

lea        dta, a5              ; Fetch dta buffer address.

calculate_program_size:

suba.l     a1, a0

return_unused_memory:

pea        (a0)                 ; Push program length.

pea        (a1)                 ; Push basepage address.

move.l     #$4A0000, -(sp)      ; Function = m_shrink = GEMDOS $4A.

trap       #1                   ; Invoke GEMDOS exception.

lea        $C(a7), sp           ; Reset stack pointer to top of stack.

set_dta:

pea        (a5)                 ; dta = address of 44 byte buffer.

move.w     #$1A, -(sp)          ; GEMDOS function = set dta.

trap       #1

addq.l     #6, sp

process_command_line:

move.b     (a3)+, d0            ; Fetch command line character count.

ext.w      d0                   ; Extend to word for next instruction.

move.b     #0, 0(a3,d0.w)       ; Store a null at end of command line input.

search_for_file:

move.w     #0, -(sp)            ; Attribute = normal access.

pea        (a3)                 ; Push address of file name.

move.w     #$4E, -(sp)          ; GEMDOS function = search first.

trap       #1

addq.l     #8, sp

tst        d0

bne.s      terminate            ; Terminate if file not found.

process_interrupt_handler_variables:

pea        fetch_interrupt_handler_address

move.w     #$26, -(sp)          ; Execute a routine in supervisor mode.

trap       #14

addq.l     #6, sp

lea        handler_address, a4  ; Fetch handler address.

movea.l    (a4), a4

adda.l     #$16C, a4            ; Add offset to variable "filesize".

move.l     $1A(a5), (a4)        ; Store file size in handler variable.

adda.l     #$188, a4            ; Add offset to buffer variable.

open_file:                       ; Function returns file handle in D0.

move.w     #0, -(a7)            ; Open as read only.

pea        (a3)                 ; Push address of file name.

move.w     #$3D, -(a7)          ; See Internals page 127.

trap       #1

addq.l     #8, a7

move.w     d0, d3               ; Store file handle in D3.

read_file:

pea        (a4)                 ; Push buffer address

move.l     $1A(a5), -(sp)       ; Number of bytes to read.

move.w     d3, -(sp)            ; File's handle number.

move.w     #$3F, -(sp)          ; GEMDOS function = read.

trap       #1

lea        $C(sp), sp           ; Reposition stack pointer.

close_file:

move.w     d3, -(sp)            ; Push file handle.

move.w     #$3E, -(sp)          ; See Internals page 128.

trap       #1

addq       #4, sp

initiate_interrupt_process:      ; Execute in supervisor mode.

pea        send_null

move.w     #$26, -(sp)

trap       #14

addq.l     #6, sp

terminate:

move.w     #0, -(sp)

trap       #1

fetch_interrupt_handler_address:

lea        handler_address, a0

move.l     $100, (a0)           ; Interrupt handler address is stored at

rts                             ; location $100.

send_null:                       ; The printer will drop the logic level of

moveq.l    #0, d0               ; pin 11 from high to low after receiving this

; character, even though nothing will be

; printed.

lea        $FF8800, a1          ; Address of Programmable Sound Generator.

move.b     #$F, (a1)            ; Select port B (register 17) of PSG.

move.b     d0, 2(a1)            ; Write character to PSG register 17.

move.b     #$E, (a1)            ; Select port A (register 16) of PSG.

move.b     (a1), d0             ; Get current register 16 value.

strobe_low:

andi.b     #$DF, d0             ; Bit 5 to zero.

move.b     d0, 2(a1)            ; Bit 5 of port A to zero.

strobe_high:

ori.b      #$20, d0             ; Bit 5 to one.

move.b     d0, 2(a1)            ; Bit 5 of port A to one.

rts

data

bss

dta:               ds.l   11

handler_address:   ds.l    1

ds.l   96     ; Program stack.

stack:             ds.l    0     ; Address of program stack.

program_end:       ds.l    0

end

How Communication Between the Programs Was Established

It is the requirement that the print buffer and the

interrupt handler be established with a LSR algorithm which

necessitates the exclusion of the printing algorithm from the LSR

program.  Later it will be possible to combine both algorithms in

one program.  Because the printing program must pass the address

of the print buffer as a parameter when invoking the GEMDOS $3F

function, a communication link must be established between the

two algorithms.  I am listing the steps that I used to establish

that communication link so that you could use a similar process

if you ever found it necessary to do so.  The addresses shown in

my steps serve only as an indication of those you might see in

your system.

1. Load PRG_7IP.S into the AssemPro editor.

2. Assemble in PC-relative mode.

3. Save with TOS and TTP extensions.

4. Go to the debugger.

5. Click on symbolic button.

6. Click on from address button.  Backspace over the $ and type the label

"interrupt_handler", sans parentheses.  Press the Return key.  Write

down address of the label = $84F8C.

7. Similarly, advance to the label "io_buffer".

8. Write addresses and contents for the following variables.  The contents

can be obtained by assigning the address of each variable a slot in the

register field.  For example: double click on A0 in the register field;

press the Esc key; type $850EC; press the Return key; note the contents

of the address in the register field.  Follow a similar procedure to

assign $850F0 to A1's slot, $850F4 to A2's slot and $85058 to A3's slot.

Variable    Address   Contents

io_buffer        $850EC    $396    ; Assembly time address of "buffer".

length           $850F0    $8000   ; Default buffer length.

extract_pointer: $850F4    $396    ; Assembly time address of "buffer".

filesize:        $850F8    0

9. Click on the from address button.  Backspace over the $ and type the

label "buffer", sans parentheses.  Press the Return key.  Write down the

label's address, but ignore the contents, which are meaningless at this

time.  Address = $85280.  Another way to obtain the buffer's address is

to assign the label "buffer" to A4's slot.  Double click on A4; press

the Esc key; type "buffer.l", sans parentheses; click on the address

button in the dialog box; press the Return key.  In the register field,

you would see: BUFFER.L  ^  $85280.

10. Click on the from address button.  Backspace over the $ and type the

label "store_io_buffer_structure_components", sans parentheses.  Press

the Return key.  Move the PC cursor to that label--press Control key and

left mouse button simultaneously.

11. Single step to the next label.  In the register field you will see the

run time address of "buffer" stored at "io_buffer" and

"extract_pointer".  Exit to the desktop.

12. Calculate required offsets to pertinent variables.

variable            address   variable   address

filesize:           $850F8    buffer:    $85280

interrupt_handler: -$84F8C    filesize: -$850F8

------               ------

offsets             $  16C               $  188

In program 70, we can easily obtain the address of the label

"interrupt_handler" because it will be stored at address $100,

the address set aside for the MFP level 0 interrupt handler.  See

pages 235-244 of the Internals book.  The reference does not

clearly state that address $100 is the relevant address, but it

can be calculated from the information given within those pages.

Once the address of "interrupt_handler" is obtained, the address

of filesize can be calculated by adding its offset, $16C, from

the address stored in location $100.  Then, the address of the

label "buffer" can be calculated by adding its offset, $188, from

"filesize" to the address calculated for filesize.

Data Transfer Rate Quantitative Analyses

I have prepared several programs that calculate data

transfer rates.  Program 71 calculates the data transfer rate for

a circular buffer by printing characters in a loop in order to

eliminate the influence of a printing algorithm on the buffer's

transfer rate.  Of course, we must assume that the character

printing loop probably influences that rate, especially since it

uses the same bios function to transmit characters to the buffer;

but, there is a limit to what we can do to limit the interaction

because the circular buffers are bound to that function via their

dependence on the trap #13 handler through which they receive

characters.  I admit that there may be esoteric design that is

not being consided, but it really doesn't matter, because it is

the linear type of buffer arrangement which offers the best

opportunity for maximum data transfer rates.  That shall be

proven before the conclusion of this chapter.

Program 71. A program that calculates a circular print buffer's

data transfer rate by printing characters in a loop.  Following

the execution instructions listed in the program's documentation.

; Program Name: PRG_7KP.S

;      Version: 1.004

; Assembly Instructions:

;      Assemble in PC-relative mode and save with a TOS extension.

; Function:

;      This program is used to determine the input data transfer rate of the

; trap #13 handler for a circular print buffer.  In order to remove the

; influence of the output data transfer rate of an algorithm that reads a file

; from a disk and sends each character to the parallel port using system

; functions, this program this program prints characters in a loop.

;      At the onset of execution the program prompts for the name of the print

; buffer that is being tested.

;      The program's output data is stored in a file bearing a name that is

; composed of that given for the print buffer and the extension DAT.  The data

; stored in that file is the number of characters printed; the name of the

; print buffer being tested and the buffer's input data transfer rate in bytes

; per second.  The name given for the print buffer can be no more than 8 valid

; ST file name characters.

;      The input data transfer rate file can be used to compare the results of

; two or more print buffers to each other.  In addition, the input data

; transfer rate of linear buffers can be compared to that of circular buffers.

; Execution Instructions:

;      1. If necessary, remove all installed print buffers by cold booting.

;      2. Turn the printer on.

;      3. If the print buffer that is to be tested is not yet installed,

;         install it now.

;      4. Execute CUSTOM.PRG to install the custom traps invoked by this

;         program.

;      5. Execute this program from the desktop.  The program's output will

;         be stored in a file that will reside in the same directory from

;         which this program is executed.

calculate_program_size:

lea        -$102(pc), a1        ; Fetch basepage start address.

lea        program_end, a0      ; Fetch program end address.

trap       #6                   ; Return unused memory to op system.

lea        stack, a7            ; Point A7 to this program's stack.

get_name_of_print_buffer:

; The first byte of the array 'buffer_name_buffer' contains the maximum

; permissible length of the input string.  After the GEMDOS $A function

; invocation, the second byte of the array will contain the number of

; characters read as input; the carriage return character will not be included

; in the string length value.  The address of the first byte of the input

; string is that of the third byte of the array.

lea        buffer_prompt, a0

bsr        print_string

lea        buffer_name_buffer, a3 ; Fetch structure's base address.

pea        (a3)                   ; Push address of the array.

move.w     #$A, -(sp)             ; GEMDOS readline function.

trap       #1

addq.l      #6, sp

store_name_for_header_use:

move.b     1(a3), d0            ; Fetch length of buffer name.

ext.w      d0                   ; Extend to word length.

addq.l     #2, a3               ; Move to name element in array.

movea.l    a3, a2               ; Prepare for suffix algorithm.

adda.w     d0, a2               ; Same preparation.

lea        buffer_name, a1      ; Fetch address of buffer_name array.

subq.w     #1, d0               ; Initialize counter.

store_character:                 ; Store name in another array for use

move.b     (a3)+, (a1)+         ; in a header.

dbra       d0, store_character

add_output_file_name_suffix:     ; Attach DAT suffix to buffer name.

move.b     #$2E, (a2)+          ; Attach a period.

move.b     #$44, (a2)+          ; Attach letter 'D'.

move.b     #$41, (a2)+          ; Attach letter 'A'.

move.b     #$54, (a2)+          ; Attach letter 'T'.

move.b     #$0, (a2)            ; Finish with a NULL.

get_start_time:

trap       #3                   ; Value of system clock returned in D0.

move.l     d0, d4               ; Store time in D4.

print_characters:

move.l     #3000, d6            ; Number of characters to print.

move.w     d6, d3               ; Initialize loop counter.

subq.w     #1, d3

move.b     #$7A, d1

reset_generator:

move.b     #$20, d5             ; Character number generator.

print_byte:

cmp.b      d1, d5

beq.s      reset_generator

addq.b     #1, d5

move.w     d5, -(sp)

move.w     #0, -(sp)

move.w     #3, -(sp)            ; See Internals page 155.

trap       #13

addq       #6, sp

dbra       d3, print_byte       ; Print 3000 characters.

transfer_rate_calculations:

trap       #3                   ; Fetch current time.

move.l     d0, d5               ; Save end time.

sub.l      d4, d0               ; Subtract start time from end time.

convert_time_to_milliseconds:    ; Multiply by 5.

move.l     d0, d2               ; Save copy to add.

asl.l      #2, d2               ; Multiply by 4.

add.l      d0, d2               ; To complete multiplication by 5.

compute_transfer_rate:

move.l     #3000, d0            ; Fetch number of characters printed.

divu       d2, d0               ; Divide size by time in milliseconds.

copy_to_d1:                      ; Will extract remainder from D1.

move.l     d0, d1               ; Quotient is in lower word of D0.

mulu       #1000, d0            ; Multiply quotient by 1000.

clr.w      d1                   ; Get rid of quotient.

swap       d1                   ; Remainder is now in lower word of D1.

mulu       #1000, d1            ; Multiply remainder by 1000.

divu       d2, d1               ; Divide remainder by time in milliseconds.

swap       d1                   ; Put remainder in lower word.

clr.w      d1                   ; Get rid of remainder.

swap       d1                   ; Get quotient back in lower word.

add.l      d0, d1               ; Transfer rate is now in D1.

; Note: The order in which things are done in the calculation algorithm

;       is critical.  The remainder obtained from the first division must

;       be manipulated just right.  One other point is that the quotient

;       will be zero whenever the time in milliseconds is a value greater

;       than the size of the file.

;       I have verified the accuracy of the above algorithm, but I have

;       decided to print the number of 5 milliseconds clock ticks obtained

;       for both the start and end times.  This will permit the transfer

;       rate to be calculated manually.  Remember that those clock ticks

;       must be multiplied by 5 to convert them to milliseconds.  And, of

;       course, the number of milliseconds must be divided by 1000 to convert

;       them to seonds; or one can divide the file size by the number of

;       milliseconds and multiply the quotient by 1000.

; The equation being used in this algorithm is:

; transfer rate (bytes/second) = [file size (bytes)/transfer time (msec)]

;                                X 1000 (msec/second).

trap       #4                   ; Convert to ASCII decimal.

lea        transfer_rate, a1    ; Fetch address of transfer_rate.

move.l     a0, (a1)             ; Store address of decimal string.

; NOTE: The variable 'transfer_rate' now contains the address of the

;       ASCII decimal string that is the transfer rate.  Trap #4 can't

;       be invoked until that value is used by the program, otherwise

;       the value will be corrupted.

create_output_file:

move.w     #0, -(sp)            ; File attribute = read/write.

pea        output_file          ; Push address of output file's name.

move.w     #$3C, -(sp)          ; Function = f_create = GEMDOS $3C.

trap       #1                   ; File handle is returned in D0.

addq.l     #8, sp

lea        output_file_handle, a1

move.w     d0, (a1)

redirect_output_bound_for_screen:

; NOTE: If the output file's handle is exchanged with the video screen's

;       handle, then the printline function = GEMDOS $9 can be used to

;       write to the file.

redirect_output:                 ; Exchange file handle with screen's handle.

move.w     d0, -(sp)            ; This is the disk file's handle.

move.w     #1, -(sp)            ; This is the video screen's handle.

move.w     #$46, -(sp)          ; Function = f_force = GEMDOS $46.

trap       #1

addq.l     #6, sp

print_data_report:

lea        heading(pc), a0      ; Print data file heading.

bsr        print_string

lea        buffer_name(pc), a0  ; Name of buffer for which transfer rate is

bsr        print_string         ; being reported.

lea        header_1(pc), a0     ; Transfer Rate header.

bsr        print_string

move.l     transfer_rate, a0    ; Transfer Rate.  Fetch from trap #4

bsr        print_string         ; location.  Quick before it goes away.

lea        header_2(pc), a0     ; Transfer Rate Units.

bsr        print_string

lea        header_3(pc), a0     ; Characters printed header.

bsr        print_string

move.l     d6, d1               ; Fetch number of bytes printed from D6.

trap       #4                   ; Convert to ASCII decimal.

bsr        print_string

lea        header_7(pc), a0     ; End Time header.

bsr        print_string

move.l     d5, d1               ; Fetch end time.

trap       #4                   ; Convert to ASCII decimal.

bsr        print_string

lea        header_8(pc), a0     ; Time units = 5-millisecond ticks.

bsr        print_string

lea        header_6(pc), a0     ; Start Time header.

bsr        print_string

move.l     d4, d1               ; Fetch start time.

trap       #4                   ; Convert to ASCII decimal.

bsr        print_string

lea        header_8(pc), a0     ; Time units = 5-millisecond ticks.

bsr        print_string

close_output_file:

move.w     output_file_handle, -(sp)

move.w     #$3E, -(sp)          ; Function = GEMDOS $3E = f_close.

trap       #1

addq.l     #4, sp

terminate:

move.w     #0, -(sp)

trap       #1

print_string:

pea        (a0)

move.w     #9, -(sp)

trap       #1

addq.l     #6, sp

rts

data

heading:      dc.b $D,$A,'PRG_7KP.TOS Execution Results',$D,$A,$D,$A

dc.b '  Print Buffer:        ',0

header_1:     dc.b $D,$A,'  Transfer Rate:      ',0

header_2:     dc.b ' bytes/second',$D,$A,$D,$A,0

header_3:     dc.b '  Characters Printed: ',0

header_7:     dc.b $D,$A,$D,$A,'  End Time:      ',0

header_6:     dc.b '  Start Time:    ',0

header_8:     dc.b ' 5-millisecond ticks',$A,$D,0

buffer_prompt:

dc.b $D,$A,$D,$A

dc.b 'Data produced by this program will be stored in a file with',$D,$A

dc.b 'name composed of the name of the print buffer and a DAT suffix.',$D,$A

dc.b $D,$A,'Name of print buffer (8 characters max, no suffix): ',0

buffer_name_buffer:

dc.b  10  ; Maximum length of input string = 10 characters.

dc.b   0  ; String's length will be stored here.

output_file:        ds.b  14  ; String will be stored here.

buffer_name:        ds.b  10  ; Buffer name for heading.

align

buffer:             dc.w  $0  ; Character buffer.

bss

output_file_handle: ds.w   1

transfer_rate:      ds.l   1

ds.l  96  ; Program stack.

stack:              ds.l   0  ; Address of program stack.

program_end:        ds.l   0

end

PRG_7KP.TOS Execution Results

Print Buffer:        PRG_7HP

Transfer Rate:       13636 bytes/second

Characters Printed:  3000

End Time:       1447716 5-millisecond ticks

Start Time:     1447672 5-millisecond ticks

Program 72 calculates the data transfer rate for a

particular printing program/circular print buffer combination.

This program prompts for the name of a file to be printed and the

name of the buffer being tested.  The data transfer rate will

vary somewhat depending on the directory path from which the file

is read, therefore, that path is included in the program's output

data.  If the program is executed from a floppy disk, the total

execution time will be much longer than the data transfer time

because the output file must also be stored in the directory from

which the file is read and from which program 72 is executed.

Program 72. Calculates the data transfer rate for a particular

printing program/circular buffer combination.  Follow the

execution instructions listed in the program's documentation.

; Program Name: PRG_7LP.S

;      Version: 1.004

; Assembly Instructions:

;      Assemble in PC-relative mode and save with a TOS extension.

; Function:

;      This program is used to determine a the data transfer rate of a printing

; program/print buffer combination.  The influence of the printing program's

; output data transfer rate is inherent for print buffers designed to intercept

; operating system trap #13 invocations.

;      The program prompts for the name of a file to be printed; then it

; prompts for the name of the print buffer that is being tested.  The

; program's output data is stored in a file bearing a name that is composed of

; that given for the print buffer and the extension DAT.  The data stored in

; that file is the name of the printed file and its length; the name of the

; print buffer and the printing program/print buffer combination's data

; transfer rate.  The name given for the print buffer can be no more than 8

; valid ST file name characters.

;      The data transfer rate file can be used to compare the results of two

; or more printing program/print buffer combinations to each other.

; Execution Instructions:

;      1. If necessary, remove all installed print buffers by cold booting.

;      2. Turn the printer on.

;      3. If the print buffer that is to be involved is a software buffer,

;         execute the program that installs the print buffer.

;      4. Execute CUSTOM.PRG to install the custom traps invoked by this

;         program.

;      5. Execute this program from the desktop.  The file that is to be

;         printed and this program must reside in the same directory.

calculate_program_size:

lea        -$102(pc), a1        ; Fetch basepage start address.

lea        program_end, a0      ; Fetch program end address.

trap       #6                   ; Return unused memory to op system.

lea        stack, a7            ; Point A7 to this program's stack.

get_name_of_file:                ; Request name of file to be printed.

lea        file_prompt, a0

bsr        print_string

; The first byte of the array 'file_name_buffer' contains the maximum

; permissible length of the input string.  After the GEMDOS $A function

; invocation, the second byte of the array will contain the number of

; characters read as input; the carriage return character will not be included

; in the string length value.  The address of the first byte of the input

; string is that of the third byte of the array.

lea        file_name_buffer, a3 ; Fetch structure's base address.

pea        (a3)                 ; Push address of the array.

move.w     #$A, -(sp)           ; Function = GEMDOS $A = readline.

trap       #1

addq.l     #6, sp

terminate_file_name_with_null:   ; So that name can be printed as a string.

move.b     1(a3), d0            ; Fetch length of file name.

ext.w      d0                   ; Extend to word length.

move.b     #0, 2(a3,d0.w)       ; Store NULL at end of string.

get_name_of_print_buffer:

lea        buffer_prompt, a0

bsr        print_string

lea        buffer_name_buffer, a3 ; Fetch structure's base address.

pea        (a3)                   ; Push address of the array.

move.w     #$A, -(sp)             ; GEMDOS readline function.

trap       #1

addq.l      #6, sp

store_name_for_header_use:

move.b     1(a3), d0            ; Fetch length of buffer name.

ext.w      d0                   ; Extend to word length.

addq.l     #2, a3               ; Move to name element in array.

movea.l    a3, a2               ; Prepare for suffix algorithm.

adda.w     d0, a2               ; Same preparation.

lea        buffer_name, a1      ; Fetch address of buffer_name array.

subq.w     #1, d0               ; Initialize counter.

store_character:                 ; Store name in another array for use

move.b     (a3)+, (a1)+         ; in a header.

dbra       d0, store_character

add_output_file_name_suffix:     ; Attach DAT suffix to buffer name.

move.b     #$2E, (a2)+          ; Attach a period.

move.b     #$44, (a2)+          ; Attach letter 'D'.

move.b     #$41, (a2)+          ; Attach letter 'A'.

move.b     #$54, (a2)+          ; Attach letter 'T'.

move.b     #$0, (a2)            ; Finish with a NULL.

get_drive:

move.w     #$19, -(sp)          ; See Internals page 116, but note the

trap       #1                   ; errors there.

addq.l     #2, sp               ; Note first error at this instruction.

add.w      #$41, d0             ; Note 2nd error at this instruction.

lea        path, a6             ; Fetch address of path array.

move.b     d0, (a6)+            ; Store drive letter in path array.

move.b     #$3A, (a6)+          ; Store a colon in path array.

get_directory_path:

move.w     #0, -(sp)

pea        (a6)                 ; Push address of next path array element.

move.w     #$47, -(sp)          ; See Internals page 135.

trap       #1

addq.l     #8, sp

print_the_file:

pea        dta                  ; dta = address of 44 byte buffer.

move.w     #$1A, -(sp)          ; GEMDOS function = set dta.

trap       #1

addq.l     #6, sp

search_for_file:

move.w     #0, -(sp)            ; Attribute = normal access.

pea        file_name            ; Push address of file name.

move.w     #$4E, -(sp)          ; GEMDOS function = search first.

trap       #1

addq.l     #8, sp

tst        d0

bne        terminate            ; Exit if file not found.

lea        dta, a0

move.l     $1A(a0), d6

lea        buffer, a3

open_file:                       ; Function returns file handle in D0.

move.w     #0, -(a7)            ; Open as read only.

pea        file_name            ; Push address of file name.

move.w     #$3D, -(a7)          ; See Internals page 127.

trap       #1

addq.l     #8, a7

move.w     d0, d3               ; Store file handle in D3.

trap       #3                   ; Value of system clock returned in D0.

move.l     d0, d4               ; Store time in D4.

read_file:                       ; Function returns 1 in D0 unless end of file

; is reached or a read error occurs.

pea        (a3)                 ; Push address of character buffer.

move.l     #1, -(a7)            ; Number of bytes to read from the file.

; GEMDOS $3F's buffer.

move.w     d3, -(a7)            ; Push file handle.

move.w     #$3F, -(a7)          ; See Internals page 129.

; Note: The system will not disturb this stack setup, therefore, it must be

;       initialized only once.  Thereafter, a branch need be taken only to the

;       following instruction.

fetch_byte:

trap       #1

tst.w      d0                   ; When NULL at end of file is read,

ble.s      end_of_file          ; D0 will be 0.

print_byte:

move.b     (a3), d0             ; Must read buffer one byte each time.

move.w     d0, -(sp)            ; But must push a word here.

move.w     #0, -(sp)

move.w     #3, -(sp)            ; See Internals page 155.

trap       #13

addq       #6, sp

bra.s      fetch_byte

end_of_file:

lea        $C(sp), sp

close_file:

move.w     d3, -(sp)            ; Push handle for the file that was read.

move.w     #$3E, -(sp)          ; See Internals page 128.

trap       #1

addq       #4, sp

transfer_rate_calculations:

trap       #3                   ; Fetch current time.

move.l     d0, d5               ; Store end time for data file.

sub.l      d4, d0               ; Subtract start time from end time.

convert_time_to_milliseconds:    ; Multiply by 5.

move.l     d0, d2               ; Save copy to add.

asl.l      #2, d2               ; Multiply by 4.

add.l      d0, d2               ; To complete multiplication by 5.

compute_transfer_rate:

move.l     d6, d0               ; Fetch file size from D6.

divu       d2, d0               ; Divide size by time in milliseconds.

copy_to_d1:                      ; Will extract remainder from D1.

move.l     d0, d1               ; Quotient is in lower word of D0.

mulu       #1000, d0            ; Multiply quotient by 1000.

clr.w      d1                   ; Get rid of quotient.

swap       d1                   ; Remainder is now in lower word of D1.

mulu       #1000, d1            ; Multiply remainder by 1000.

divu       d2, d1               ; Divide remainder by time in milliseconds.

swap       d1                   ; Put remainder in lower word.

clr.w      d1                   ; Get rid of remainder.

swap       d1                   ; Get quotient back in lower word.

add.l      d0, d1               ; Transfer rate is now in D1.

; Note: The order in which things are done in the calculation algorithm

;       is critical.  The remainder obtained from the first division must

;       be manipulated just right.  One other point is that the quotient

;       will be zero whenever the time in milliseconds is a value greater

;       than the size of the file.

;       I have verified the accuracy of the above algorithm, but I have

;       decided to print the number of 5 milliseconds clock ticks obtained

;       for both the start and end times.  This will permit the transfer

;       rate to be calculated manually.  Remember that those clock ticks

;       must be multiplied by 5 to convert them to milliseconds.  And, of

;       course, the number of milliseconds must be divided by 1000 to convert

;       them to seonds; or one can divide the file size by the number of

;       milliseconds and multiply the quotient by 1000.

; The equation being used in this algorithm is:

; transfer rate (bytes/second) = [file size (bytes)/transfer time (msec)]

;                                X 1000 (msec/second).

trap       #4                   ; Convert to ASCII decimal.

lea        transfer_rate, a1

move.l     a0, (a1)             ; Store address of decimal string.

; NOTE: The variable 'transfer_rate' now contains the address of the

;       ASCII decimal string that is the transfer rate.  Trap #4 can't

;       be invoked until that value is used by the program, otherwise

;       the value will be corrupted.

create_output_file:

move.w     #0, -(sp)            ; File attribute = read/write.

pea        output_file          ; Push address of output file's name.

move.w     #$3C, -(sp)          ; Function = f_create = GEMDOS $3C.

trap       #1                   ; File handle is returned in D0.

addq.l     #8, sp

lea        output_file_handle, a0

move.w     d0, (a0)

redirect_output_bound_for_screen:

; NOTE: If the output file's handle is exchanged with the video screen's

;       handle, then the printline function = GEMDOS $9 can be used to

;       write to the file.

redirect_output:                 ; Exchange file handle with screen's handle.

move.w     d0, -(sp)            ; This is the disk file's handle.

move.w     #1, -(sp)            ; This is the video screen's handle.

move.w     #$46, -(sp)          ; Function = f_force = GEMDOS $46.

trap       #1

addq.l     #6, sp

print_data_report:

lea        heading, a0          ; Print data file heading.

bsr        print_string

lea        buffer_name, a0      ; Name of buffer for which transfer rate is

bsr        print_string         ; being reported.

lea        header_1, a0         ; Transfer Rate header.

bsr        print_string

move.l     transfer_rate, a0    ; Transfer Rate.  Fetch from trap #4

bsr        print_string         ; location.  Quick before it goes away.

lea        header_2, a0         ; Transfer Rate Units.

bsr        print_string

lea        header_3, a0         ; File name header.

bsr        print_string

lea        file_name, a0        ; File name.

bsr        print_string

lea        header_4, a0         ; File length header.

bsr        print_string

move.l     d6, d1               ; Fetch file size from D6.

trap       #4                   ; Convert to ASCII decimal.

bsr        print_string

lea        header_5, a0         ; File size units.

bsr        print_string

lea        header_9, a0         ; Directory path.

bsr        print_string

lea        path, a0

bsr        print_string

lea        header_7, a0         ; End Time header.

bsr        print_string

move.l     d5, d1               ; Fetch end time.

trap       #4                   ; Convert to ASCII decimal.

bsr        print_string

lea        header_8, a0         ; Time units = 5-millisecond ticks.

bsr        print_string

lea        header_6, a0         ; Start Time header.

bsr        print_string

move.l     d4, d1               ; Fetch start time.

trap       #4                   ; Convert to ASCII decimal.

bsr        print_string

lea        header_8, a0         ; Time units = 5-millisecond ticks.

bsr        print_string

close_output_file:

move.w     output_file_handle, -(sp)

move.w     #$3E, -(sp)          ; Function = GEMDOS $3E = f_close.

trap       #1

addq.l     #4, sp

terminate:

move.w     #0, -(sp)

trap       #1

print_string:

pea        (a0)

move.w     #9, -(sp)

trap       #1

addq.l     #6, sp

rts

data

heading:      dc.b $D,$A,'PRG_7LP.TOS Execution Results',$D,$A,$D,$A

dc.b '  Print Buffer:   ',0

header_1:     dc.b $D,$A,'  Transfer Rate: ',0

header_2:     dc.b ' bytes/second',$D,$A,0

header_3:     dc.b '  File Printed:   ',0

header_4:     dc.b $D,$A,'  File Length:  ',0

header_5:     dc.b ' bytes',$D,$A,0

header_9      dc.b $D,$A,'  Path: ',0

header_7:     dc.b $D,$A,$D,$A,'  End Time:      ',0

header_6:     dc.b '  Start Time:    ',0

header_8:     dc.b ' 5-millisecond ticks',$A,$D,0

file_prompt:  dc.b $D,$A,'Name of file to be printed (include suffix): ',0

buffer_prompt:

dc.b $D,$A,$D,$A

dc.b 'Data produced by this program will be stored in a file with',$D,$A

dc.b 'name composed of the name of the print buffer and a DAT suffix.',$D,$A

dc.b $D,$A,'Name of print buffer (8 characters max, no suffix): ',0

file_name_buffer:

dc.b  14  ; Maximum length of input string = 14 characters.

dc.b   0  ; String's length will be stored here.

file_name:           ds.b  14  ; String will be stored here.

buffer_name_buffer:

dc.b  10  ; Maximum length of input string = 10 characters.

dc.b   0  ; String's length will be stored here.

output_file:         ds.b  14  ; String will be stored here.

buffer_name:         ds.b  10  ; Buffer name for heading.

align

buffer:              dc.w  $0  ; Character buffer.

bss

output_file_handle:  ds.w   1

dta:                 ds.l  11

transfer_rate:       ds.l   1

path:                ds.b 120

ds.l  96  ; Program stack.

stack:               ds.l   0  ; Address of program stack.

program_end:         ds.l   0

end

PRG_7LP.TOS Execution Results

Print Buffer:   PRG_7HP

Transfer Rate:  995 bytes/second

File Printed:   PRG_7LP.S

File Length:   15107 bytes

Path: P:        Note: P was a ram disk.

End Time:       670922 5-millisecond ticks

Start Time:     667886 5-millisecond ticks

PRG_7LP.TOS Execution Results

Print Buffer:   PRG_7HP

Transfer Rate:  797 bytes/second

File Printed:   PRG_7LP.S

File Length:   15107 bytes

Path: A:\PRG_7

End Time:       732462 5-millisecond ticks

Start Time:     728672 5-millisecond ticks

Program 73 has been designed to measure the data transfer

rate through the ST's parallel interface port to a device

connected to that port.  For the test, I used my Supra

Corporation MicroStuffer printer buffer.  Because I believe that

this external buffer can accept data as fast as the ST's port can

move it, I consider the test to reflect the capacity of the ST's

circuitry.  And even though I can't prove that fact,

specifically, I can state that there is evidence in at least one

reference of a limit to the port's data transfer rate.  On page

3-26 of the Peel book, the ST port's rate is said to be 4000

bytes per second, typically.  The point is this: no matter the

quality of an external buffer connected to the ST's port, the

data transfer rate to that buffer will never approach the speed

at which we are able to transfer data to a linear buffer within

ST ram.  Program 74 will illustrate that clearly enough.  Please

especially note the data transfer rate achieved when the file

being printed is read from a ram disk.

Program 73.  This program will measure the data transfer rate of

the ST's parallel port when an external buffer is connected to

that port.

; Program Name: PRG_7MP.S

;      Version: 1.004

; Assembly Instructions:

;      Assemble in PC-relative mode and save with a TOS extension.

; Function:

;      A program to measure the transfer rate of data through the ST's

; parallel interface port to a device connected to that port.  In this

; particular instance the device connected to the port may be the printer,

; the printer's internal buffer or an external print buffer such as my Supra

; Corporation MicroStuffer printer buffer.  Unfortunately, MicroStuffer's

; input data transfer rate is not specified in its operator's manual;

; therefore, I can't compare the results obtained with this program to its

; hardware specification.

;      However, in appendix G of my Star NX-10 User's Manual, the data

; transfer rate for the printer's parallel interface is given: it is listed

; as 1,000 to 6,000 characters per second.

;      In addition, there is some information given on page 3-26 of the

; Peel book.  The data transfer rate for the ST's parallel port is mentioned

; there: it is said to be 4000 bytes per second, typically.

;      It is reasonable to assume that, regardless of any faster data transfer

; rate that may be possible with the MicroStuffer printer buffer, we are not

; going to be able to send data to it faster than the maximum data transfer

; rate of the ST's parallel interface port.

;      Still, in effect, since there are two devices involved in the

; experiment, and because there is no way to isolate the influence of each

; on the outcome of the experiment, the validity of the results is relevant

; only as they are applied to the device combination.

;

;      The program does not use system functions to print; instead it prints

; directly to the port.  Furthermore, this program does not read and print a

; file.  Instead, it prints characters in a loop, thereby eliminating the

; influence of a file reading algorithm.

;      The purpose of this experiment is to obtain information so that the

; data transfer rate obtained with an external buffer connected to the

; parallel interface port can be compared to that which is obtained with

; buffers created within the ST by software.  I assume that all print buffers

; involved in the comparison permit interrupt controlled printing (background

; printing), thereby effecting a multi-tasking environment.

;      NOTE: THE PRINTER DOES NOT HAVE TO BE ON WHEN THE EXTERNAL BUFFER

;            IS CONNECTED.  CAN SAVE PAPER THAT WAY.  JUST CLEAR THE

;            EXTERNAL BUFFER AFTER A TEST RUN.

;      This program prints characters in a loop, so it does not prompt for

; the name of a file to be printed, but it does prompt for the name of the

; print buffer that is being tested.  The program's output data is stored in

; a file bearing a name that is composed of that given for the print buffer

; and the extension DAT.  The data stored in that file is the number of

; characters printed; and the name of the print buffer and its input data

; transfer rate in bytes per second.  The name given for the print buffer can

; be no more than 8 valid ST file name characters.

; Execution Instructions:

;      1. If necessary, remove all installed print buffers by cold booting.

;      2. Turn the printer on.

;      3. Turn the external buffer on.

;      4. Execute CUSTOM.PRG to install the custom traps invoked by this

;         program.

;      5. Execute this program from the desktop.  The program's output will

;         be stored in a file that will reside in the same directory from

;         which this program is executed.

calculate_program_size:

lea        -$102(pc), a1       ; Fetch basepage start address.

lea        program_end, a0     ; Fetch program end address.

trap       #6                  ; Return unused memory to op system.

lea        stack, a7           ; Point A7 to this program's stack.

trap       #0                  ; Enter supervisor mode.

get_name_of_print_buffer:

; The first byte of the array 'buffer_name_buffer' contains the maximum

; permissible length of the input string.  After the GEMDOS $A function

; invocation, the second byte of the array will contain the number of

; characters read as input; the carriage return character will not be included

; in the string length value.  The address of the first byte of the input

; string is that of the third byte of the array.

lea        buffer_prompt, a0

bsr        print_string

lea        buffer_name_buffer, a3 ; Fetch structure's base address.

pea        (a3)                   ; Push address of the array.

move.w     #$A, -(sp)             ; GEMDOS readline function.

trap       #1

addq.l      #6, sp

store_name_for_header_use:

move.b     1(a3), d0           ; Fetch length of buffer name.

ext.w      d0                  ; Extend to word length.

addq.l     #2, a3              ; Move to name element in array.

movea.l    a3, a2              ; Prepare for suffix algorithm.

adda.w     d0, a2              ; Same preparation.

lea        buffer_name, a1     ; Fetch address of buffer_name array.

subq.w     #1, d0              ; Initialize counter.

store_character:                ; Store name in another array for use

move.b     (a3)+, (a1)+        ; in a header.

dbra       d0, store_character

add_output_file_name_suffix:    ; Attach DAT suffix to buffer name.

move.b     #$2E, (a2)+         ; Attach a period.

move.b     #$44, (a2)+         ; Attach letter 'D'.

move.b     #$41, (a2)+         ; Attach letter 'A'.

move.b     #$54, (a2)+         ; Attach letter 'T'.

move.b     #$0, (a2)           ; Finish with a NULL.

get_start_time:

trap       #3                  ; Value of system clock returned in D0.

move.l     d0, d4              ; Store time in D4.

print_characters:

move.l     #3000, d6             ; Number of characters to print.

move.w     d6, d3                ; Initialize loop counter.

subq.w     #1, d3

move.b     #$7A, d1

reset_generator:

move.b     #$20, d5             ; Character number generator.

print_byte:

cmp.b      d1, d5

beq.s      reset_generator

addq.b     #1, d5

printer_ready_test:

btst       #0, $FFFA01         ; Check logic level of pin 11.

bne.s      printer_ready_test  ; Loop until printer is ready to receive.

lea        $FF8800, a2         ; Fetch address of PSG data bus.

move.b     #$F, (a2)           ; Latch address of port B.

move.b     d5, 2(a2)           ; Write character to port B.

move.b     #$E, (a2)           ; Latch address of port A.

move.b     (a2), d0            ; Read content of port A.

andi.b     #$DF, d0            ; Reset bit 5 of d1 to zero.

move.b     d0, 2(a2)           ; Reset bit 5 of port A. Strobe low.

ori.b      #$20, d0            ; Set bit 5 of d1.

move.b     d0, 2(a2)           ; Set bit 5 of port A. Strobe high.

dbra       d3, print_byte      ; Print 3000 characters.

transfer_rate_calculations:

trap       #3                  ; Fetch current time.

move.l     d0, d5              ; Save end time.

sub.l      d4, d0              ; Subtract start time from end time.

convert_time_to_milliseconds:   ; Multiply by 5.

move.l     d0, d2              ; Save copy to add.

asl.l      #2, d2              ; Multiply by 4.

add.l      d0, d2              ; To complete multiplication by 5.

compute_transfer_rate:

move.l     #3000, d0           ; Fetch number of characters printed.

divu       d2, d0               ; Divide size by time in milliseconds.

copy_to_d1:                      ; Will extract remainder from D1.

move.l     d0, d1               ; Quotient is in lower word of D0.

mulu       #1000, d0            ; Multiply quotient by 1000.

clr.w      d1                   ; Get rid of quotient.

swap       d1                   ; Remainder is now in lower word of D1.

mulu       #1000, d1            ; Multiply remainder by 1000.

divu       d2, d1               ; Divide remainder by time in milliseconds.

swap       d1                   ; Put remainder in lower word.

clr.w      d1                   ; Get rid of remainder.

swap       d1                   ; Get quotient back in lower word.

add.l      d0, d1               ; Transfer rate is now in D1.

; Note: The order in which things are done in the calculation algorithm

;       is critical.  The remainder obtained from the first division must

;       be manipulated just right.  One other point is that the quotient

;       will be zero whenever the time in milliseconds is a value greater

;       than the size of the file.

;       I have verified the accuracy of the above algorithm, but I have

;       decided to print the number of 5 milliseconds clock ticks obtained

;       for both the start and end times.  This will permit the transfer

;       rate to be calculated manually.  Remember that those clock ticks

;       must be multiplied by 5 to convert them to milliseconds.  And, of

;       course, the number of milliseconds must be divided by 1000 to convert

;       them to seonds; or one can divide the file size by the number of

;       milliseconds and multiply the quotient by 1000.

; The equation being used in this algorithm is:

; transfer rate (bytes/second) = [file size (bytes)/transfer time (msec)]

;                                X 1000 (msec/second).

trap       #4                  ; Convert to ASCII decimal.

lea        transfer_rate, a1   ; Fetch address of transfer_rate.

move.l     a0, (a1)            ; Store address of decimal string.

; NOTE: The variable 'transfer_rate' now contains the address of the

;       ASCII decimal string that is the transfer rate.  Trap #4 can't

;       be invoked until that value is used by the program, otherwise

;       the value will be corrupted.

create_output_file:

move.w     #0, -(sp)           ; File attribute = read/write.

pea        output_file         ; Push address of output file's name.

move.w     #$3C, -(sp)         ; Function = f_create = GEMDOS $3C.

trap       #1                  ; File handle is returned in D0.

addq.l     #8, sp

lea        output_file_handle, a1

move.w     d0, (a1)

redirect_output_bound_for_screen:

; NOTE: If the output file's handle is exchanged with the video screen's

;       handle, then the printline function = GEMDOS $9 can be used to

;       write to the file.

redirect_output:                ; Exchange file handle with screen's handle.

move.w     d0, -(sp)           ; This is the disk file's handle.

move.w     #1, -(sp)           ; This is the video screen's handle.

move.w     #$46, -(sp)         ; Function = f_force = GEMDOS $46.

trap       #1

addq.l     #6, sp

print_data_report:

lea        heading(pc), a0      ; Print data file heading.

bsr        print_string

lea        buffer_name(pc), a0  ; Name of buffer for which transfer rate is

bsr        print_string         ; being reported.

lea        header_1(pc), a0     ; Transfer Rate header.

bsr        print_string

move.l     transfer_rate, a0    ; Transfer Rate.  Fetch from trap #4

bsr        print_string         ; location.  Quick before it goes away.

lea        header_2(pc), a0     ; Transfer Rate Units.

bsr        print_string

lea        header_3(pc), a0     ; Characters printed header.

bsr        print_string

move.l     d6, d1               ; Fetch number of bytes printed from D6.

trap       #4                   ; Convert to ASCII decimal.

bsr        print_string

lea        header_7(pc), a0     ; End Time header.

bsr        print_string

move.l     d5, d1               ; Fetch end time.

trap       #4                   ; Convert to ASCII decimal.

bsr        print_string

lea        header_8(pc), a0     ; Time units = 5-millisecond ticks.

bsr        print_string

lea        header_6(pc), a0     ; Start Time header.

bsr        print_string

move.l     d4, d1               ; Fetch start time.

trap       #4                   ; Convert to ASCII decimal.

bsr        print_string

lea        header_8(pc), a0     ; Time units = 5-millisecond ticks.

bsr        print_string

close_output_file:

move.w     output_file_handle, -(sp)

move.w     #$3E, -(sp)         ; Function = GEMDOS $3E = f_close.

trap       #1

addq.l     #4, sp

terminate:

andi.w     #$DFFF, SR           ; Return to User Mode.

move.w     #0, -(sp)

trap       #1

print_string:

pea        (a0)

move.w     #9, -(sp)

trap       #1

addq.l     #6, sp

rts

data

heading:      dc.b $D,$A,'PRG_7MP.TOS Execution Results',$D,$A,$D,$A

dc.b '  Print Buffer:        ',0

header_1:     dc.b $D,$A,'  Transfer Rate:      ',0

header_2:     dc.b ' bytes/second',$D,$A,$D,$A,0

header_3:     dc.b '  Characters Printed: ',0

header_7:     dc.b $D,$A,$D,$A,'  End Time:      ',0

header_6:     dc.b '  Start Time:    ',0

header_8:     dc.b ' 5-millisecond ticks',$A,$D,0

buffer_prompt:

dc.b $D,$A,$D,$A

dc.b 'Data produced by this program will be stored in a file with',$D,$A

dc.b 'name composed of the name of the print buffer and a DAT suffix.',$D,$A

dc.b $D,$A,'Name of print buffer (8 characters max, no suffix): ',0

buffer_name_buffer:

dc.b  10  ; Maximum length of input string = 10 characters.

dc.b   0  ; String's length will be stored here.

output_file:         ds.b  14  ; String will be stored here.

buffer_name:         ds.b  10  ; Buffer name for heading.

align

buffer:              dc.w  $0  ; Character buffer.

bss

output_file_handle:  ds.w   1

transfer_rate:       ds.l   1  ; Pointer to ASCII decimal string.

ds.l  96  ; Program stack.

stack:               ds.l   0  ; Address of program stack.

program_end:         ds.l   0

end

PRG_7NP.TOS Execution Results

Print Buffer:        EXTERNAL

Transfer Rate:       6451 bytes/second

Characters Printed:  3000

End Time:       21991 5-millisecond ticks

Start Time:     21898 5-millisecond ticks

Program 74. Measures the data transfer rate of the linear buffer.

The printing program has much less effect on the data transfer

rate of the printing program/linear buffer combination than that

experienced with printing program/circular buffer combinations.

Program PRG_7IP.TOS or TTP must be executed prior to the

execution of this program.

; Program Name: PRG_7NP.S

;      Version: 1.002

; Assembly Instructions:

;      Assemble in PC-relative mode and save with a TPP extension.

; Function:

;      Reads the file designated by the parameter line input.  The address

; of the buffer passed to the GEMDOS $3F function is that of the parallel

; interface interrupt handler installed by PRG_7IP.TOS or TTP.  This program

; is a version of PRG_7JP.  This version measures the data transfer rate for

; the PRG_7IP/PRG_7JP combination.  The name of this program's output file is

; PRG_7IP.DAT.

;      This program initiates the interrupt controlled printing process by

; sending a NULL character to the printer.

; Execution Instructions:

;      PRG_7IP.TOS or TTP must install the parallel interface interrupt

; handler before this program is executed.  Execute this program from the

; desktop.  This program and the file to be printed must reside in the same

; directory.

fetch_pertinent_addresses:

lea        -$82(pc), a3         ; Fetch "command line" address.

lea        -$80(a3), a1         ; Fetch "basepage" address.

lea        program_end, a0      ; Fetch "end of program" address.

lea        stack, a7            ; Fetch stack address.

lea        dta, a5              ; Fetch dta buffer address.

calculate_program_size:

suba.l     a1, a0

return_unused_memory:

pea        (a0)                 ; Push program length.

pea        (a1)                 ; Push basepage address.

move.l     #$4A0000, -(sp)      ; Function = m_shrink = GEMDOS $4A.

trap       #1                   ; Invoke GEMDOS exception.

lea        $C(a7), sp           ; Reset stack pointer to top of stack.

set_dta:

pea        (a5)                 ; dta = address of 44 byte buffer.

move.w     #$1A, -(sp)          ; GEMDOS function = set dta.

trap       #1

addq.l     #6, sp

get_drive:

move.w     #$19, -(sp)          ; See Internals page 116, but note the

trap       #1                   ; errors there.

addq.l     #2, sp               ; Note first error at this instruction.

add.w      #$41, d0             ; Note 2nd error at this instruction.

lea        path, a6             ; Fetch address of path array.

move.b     d0, (a6)+            ; Store drive letter in path array.

move.b     #$3A, (a6)+          ; Store a colon in path array.

get_directory_path:

move.w     #0, -(sp)

pea        (a6)                 ; Push address of next path array element.

move.w     #$47, -(sp)          ; See Internals page 135.

trap       #1

addq.l     #8, sp

process_command_line:

move.b     (a3)+, d0            ; Fetch command line character count.

ext.w      d0                   ; Extend to word for next instruction.

move.b     #0, 0(a3,d0.w)       ; Store a null at end of command line input.

lea        file_name, a0        ; Fetch address of variable.

move.l     a3, (a0)             ; Store address of file name for output data.

search_for_file:

move.w     #0, -(sp)            ; Attribute = normal access.

pea        (a3)                 ; Push address of file name.

move.w     #$4E, -(sp)          ; GEMDOS function = search first.

trap       #1

addq.l     #8, sp

tst        d0

bne        terminate            ; Terminate if file not found.

process_interrupt_handler_variables:

pea        fetch_interrupt_handler_address

move.w     #$26, -(sp)          ; Execute a routine in supervisor mode.

trap       #14

addq.l     #6, sp

lea        handler_address, a4  ; Fetch handler address.

movea.l    (a4), a4

adda.l     #$16C, a4            ; Add offset to variable "filesize".

move.l     $1A(a5), d6          ; Fetch file size.

move.l     d6, (a4)             ; Store file size in handler variable.

adda.l     #$188, a4            ; Add offset to buffer variable.

open_file:                       ; Function returns file handle in D0.

move.w     #0, -(a7)            ; Open as read only.

pea        (a3)                 ; Push address of file name.

move.w     #$3D, -(a7)          ; See Internals page 127.

trap       #1

addq.l     #8, a7

move.w     d0, d3               ; Store file handle in D3.

trap       #3                   ; Value of system clock returned in D0.

move.l     d0, d4               ; Store time in D4.

read_file:

pea        (a4)                 ; Push buffer address

move.l     $1A(a5), -(sp)       ; Number of bytes to read.

move.w     d3, -(sp)            ; File's handle number.

move.w     #$3F, -(sp)          ; GEMDOS function = read.

trap       #1

lea        $C(sp), sp           ; Reposition stack pointer.

close_file:

move.w     d3, -(sp)            ; Push file handle.

move.w     #$3E, -(sp)          ; See Internals page 128.

trap       #1

addq       #4, sp

transfer_rate_calculations:

trap       #3                   ; Fetch current time.

move.l     d0, d5               ; Store end time for data file.

sub.l      d4, d0               ; Subtract start time from end time.

convert_time_to_milliseconds:    ; Multiply by 5.

move.l     d0, d2               ; Save copy to add.

asl.l      #2, d2               ; Multiply by 4.

add.l      d0, d2               ; To complete multiplication by 5.

compute_transfer_rate:

move.l     d6, d0               ; Fetch file size from D6.

divu       d2, d0               ; Divide size by time in milliseconds.

copy_to_d1:                      ; Will extract remainder from D1.

move.l     d0, d1               ; Quotient is in lower word of D0.

mulu       #1000, d0            ; Multiply quotient by 1000.

clr.w      d1                   ; Get rid of quotient.

swap       d1                   ; Remainder is now in lower word of D1.

mulu       #1000, d1            ; Multiply remainder by 1000.

divu       d2, d1               ; Divide remainder by time in milliseconds.

swap       d1                   ; Put remainder in lower word.

clr.w      d1                   ; Get rid of remainder.

swap       d1                   ; Get quotient back in lower word.

add.l      d0, d1               ; Transfer rate is now in D1.

; Note: The order in which things are done in the calculation algorithm

;       is critical.  The remainder obtained from the first division must

;       be manipulated just right.  One other point is that the quotient

;       will be zero whenever the time in milliseconds is a value greater

;       than the size of the file.

;       I have verified the accuracy of the above algorithm, but I have

;       decided to print the number of 5 milliseconds clock ticks obtained

;       for both the start and end times.  This will permit the transfer

;       rate to be calculated manually.  Remember that those clock ticks

;       must be multiplied by 5 to convert them to milliseconds.  And, of

;       course, the number of milliseconds must be divided by 1000 to convert

;       them to seonds; or one can divide the file size by the number of

;       milliseconds and multiply the quotient by 1000.

; The equation being used in this algorithm is:

; transfer rate (bytes/second) = [file size (bytes)/transfer time (msec)]

;                                X 1000 (msec/second).

trap       #4                   ; Convert to ASCII decimal.

lea        transfer_rate, a1

move.l     a0, (a1)             ; Store address of decimal string.

; NOTE: The variable 'transfer_rate' now contains the address of the

;       ASCII decimal string that is the transfer rate.  Trap #4 can't

;       be invoked until that value is used by the program, otherwise

;       the value will be corrupted.

create_output_file:

move.w     #0, -(sp)            ; File attribute = read/write.

pea        output_file          ; Push address of output file's name.

move.w     #$3C, -(sp)          ; Function = f_create = GEMDOS $3C.

trap       #1                   ; File handle is returned in D0.

addq.l     #8, sp

lea        output_file_handle, a0

move.w     d0, (a0)

redirect_output_bound_for_screen:

; NOTE: If the output file's handle is exchanged with the video screen's

;       handle, then the printline function = GEMDOS $9 can be used to

;       write to the file.

redirect_output:                 ; Exchange file handle with screen's handle.

move.w     d0, -(sp)            ; This is the disk file's handle.

move.w     #1, -(sp)            ; This is the video screen's handle.

move.w     #$46, -(sp)          ; Function = f_force = GEMDOS $46.

trap       #1

addq.l     #6, sp

print_data_report:

lea        heading, a0          ; Print data file heading.

bsr        print_string

lea        header_1, a0         ; Transfer Rate header.

bsr        print_string

move.l     transfer_rate, a0    ; Transfer Rate.  Fetch from trap #4

bsr        print_string         ; location.  Quick before it goes away.

lea        header_2, a0         ; Transfer Rate Units.

bsr        print_string

lea        header_3, a0         ; File name header.

bsr        print_string

lea        file_name, a1        ; File name.

movea.l    (a1), a0             ; File's name is stored in command line.

bsr        print_string

lea        header_4, a0         ; File length header.

bsr        print_string

move.l     d6, d1               ; Fetch file size from D6.

trap       #4                   ; Convert to ASCII decimal.

bsr        print_string

lea        header_5, a0         ; File size units.

bsr        print_string

lea        header_9, a0         ; Directory path.

bsr        print_string

lea        path, a0

bsr        print_string

lea        header_7, a0         ; End Time header.

bsr        print_string

move.l     d5, d1               ; Fetch end time.

trap       #4                   ; Convert to ASCII decimal.

bsr        print_string

lea        header_8, a0         ; Time units = 5-millisecond ticks.

bsr        print_string

lea        header_6, a0         ; Start Time header.

bsr        print_string

move.l     d4, d1               ; Fetch start time.

trap       #4                   ; Convert to ASCII decimal.

bsr        print_string

lea        header_8, a0         ; Time units = 5-millisecond ticks.

bsr        print_string

close_output_file:

move.w     output_file_handle, -(sp)

move.w     #$3E, -(sp)          ; Function = GEMDOS $3E = f_close.

trap       #1

addq.l     #4, sp

initiate_interrupt_process:      ; Execute in supervisor mode.

pea        send_null

move.w     #$26, -(sp)

trap       #14

addq.l     #6, sp

terminate:

move.w     #0, -(sp)

trap       #1

fetch_interrupt_handler_address:

lea        handler_address, a0

move.l     $100, (a0)           ; Interrupt handler address is stored at

rts                             ; location $100.

send_null:                       ; The printer will drop the logic level of

moveq.l    #0, d0               ; pin 11 from high to low after receiving this

; character, even though nothing will be

; printed.

lea        $FF8800, a1          ; Address of Programmable Sound Generator.

move.b     #$F, (a1)            ; Select port B (register 17) of PSG.

move.b     d0, 2(a1)            ; Write character to PSG register 17.

move.b     #$E, (a1)            ; Select port A (register 16) of PSG.

move.b     (a1), d0             ; Get current register 16 value.

strobe_low:

andi.b     #$DF, d0             ; Bit 5 to zero.

move.b     d0, 2(a1)            ; Bit 5 of port A to zero.

strobe_high:

ori.b      #$20, d0             ; Bit 5 to one.

move.b     d0, 2(a1)            ; Bit 5 of port A to one.

rts

print_string:

pea        (a0)

move.w     #9, -(sp)

trap       #1

addq.l     #6, sp

rts

data

heading:      dc.b $D,$A,'PRG_7NP.TOS Execution Results',$D,$A,$D,$A

dc.b '  Print Buffer:   PRG_7IP',0

header_1:     dc.b $D,$A,'  Transfer Rate: ',0

header_2:     dc.b ' bytes/second',$D,$A,0

header_3:     dc.b '  File Printed:   ',0

header_4:     dc.b $D,$A,'  File Length:   ',0

header_5:     dc.b ' bytes',$D,$A,0

header_9      dc.b $D,$A,'  Path: ',0

header_7:     dc.b $D,$A,$D,$A,'  End Time:      ',0

header_6:     dc.b '  Start Time:    ',0

header_8:     dc.b ' 5-millisecond ticks',$A,$D,0

output_file:  dc.b 'PRG_7IP.DAT',0

bss

output_file_handle:  ds.w   1

transfer_rate:       ds.l   1

dta:                 ds.l  11

file_name:           ds.l   1   ; Address of command line (input file name).

handler_address:     ds.l   1

path:                ds.b 120

ds.l  96   ; Program stack.

stack:               ds.l   0   ; Address of program stack.

program_end:         ds.l   0

end

PRG_7NP.TOS Execution Results

Print Buffer:   PRG_7IP

Transfer Rate:  434133 bytes/second

File Printed:   PRG_7NP.S

File Length:    13024 bytes

Path: P:        ; Ram disk.

End Time:       333018 5-millisecond ticks

Start Time:     333012 5-millisecond ticks

PRG_7NP.TOS Execution Results

Print Buffer:   PRG_7IP

Transfer Rate:  41346 bytes/second

File Printed:   PRG_7NP.S

File Length:    13024 bytes

Path: E:\PRG_7  ; Hard disk.

End Time:       462620 5-millisecond ticks

Start Time:     462557 5-millisecond ticks

PRG_7NP.TOS Execution Results

Print Buffer:   PRG_7IP

Transfer Rate:  9336 bytes/second

File Printed:   PRG_7NP.S

File Length:    13024 bytes

Path: A:\PRG_7

End Time:       426667 5-millisecond ticks

Start Time:     426388 5-millisecond ticks

The final program in this chapter has been designed to help

you to find out a few facts about your printer's buffers.  It

should permit you to determine the length of your printer's line

buffer and some details of its operation.  It's best that you

read the program completely so that you can understand just what

it attempts to accomplish.  Program 75 is short and uncluttered,

so you should be able to alter it to conform to any particular

experiment you might want to attempt.

Program 75. A program to let you experiment with your

printer's line buffer.

; Program Name: PRG_7OP.S

;      Version: 1.002

; Assembly Instructions:

;      Assemble in PC-relative mode and save with a TOS suffix.

; Function:

;      Provides direct output via the Parallel Port Interface.  This program

; is to be used to illustrate the operation of a printer's line buffer.  First

; it prints a short string which is terminated with a carriage return/line

; feed.  Then it prints a character string that is greater than 200 characters

; long so that the length of the line buffer can be determined.  Finally, it

; prints a short string with no terminating carriage return/line feed.

mainline:

; NOTE: Must enter supervisor mode in order to access MFP and PSG addresses.

enter_supervisor_mode:

move.l    #0, -(sp)

move.w    #$20, -(sp)          ; Function = SUPER.

trap      #1                   ; Supervisor mode is active after TRAP.

addq.l    #6, sp               ; D0 = SSP.

print_string_1:

lea       string_1, a3         ; Fetch address of string.

bsr       print_character      ; Print the string.

print_strin_2:

lea       string_2, a3

bsr       print_character

wait_for_return_key_press:

move.w    #8, -(sp)

trap      #1

addq.l    #2, sp

print_string_3:

lea       string_3, a3

bsr       print_character

enter_user_mode:

move.l    d0, -(sp)            ; D0 contains SSP.

move.w    #$20, -(sp)          ; Function = SUPER.

trap      #1                   ; User mode is active after TRAP.

addq.l    #6, sp

terminate:

move.w    #0, -(sp)           ; Function = p_term_old = GEMDOS $0.

trap      #1                  ; GEMDOS call.

print_character:

move.b    (a3)+, d1

beq       return

printer_ready_test:

btst      #0, $FFFA01          ; Check logic level of pin 11.

bne.s     printer_ready_test   ; Loop until printer is ready to receive.

lea       $FF8800, a2          ; Fetch address of PSG data bus.

move.b    #$F, (a2)            ; Latch address of port B.

move.b    d1, 2(a2)            ; Write character to port B.

move.b    #$E, (a2)            ; Latch address of port A.

move.b    (a2), d1             ; Read content of port A.

andi.b    #$DF, d1             ; Reset bit 5 of d1 to zero.

move.b    d1, 2(a2)            ; Reset bit 5 of port A. Strobe low.

ori.b     #$20, d1             ; Set bit 5 of d1.

move.b    d1, 2(a2)            ; Set bit 5 of port A. Strobe high.

bra       print_character

return:

rts

data

string_1: dc.b  'This is the short string with cr/lf.',$D,$A,0

string_2: dc.b  'This is the long string which does not terminate '

dc.b  'with a carriage return/line feed.  It is greater than two '

dc.b  'hundred characters long, which should be longer than '

dc.b  'the printers line buffer.  All but the last line should '

dc.b  'probably print because the line buffer fills; but '

dc.b  'you should probably have to turn the printer off-line, '

dc.b  'then back on the get the final line to print.  The ',

dc.b  'program will pause for you to do this.',0

string_3: dc.b  'This string has no cr/lf.  The off-line/on-line sequence '

dc.b  'should make it print.',0

align

end

A Note About GEMDOS Function $46

I have used this function in several programs to redirect

data flow via GEMDOS function $9 so that the string of characters

is written to a file instead of to the screen.  If you read the

description of this function on page 134 of the Internals book, I

think that you will conclude, as I did, that it should also be

possible to redirect output bound for the printer to a file also.

I know that the reference seems to indicate that GEMDOS function

$46 might be more appropriately used in conjunction with the

Write functions, but you have seen that I use it with the Print

Line function without problem.  Anyway, I have not been able to

use function $46 to redirect output from the printer to a file.

I can't conclude that the function doesn't work that way, but I

think that it would be a valuable asset if it did work in the

manner that I would like.

Conclusion

I have always considered the output from a computer to be

its most valuable service.  I've never been impressed with what a

computer could do unless it could show me the results quickly and

in forms that permitted me to rapidly assess the output data.

But I've always been amazed by the lack of attention to detail

paid by most reference books and owner's manuals concerning such

output.  Let me not be guilty of that in this book.

At the time that I am writing this, I am reading the newest

COMPUTE!'s technical reference guide for the Atari ST, TOS, by

Sheldon Leemon.  There is sufficient assembly language references

in the book to permit me to heartily recommend it, if you can

afford the purchase.  Much of the material in the book is also

covered in the Internals book, but in many areas Leemon does a

better job of presenting the material.  I wish that I could as

heartily recommend COMPUTE!'s assembly language programming book,

but I find it to be a feeble attempt.  Nevertheless, it is

certainly much better than the Abacus attempt on the same

subject.